DELEGATED APPLICATION REPORT 				Target Date:	11/05/2016

Application No: 160478/HOU

Address: 1 Southcote Farm Lane, Reading

Proposal: New detached garage to replace existing

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a two storey semi-detached property located on the corner of Southcote Farm Lane and Southcote Lane.  The property has a detached flat roof garage and a concrete driveway and two sheds.  There is 2.4m high hedging along the boundary with Southcote Farm Lane.  The surrounding area is predominantly residential with similar style properties.

Proposal:

The proposal involves the removal of the existing garage and sheds and the construction of a new garage with a study measuring 5.9m in width and 5m in depth.  The proposed building measures 2.2m in height to eaves level and 3.1m in height at its highest point.  The proposal has a shallow pitched roof with double doors to the garage, a single door and window to the study and a side window facing the property.

The proposed garage is located 1.5m from the boundary with Southcote Farm Lane and will be partially screened by existing 2m high fencing and 2.4m high hedging.

The proposal involves extending the existing driveway by removing some of the grass.  The proposed site plan states the driveway will have a concrete surface but a condition will be imposed which requires the surface to be of porous materials.

The materials will consist of pre-fabricated timber interlocking logs with a felt covered pitched roof.  The manufacturers details show a natural wood colour however, the applicant has advised that the building would be painted in a darker colour.

Plans Considered:

The agent submitted the following drawings, received on 15th March 2016:

Drawing No: AD/01 Rev A – Existing Site & Location Plans

Drawing No: AD/02 Rev B – Proposed Site Plan & Details

Tuindeco International BV – Garage Moa ref: 40.0803 (manufacturer details)

Relevant Planning History:

None.

Consultation Responses:

Transport raised no objection subject to a condition requiring vehicle parking in accordance with the approved plans.

Properties at 75 Southcote Lane and 3 Southcote Farm Lane were consulted.  One letter in support of the application has been received.

Summary of discussions with applicant/agent:

None.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, although the proposed garage will be visible from the road it is 1.5m away from the boundary with an eaves height of 2.2m and a maximum height of 3.1m.  The boundary hedging is 2.4m in height and as such the proposed garage is not considered to detract from the appearance of the original building.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO, although the proposal will be visible from the road it is not considered to detract from the appearance of the area due to the single storey nature of the proposal and the screening provided by the 2.4m high hedging along the boundary.

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO

- Visual dominance and overbearing effects of a development; NO

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time limit

- Materials

- Approved plans

- Vehicle parking in accordance with approved plans

- Driveway materials to be porous

Informatives relating to:

- Terms and conditions

2.	Building regulations

3.	Construction and demolition

4. 	Positive and proactive