DELEGATED APPLICATION REPORT 				Target Date:	 18/05/2016

Application No: 160560/HOU

Address: 5 Treforgan

Proposal: Single storey rear extension, insertion of ground floor side elevation window to dining room. Garage conversion to create a habitable room, and increased hard surfacing to front driveway.

Case officer: Heather Banks

Site Description and Surrounding Area:

The application relates to a two storey, red brick detached property on the west side of Treforgan. The host dwelling has a driveway and an integral garage and the garden to the rear of the property slopes steeply up at the back. The property is situated within the 1990s Shipnells Farm development. The surrounding area is residential in nature and characterised by detached properties of similar design, on reasonably small plots properties.

Proposal:

The application is for a single storey rear extension, the conversion of the garage to a habitable room, the insertion of a ground floor side elevation window and additional hard surfacing to the front driveway.

The single storey rear extension will be 5.3m deep and 3.5m wide and will run flush with the south facing side elevation of the application property. It will have a pitched roof with an eaves height of 2.4m above ground level and a ridge height of 3.5m. It is proposed that there will be a window in the rear facing elevation and a 3.6m wide opening fitted with bi-fold doors, to the north facing side elevation. It is also proposed that there will be two sky lights fitted to each roof slope.

The conversion of the garage to a habitable room will involve the removal of the garage door to be replaced with brick work to match the existing property and a 2.3m window in a style to match the existing windows.

It is also proposed to insert a small window in the south facing side elevation of the existing property to provide light to the dining doom. The window will be no higher than the existing door in this side elevation.

The area to the front of the property which is hard standing is to be increased. It is proposed that the hard standing will span the full width of the property in the area closest to the house, it will then narrow to its existing width closer to the road. This is to allow for three cars to be parking on the driveway.

The materials to be used in the above works will be selected to match those of the existing property.

Plans Considered:

The agent submitted the following drawings, received on 23rd March 2016

Drawing No: PL-01 Rev P1 – Location Plan

Drawing No: PL-02 Rev P1 – Block Plan

Drawing No: EX-01 Rev P1 – Existing ground floor and roof plans

Drawing No: EX-02 Rev P1 – Existing Elevations

Drawing No: PL-03 Rev P1 – Proposed ground floor and roof plans

Drawing No: PL-04 Rev P1 – Proposed Elevations

Relevant Planning History:

94-00014-RM - 37 dwellings, roads and landscaping for Hassall Homes Ltd (To Replace 37 Dwellings already permitted for Higgs and Hill by Planning Consent 91/0879/RM)

Consultation Responses:

RBC Transport Strategy has stated that there are no transport objections. The proposed conversion of the garage will have no transport implications as the plans submitted indicate provision has been made for three off road vehicles.

Properties at 3, 7 and 9 Treforgan were consulted and no comments have been received.

Summary of discussions with applicant/agent:

None

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy, 2008 (altered 2015)

CS7 (Design and the Public Realm)

CS24 (Car/Cycle Parking)

Sites and Detailed Policies Document, Oct 2012, (altered 2015)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

DM12: Access, Traffic and Highway related matters

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal

The main issues are considered to be:

(i)   	Impact on the character of the host property

(ii)  	Impact on the character of neighbouring properties and the surrounding area

(iii)  	Amenity

(iv)	Parking provision

Impact on the character of the host property

Policy DM9 of the Sites and Detailed Policies Document, Oct 2012 (altered 2015) states that an extension to a house will be acceptable where it respects the character of the house in terms of scale, location, materials and design.

It is considered that the proposed developments will not have a harmful impact on the character of the host property. Whilst the proposed extension is relatively deep in comparison to the host property, it is considered that as it will be single storey, clearly subservient to the existing property, well-proportioned, relatively restrained in width and will match the existing property in terms of materials it will not appear incongruous or out of keeping with the character of the property.

The other proposed changes to the property will also match the existing property in terms of the materials used so as to respect the character and appearance of the property.

The new window to the front of the property, although it will have a lower sill level than the other existing ground floor, front facing window, is not considered to appear out of character with the property as it is well proportioned, and being a wider window can accommodate the increase in height without appearing out of place.

Impact on the character of neighbouring properties and the street scene

Policy DM9 of the Sites and Detailed Policies Document, Oct 2012 (altered 2015) states that an extension to a house will be acceptable where it respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design.

The elements of this proposal that will have the greatest impact on the street scene are the removal of the existing garage door to be replaced with a window and the increase in the area of hard standing to the front of the property which will necessitate the removal of a section of existing greenery. Although the conversion of the garage to a habitable room will result in this property being the only property in the vicinity to not have a garage it is considered that, due to the existing variations between the surrounding properties and the fact that the proposed replacement window will be similar to the large ground floor front windows at 1,2, 3 and 7 Treforgan, the change will not appear out of place in the street scene. With regards to the loss of greenery, although its retention would be preferable, it is considered that the increase in hard standing would not be out of character with the street scene given the size of surrounding driveways and that the planting closest to the road is to be retained.

The depth of the rear extension at only 1m from the boundary from 3 Treforgan, is considered to be relatively large. However, given that it will be constructed using materials to match those of the surrounding buildings, will only be single storey, and will be a total of 2.5m from the side of the property at 3 Treforgan, it is not considered that the character of the property at 3 Treforgan will be significantly harmed by the rear extension.

Amenity

DM4 of the Sites and Detailed Policies Document, Oct 2012 (altered 2015) states development should not cause a significant detrimental impact to the living environment of existing of new residential properties in terms of overshadowing, privacy and overlooking and visual dominance and overbearing effects of a development.

The proposed extension will project 5.3m from the rear of the existing property and will be stepped in by 1m from the boundary with 3 Treforgan. Also due to the stagger of the properties the extension will actually extend approximately 7m from the rear of 3 Treforgan. The depth of the proposed extension is longer than the 4m recommended in the Supplementary Planning Guidance - A Design Guide to House Extensions (2003). However, as the extension is located to the north of 3 Treforgan it is not considered that it will cause any overshadowing of this property. Also, as the extension will be 2.5 from side of 3 Treforgan, will only be single storey and will have a roof that will slope away from this property, it is not considered that the extension would appear overbearing to the occupiers of 3 Treforgan.

The proposed rear extension will be fitted with side facing bi-fold doors. As the extension is single storey, and there is no change in ground level between the gardens of the application property and 7 Treforgan, it is not considered that these side facing glazed doors will cause any significant overlooking and privacy issues.

A ground floor side facing window is proposed in the south facing façade of the existing property. Given that it is a ground floor window, the high boundary fence, the lack of side facing windows at 3 Treforgan and the small nature of the window, being no higher than the existing door in this elevation, it is considered that this window will not create issues of privacy and overlooking.

Parking Provision

The original planning permission for this development, 94-00014-RM , contained the following condition:

Condition 22:

The garage(s) hereby permitted, shall be used soley for the garaging of private motor vehicles owned by and/or used by residents of the domestic property to which the garage(s), hereby permitted, relates. Any repair of behicles shall be restricted to the said private vehicles only and shall take place only within the garage(s), not on the forecourt, the adjoining garden or street. No commercial repairs, panel beating, cellulose spraying, nor any other trade or business shall be carried on, in or from the garage(s).

Reason: To enable the local planning authority to control the use of such garages in the interests of amenity and road safety

After consultation relating to the proposed conversion of the garage at the application property to a habitable room, the RBC Transport Strategy department has stated that, as provision of off road parking for up to 3 vehicles has been provided, they have no objections to the loss of the garage. An informative will therefore denote that condition 22 no longer applies to the application site.

RECOMMENDATION:

GRANT

Conditions:

- The development to which this permission relates must be begun not later than the expiration of three years beginning with the date on which the permission was granted.

Reason: to prevent an accumulation of unimplemented planning permissions and in accordance with Section 91 of the Town & Country Planning Act 1990 as amended by the Planning and Compulsory Purchase Act 2004.

2. 	The materials to be used in the construction of the external surfaces of the development shall match those used on the existing building unless where otherwise stated.

Reason: To ensure that the new development appears in keeping with the existing building, in the interests of the character of the area, in accordance with Policy CS7 of the Reading Borough Local Development Framework Core Strategy 2008 (altered 2015) and Policy DM9 of the Sites and Detailed Policies Document 2012 (altered 2015).

- The permission relates only to the following plans and drawings:

Drawing No: PL-01 Rev P1 – Location Plan

Drawing No: PL-02 Rev P1 – Block Plan

Drawing No: EX-01 Rev P1 – Existing ground floor and roof plans

Drawing No: EX-02 Rev P1 – Existing Elevations

Drawing No: PL-03 Rev P1 – Proposed ground floor and roof plans

Drawing No: PL-04 Rev P1 – Proposed Elevations

Received 23rd March 2016

Reason: To clarify which plans have been approved and in the interests of proper planning.

Informatives relating to:

  - Your attention is drawn to the terms and conditions of this permission. Any development which is carried out but which differs materially from the approved plans and details, or does not comply with any condition(s) or planning obligation(s) attached to the permission may result in the Council taking action to remedy the breach of planning control. If you are in any doubt please contact the Council.

  - The applicant’s attention is drawn to the requirement for submission of a Building Regulations application for the proposed development.  Please contact Building Control on 0118 937 2449 for advice.

  - The construction (and demolition) of the development will be subject to environmental health law and the applicant is referred to:

http://beta.reading.gov.uk/environmentalprotection

  - During the course of construction, the developer should take care to ensure that no part of this development (foundations, eaves, guttering, etc.) would encroach on, under or across the boundary line with the adjoining property, as the requisite Notice has not been served on the owner of that property.  This permission does not confer any right of access onto land that is not under the developer's control. The developer should also be mindful of the obligations and requirements of the Party Wall Act (1996) details of which can be found at: http://www.communities.gov.uk/publications/planningandbuilding/partywall

  - The decision to grant planning permission follows the positive and proactive consideration of the issues raised by this planning application and issuing the decision within the eight week time period.

  - By consequence of this permission, it is confirmed that condition 22 of planning permission 94/00014/RM, as granted on 01/06/1994, no longer applies to No. 5 Treforgan only.