Delegated Application Report 					Target Date: 27/09/16

Address: 13 Beverley Road

Application No: 161431/CLP

Proposal: Demolition of existing rear extension, construction of single storey rear extension

Case Officer: Tom Hughes

Introduction and Proposal

The application site is an end of terrace property with a hipped roof and two storey front gable. The property has a single storey rear extension, a porch with a mono-pitch roof and an adjoining garage.  The surrounding area is predominantly residential.

The applicant proposes the demolition of the existing rear extension, and the erection of a new single storey rear extension with a mono-pitch roof. The proposed extension would extend to a depth of 3m, across the full width of the property by 7.39m. The mono-pitch roof would contain two skylights and have a height of 3m, with a height to eaves of 2.1m. Bi-fold doors would be located on the rear elevation.

Plans Submitted

The agent supplied the following drawings, received on 1st August 2016:

Drawing No: 1607/01 – Existing Plans and Elevations

Drawing No: 1607/02 – Proposed Plans and Elevations

Drawing No: 1607/03 – Proposed Section A-A

Drawing No: 1607/05 – Site Location and Block Plan

Relevant Planning History

950005 – 14/06/95 – brick garage adjoining side of house – Permitted 18/08/95

021030 – 29/01/02 – erection of porch – Permitted 03/04/02

030761 – 18/06/03 – rear conservatory – Permitted 02/09/03

Summary of Consultations

No consultations were undertaken as it is not required for an application of this type.

Relevant Planning Policy and Guidance

Annex 8 "Lawfulness and the Lawful Development Certificate" of Planning Circular 10/97 'Enforcing Planning Control: Legislative Provisions and Procedural Requirements" sets out the procedure for assessing applications for certificates of lawfulness:

Para 8.26 states, "if the LPA are supplied with information satisfying them that the operations described in the application would be lawful, if instituted or begun at the time of the application, they shall issue a certificate to that effect; and, in any other case, they shall refuse the application." "The burden of proof in a LDC application is firmly on the applicant."

The Town and Country Planning (General Permitted Development) (England) Order 2015 (GPDO)

Permitted development rights for householders – Technical Guidance April 2016

A Design Guide to House Extensions (Supplementary Planning Guidance)

Assessment

A Certificate of Lawfulness for a proposed development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

The proposed single storey rear extension must be assessed to ensure that it complies with the conditions of Class A, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015, which relates to the ‘enlargement, improvement or other alteration of a dwellinghouse’.

The relevant conditions of the GPDO for a single storey rear extension, which is not listed or in a conservation area, are as follows:

A.1 Development is not permitted by Class A if:

A.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); complies

A.1 (b) as a result of the works, the total area of ground covered by buildings within the curtilage of the dwellinghouse (other than the original dwellinghouse) would exceed 50% of the total area of the curtilage (excluding the ground area of the original dwellinghouse); complies

A.1 (c) the height of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the highest part of the roof of the existing dwellinghouse; complies

A.1 (d) the height of the eaves of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the eaves of the existing dwellinghouse; complies

A.1 (e) the enlarged part of the dwellinghouse would extend beyond a wall which (i) forms the principle elevation of the original dwellinghouse; complies or (ii) fronts a highway and forms a side elevation of the original dwellinghouse; complies

A.1 (f) subject to paragraph (g), the enlarged part of the dwellinghouse would have a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 4 metres in the case of a detached dwellinghouse, or 3 metres in the case of any other dwellinghouse, complies or (ii) exceed 4 metres in height; complies

A.1 (g) until 30th May 2019, for a dwellinghouse not on article 2(3) land nor on a site of special scientific interest, the enlarged part of the dwellinghouse would have a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 8 metres in the case of a detached dwellinghouse, n/a or 6 metres in the case of any other dwellinghouse, complies or (ii) exceed 4 metres in height; complies

A.1 (h) the enlarged part of the dwellinghouse would have more than a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 3 metres, n/a or (ii) be within 7 metres of any boundary of the curtilage of the dwellinghouse opposite the rear wall of the dwellinghouse; n/a

A.1 (i) the enlarged part of the dwellinghouse would be within 2 metres of the boundary of the curtilage of the dwellinghouse, and the height of the eaves of the enlarged part would exceed 3 metres; complies

A.1 (j) the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse, and would – (i) exceed 4 metres in height, n/a (ii) have more than a single storey, n/a or (iii) have a width greater than half the width of the original dwellinghouse; n/a or

A.1 (k) it would consist of or include – (i) the construction or provision of a verandah, balcony or raised platform, complies (ii) the installation, alteration or replacement of a microwave antenna, complies (iii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe, complies or (iv) an alteration to any part of the roof of the dwellinghouse. complies

A.2 In the case of a dwellinghouse on article 2(3) land, development is not permitted by Class A if:

A.2 (a) it would consist of or include the cladding of any part of the exterior of the dwellinghouse with stone, artificial stone, pebble dash, render, timber, plastic or tiles; n/a

A.2 (b) the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse; n/a or

A.2 (c) the enlarged part of the dwellinghouse would have more than a single storey and extend beyond the rear wall of the original dwellinghouse. n/a

Conditions

The proposed single storey rear extension is permitted by Class A subject to the following conditions:

A.3 (a) the materials used in any exterior work (other than materials used in the construction of a conservatory) must be of a similar appearance to those used in the construction of the exterior of the dwellinghouse; complies, subject to condition

A.3 (b) any upper-floor window located in a wall or roof slope forming a side elevation of the dwellinghouse must be (i) obscure-glazed, n/a and (ii) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed; n/a and

A.3 (c) where the enlarged part of the dwellinghouse has more than a single storey, the roof pitch of the enlarged part must, so far as practicable, be the same as the roof pitch of the original dwellinghouse. n/a

Recommendation

Sufficient evidence has been provided to demonstrate that a Certificate of Lawfulness should be granted for the proposed demolition of the existing rear extension and construction of single storey rear extension with a maximum depth of 3m and width of 7.39m, and with a maximum height of 3m with a height to eaves of 2.1m.

The proposal has been assessed against the Town and Country Planning Act 1990 (as amended) and Class A, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015. It can be confirmed that the proposed operation would be lawful for planning purposes on the basis of the drawings received on 1st August 2016:

Drawing No: 1607/01 – Existing Plans and Elevations

Drawing No: 1607/02 – Proposed Plans and Elevations

Drawing No: 1607/03 – Proposed Section A-A

Drawing No: 1607/05 – Site Location and Block Plan

Therefore the Local Planning Authority can issue the applicant with a Certificate of Lawfulness for the proposed development.

Informatives

Building Regulations

Encroachment and Party Wall

Construction and Demolition

Materials