DELEGATED APPLICATION REPORT 				Target Date: 21/10/2016

Application No: 161620/HOU

Address: 252 Kidmore Road

Proposal: Part 2 storey part single storey side and rear extension

Case officer: Heather Banks

Site Description and Surrounding Area:

The application property is a two storey, red brick, semi-detached property, with a conservatory to the rear of the property, the main entrance to the side elevation and a low eaves height to the main roof. There is off road parking for two cars to the front of the property and side access to the rear garden. The attached neighbour and the neighbouring semi-detached pair to the south of the property are of the same design, whilst the other properties in the vicinity are a mix of semi-detached and detached properties of a similar scale but varied designs.  The pair of semis is wide, and has a one large gabled front and symmetrical chimneys either side, so the pair could easily be mistaken for one large, detached property.

Proposal:

Planning permission is sought for the demolition of the existing rear conservatory and the construction of a part single storey part two storey side and rear extension.

The extension would project 5.35m from the rear of the property and have a width of 5.6m from the boundary with 250 Kidmore Road. The first floor element of the proposal will be wholly to the rear of the existing rear elevation of the property and chamfered away from the boundary with no. 250, to avoid a line taken from 45 degrees from the first floor window at this property. The roof form will be pitched, following the pitch of the original property but having a considerably lower ridge line. The roof will extend the full 5.35m over the extension, and will not be chamfered in line with the first floor element. The single storey element below the chamfer will have a hipped roof. There will also be a single storey element to the side of the original property, which along with the rear extension, will reduce the side access to a width of 0.9m. There is one new, side facing, first floor window proposed to the side elevation of the original house. There is also a first floor window proposed to the rear extension, which is to have an irregular shape to match the pitch of the roof.

The materials to be used in the proposed extensions are to match those of the existing property

Plans Considered:

The agent submitted the following drawings, received on 26th August 2016

Drawing No: 160406-1 	– Location Plan

Drawing No: 160406-2 	– Existing Site Plan

Drawing No: 160406-3A 	– Proposed Site Plan

Drawing No: 160406-4 	– Existing Elevations

Drawing No: 160406-5 	– Existing Plans

Drawing No: 160406-6A 	– Proposed Elevations

Drawing No: 160406-7A 	– Proposed Plans

Drawing No: 160406-8 	– Proposed Section

Relevant Planning History:

None

Consultation Responses:

The RBC Ecologist requested the submission of a bat survey on the 7th October. The agent was notified of this on that date. No bat survey has been received.

RBC Transport Strategy stated that there were no transport objections to the proposal as the parking provision at this property with the two off road parking spaces being retained.

The properties at 248b, 250 and 254 Kidmore Road were consulted and no comments were received.

Summary of discussions with agent/applicant:

The agent was advised on 7th October was advised to withdraw the application as it would be recommended for refusal. The agent was advised that it was considered that the proposal would cause unnacceptable harm to the amenity of the occupiers of both 250 and 254 Kidmore Road, in terms of loss of light and creating an overbearing. Also it was considered that the proposal would cause harm to the character of the application property and its adjoined neighbour, 250 Kidmore Road, through the contrived design of the proposed rear extension. The agent was also advised that, should the rest of the application be considered acceptable, a bat survey would still need to be carried out, submitted and approved before approval could be granted. No response was received from the agent following this email.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise.  Material considerations include relevant policies in the National Planning Policy framework (NPPF) - among them the 'presumption in favour of sustainable development'.  However the NPPF does not change the statutory status of the development plan as the starting point for decision making (NPPF paragraph 12).

In this regard, the NPPF states that due weight should be given to the adopted policies of the Local Development Framework (LDF) (Core Strategy and Sites and Detailed Policies Document) according to their degree of consistency with the NPPF (the closer the policies in the plan to the policies in the NPPF, the greater the weight that may be given).

Accordingly, the National Planning Policy Framework and the following development plan policies and supplementary planning guidance are relevant:

Reading Borough Local Development Framework Core Strategy, 2008 (altered 2015)

CS7 (Design and the Public Realm)

CS36 (Biodiversity and Geology)

Sites and Detailed Policies Document, Oct 2012 (altered 2015)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal

The main issues are considered to be:

(i) Impact on the character of the application property and its adjoined neighbour

(iii) Impact on amenity of neighbouring properties

Character of application property and its adjoined neighbour

Policy DM9: House Extensions and Ancillary Accommodation of the Sites and Detailed Policies Document (2012) states that an extension to a house should respect the character of the house in terms of scale, location, materials and design. Core Strategy Policy CS7: Design and the Public Realm states all development must be of a high design quality that maintains and enhances the character and appearance of the area Extensions should also follow the design guidance set out in the Supplementary Planning Guidance - A Design Guide to House Extensions (2003), and should not overwhelm or otherwise dominate the original building by being too large, out of proportion or inappropriately detailed.

This would be a difficult property to extend, given its rather unusual design and careful attention would be required.  It is considered that the proposed chamfered corner to the 2 storey rear extension, with the overhanging roof from above, would be out of character with the original property and would constitute poor design. The chamfered form of the first floor element is not in keeping with the more traditional form of the original building and it is considered that the overhanging roof form and the hipped roof of the ground floor element below would appear awkward and contrived. Also the design of the proposed first floor rear facing window, would appear contrived, the irregular form only being employed due to the low eaves height of the roof. It is considered that any two storey rear extension should endeavour to respect and reference the characteristics of the original property and it is considered that the above factors prevent this being achieved under the current proposal.

It is also considered that the proposal would be harmful to the character of the adjoined neighbouring property, 250 Kidmore Road. The depth of the proposed extension, at two storey, and the irregular form of the chamfered first floor element would form a dominant and jarring feature to the rear of the property, obscuring the original symmetrical character of the properties and creating a large and awkwardly composed feature on the boundary between the two properties

It is therefore considered that the extension would be contrary to Policy DM9 of the Sites and Detailed Policies Document, Oct 2012 (altered 2015) and Core Strategy Policy CS7: Design and the Public Realm, and as such the application should be refused.

Amenity

Policy DM4: Safeguarding Amenity, of the Sites and Detailed Policies Document (2012) states that a development should not cause a significant detrimental impact to the living environment of existing or new residential properties, in terms of privacy and overlooking, access to sunlight and daylight and the visual dominance and overbearing effects of the development

It is considered that the proposed two storey rear extension would form an overbearing an incongruous feature on the boundary with no.250. Although the wall of the first floor extension had been angled away from no.250 to avoid causing a loss of light, the roof is to extend for the full 5.3m, and it is consider that this mass at this height, in combination with the unusual form of the extension would be unacceptably overbearing to the occupiers of this property. It is not, however, considered that the extension would cause an unacceptable loss of light to this property given the orientation of the properties, the location of the rear facing windows at no.250 and the chamfered nature of the first floor element proposed.

With relation to 254 Kidmore Road, although this property has not been fully represented on the submitted drawings, from the case officer’s site visit and a study of the Local Planning Authority’s GIS mapping system, it is considered that the proposed two storey extension would be likely to cut a 45 degree line taken from the centre of the window of the closest habitable room at no.254. As such, this would cause an unacceptable loss of light to this property. It is also considered that given the orientation of the properties the extension would cause unacceptable shading of the amenity space immediately to the rear of no.254. There is one new side facing window proposed at first floor level as part of this scheme. However, it is not considered that this would cause an unacceptable overlooking relationship with no.254 given the depth of no.254, the fact that the window is to be to the original house rather than the proposed extension and there is only one small window to the side elevation of no.254 which does not serve a habitable room.

It is considered that whilst the proposed depth of the extension may be acceptable at ground floor level, the depth of the extension at 2 storeys would cause unacceptable harm to the amenity of the neighbouring properties and it is therefore considered that that the proposed side and rear extension would be contrary to Policy DM4 of the Sites and Detailed Policies Document, Oct 2012 (altered 2015), and should be refused.

Other issues:

Given the impact on the roof of the original house, which may well support a bat roost, a bat survey is required to be submitted before determination of the application.  This has not been provided, therefore the proposal is also not able to confirm compliance with Policy CS36.

The representation of the hipped roof to the single storey element of the rear extension, and the gabled roof to the two storey element of the rear extension appear to have been misrepresented on the submitted plans, making the drawings difficult to interpret. The inaccuracy of the submitted plans makes it difficult to fully assess the impacts of the proposed extension. Were the rest of the scheme acceptable these inaccuracies would need to be resolved before permission could be granted.

Conclusion:

Due to the depth, two storey nature and contrived design of the proposed extension it is considered that the proposal would cause unacceptable harm to the amenity of the neighbouring properties at 250 and 254 Kidmore Road and to the character of the application property and its adjoined neighbour 250 Kidmore Road. Due to the lack of a bat survey being submitted it is also considered that insufficient information has been submitted to insure that this protected species will not be harmed by the proposed works. The proposals would therefore be contrary to policies CS7 and CS36 of the Core Strategy and policies DM4 and DM9 of the Sites and Detailed Policies document and should therefore be refused.

RECOMMENDATION:

REFUSE

By virtue of its depth, two storey nature and contrived design it is considered that the proposed extension would cause unacceptable harm to the amenity of the neighbouring properties at 250 and 254 Kidmore Road in terms of loss of light and visual dominance. It is also considered to appear awkward and convoluted, representing poor design quality which fails to respect the character of the property and its adjoined neighbour. The proposal is therefore considered to be contrary to policies CS7 (Design and the Public Realm) of the Reading Borough LDF: Core Strategy (2008, as amended 2015) and policy DM9 (House Extensions and Ancillary Accommodation) of the Reading Borough LDF: Sites and Detailed Policies Document (2012, as amended 2015) and the Council’s adopted SPG, “A Design Guide to House Extensions” (2003).

The applicant has failed to submit sufficient information for the council to determine whether or not protected species (a material consideration) will be affected by the proposed development. As such this proposal is considered contrary to CS36 (Biodiversity and Geology) of the Reading Borough LDF: Core Strategy (2008, as amended 2015).

Informatives

- The plans considered in this planning application are as follows:

Drawing No: 160406-1 		– Location Plan

Drawing No: 160406-2 		– Existing Site Plan

Drawing No: 160406-3A 	– Proposed Site Plan

Drawing No: 160406-4 		– Existing Elevations

Drawing No: 160406-5 		– Existing Plans

Drawing No: 160406-6A 	– Proposed Elevations

Drawing No: 160406-7A 	– Proposed Plans

Drawing No: 160406-8 		– Proposed Section

Received 26th August 2016

- The Local Planning Authority has worked with the applicant in a positive and proactive manner. The applicant was made aware of the issues relating to the refusal of this application and advice was provided to overcome the issues raised.  The applicant decided not to respond to the advice given and so has not been possible to overcome these issues, hence the issuing of this notice of refusal of permission.

- This planning application was submitted without first having sought pre-application advice, which can often avoid the need to refuse planning permission.  The council's pre-application advice service is here:

http://www.reading.gov.uk/media/1190/Pre-Application-Enquiry-Form/pdf/Pre-app_1_Revised_Nov_2015.pdf