DELEGATED APPLICATION REPORT 			Target Date:	19/10/2016

Application No: 161529/HOU

Address: 34 Westbourne Terrace, Reading

Proposal: Single storey rear extensions

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a two storey mid-terrace property with a two storey rear gable.  There are steps down to the rear garden.  The neighbouring property at 36 Westbourne Terrace has a single storey rear extension and the neighbouring property at 32 Westbourne Terrace does not have any rear extensions.  The surrounding area is predominantly residential.

Proposal:

The application is for a single storey rear extension (similar to a previously permitted application ref: 11/01045/FUL (Civica Ref: 111730) and a single storey extension to the side of the rear gable.  The proposed rear extension measures 2.8m in depth and extends the width of the existing gable.  The proposal has a mono pitch roof and measures 2.8m in height to eaves level and 3.7m in height at its highest point.  The proposed extension to the side of the gable measures 3.7m in depth and extends to the rear of the existing gable.  The proposed side extension extends along the boundary with 32 Westbourne Terrace and has a mono pitch roof measuring 3m in height to eaves level and 3.3m in height at its highest point.

The materials will consist of brick and tiles to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 15th August 2016:

Drawing No: 001

Drawing No: 002

Drawing No: 003

Drawing No: 004

Drawing No: 005

Drawing No: 006

Drawing No: 007

Structural Engineering calculations

Relevant Planning History:

11/01045/FUL (Civica Ref: 111730) - Single storey rear extension.  Permitted 13/09/2011.

Consultation Responses:

Properties at 32 and 36 Westbourne Terrace were consulted.  One comment was received advising of the Party Wall Act.

Summary of discussions with applicant/agent:

None.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7: Design and the Public Realm

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; There may be some loss of light to the side windows of 32 Westbourne Terrace from the proposed side extension but the orientation of the properties is acceptable and the roof slopes away from this neighbouring property which mitigates any impact on this neighbouring property.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO, the proposal involves a new small terrace area to the rear of the extensions which would extend further than neighbouring properties however, there is already an element of overlooking from existing terrace areas and the proposed terrace is not considered significantly more harmful than the existing arrangement.

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO, see comments above.

- Access to sunlight and daylight; See comments above.

- Visual dominance and overbearing effects of a development; NO, the neighbouring property at 32 Westbourne Terrace will notice the additional height and depth of the proposals, particularly the side extension, however the proposal is only 0.7m deeper than what would be allowed under permitted development and there has been no neighbour objection.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time limit

- Materials

- Approved plans

Informatives relating to:

- Terms and conditions

2.	Building regulations

3.	Construction and demolition

4.	Encroachment and Party Wall

5.	Positive and proactive