DELEGATED APPLICATION REPORT 				Target Date:	 30/08/2016

Application No: 161253/HOU

Address: 24 Grove Hill

Proposal: Two storey side extension and front porch

Case officer: Heather Banks

Site Description and Surrounding Area:

The application property is a detached property, with a small front gable, located on a sub-road of Grove Hill. Grove Hill is a steeply sloped road and the application property is at a higher level than the properties on Grove Hill and Rotherfield Way which are located to the south of the property. The rear elevation of the application property faces out over these properties.

Proposal:

Planning permission is sought for the erection of a two storey side extension, to the side elevation of the property closest to 22 Grove Hill, and an enclosed front porch to replace the existing exterior porch.

It is proposed that the front and rear elevations of the two storey extension will match those of the existing property (excluding the front gable). The ridge line, eaves height and materials used in the external appearance of the extension will also match the existing property. The width of the extension will result in a 1m gap to the boundary with no. 22 at the front of the property and a 0.8m gap to the boundary at the rear of the property.

The proposed porch extension will have the same width as the existing porch, but will have a depth to match that of the front gable and a mono-pitch roof. It is also proposed that the two storey side extension will have a single storey front extension to it which will also have a depth to match the existing front gable and a mono-pitch roof with the same eaves height and maximum height as the proposed porch. The materials used in the external appearance of these elements will also match those of the existing property.

Plans Considered:

The agent submitted the following drawings, received on 5th July 2016:

Location Plan

Block Plan

Drawing No: KA.001 – Existing Elevations

Drawing No: KA.002 – Proposed Elevations

Drawing No: KA.003 – Existing Floor Plans

Drawing No: KA.004 – Proposed Floor Plans

Relevant Planning History:

None

Consultation Responses:

The RBC Ecologist stated that a Bat Survey would need to be submitted for this application. The agent was requested to submit a bat survey on the 20th July. The survey was received on the 17th August and the RBC Ecologist states that the bat survey had been carried out to an appropriate standard and concluded that the risk of bats being affected by the proposals was minimal.  There were therefore no objections to this application on ecology grounds.

RBC Transport Strategy stated that there were no transport objections as the proposed extension will have no transport implications as there are no changes to the existing parking arrangements and no impact on other road users.

Properties at St Agnes, The Flat St Agnes, 22 and 26 Grove Hill were consulted and no comments have been received.

Summary of discussions with applicant/agent:

The agent was advised on the 20th July that a Bat Survey had been requested by the RBC Ecologist. A Bat Survey was submitted on 17th August.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy, 2008 (altered 2015)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document, Oct 2012, (altered 2015)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; No. The proposal is considered to be in keeping with the proportions and design of the original property.

- reduce the amount of usable amenity space for the property to an unacceptable degree; No

- require the loss of trees in the garden or put significant trees at risk; No

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties;

No. The neighbouring property at 22 Grove Hill, has some side facing glazing at both ground and first floor levels, but as these either do not provide the only glazed opening for the room they serve, or they serve non-habitable rooms (eg. Bathroom), it is not considered that the proposed side extension, which will bring the application property closer to no. 22 (3m separation rather than 6m separation), will significantly reduce the daylight enjoyed by this property.

- invade the privacy of neighbouring properties through overlooking from windows or balconies;

No. There are no side facing windows proposed. Also, it is considered that given the existing relationship between the application property and those properties at a lower level to the south, and the separation distance between the application property and these properties, the proposed extension will not cause a reduction in the privacy currently enjoyed at these properties

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design;

No

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing;

No

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking;

No. The extension will sit 20m and 28m from the properties set at a lower level to the south, and there are no side windows proposed which could cause issues with the neighbouring property at No. 22.

- Access to sunlight and daylight;

No. The existing side windows at No. 22 are either secondary windows or windows to non-habitable rooms, therefore whilst the proposed side extension will reduce the light levels received at these windows, to some degree, it is not considered that this will cause significant detrimental harm to the living environment of the occupiers of No. 22.

- Visual dominance and overbearing effects of a development;

No. A gap of 3m will be maintained between the application property and No. 22 and the extensions will project no further to the front and rear than the existing property.

- Noise and disturbance;

No

- Artificial lighting;

No

- Vibration;

No

- Dust and fumes;

No

- Smell;

No

- Crime and safety;

No

RECOMMENDATION:

Grant

Conditions:

- The development to which this permission relates must be begun not later than the expiration of three years beginning with the date on which the permission was granted.

Reason: to prevent an accumulation of unimplemented planning permissions and in accordance with Section 91 of the Town & Country Planning Act 1990 as amended by the Planning and Compulsory Purchase Act 2004.

2. 	The materials to be used in the construction of the external surfaces of the development shall match those used on the existing building unless otherwise specified in the submitted documents.

Reason: To ensure that the new development appears in keeping with the existing building, in the interests of the character of the area, in accordance with Policy CS7 of the Reading Borough Local Development Framework Core Strategy 2008 (altered 2015) and Policy DM9 of the Sites and Detailed Policies Document 2012 (altered 2015).

- The permission relates only to the following plans and drawings:

Location Plan

Block Plan

Drawing No: KA.001 – Existing Elevations

Drawing No: KA.002 – Proposed Elevations

Drawing No: KA.003 – Existing Floor Plans

Drawing No: KA.004 – Proposed Floor Plans

Received 5th July 2016

Reason: To clarify which plans have been approved and in the interests of proper planning.

Informatives relating to:

  - Your attention is drawn to the terms and conditions of this permission. Any development which is carried out but which differs materially from the approved plans and details, or does not comply with any condition(s) or planning obligation(s) attached to the permission may result in the Council taking action to remedy the breach of planning control. If you are in any doubt please contact the Council.

  - The applicant’s attention is drawn to the requirement for submission of a Building Regulations application for the proposed development.  Please contact Building Control on 0118 937 2449 for advice.

  - The construction (and demolition) of the development will be subject to environmental health law and the applicant is referred to:

http://beta.reading.gov.uk/environmentalprotection

  - During the course of construction, the developer should take care to ensure that no part of this development (foundations, eaves, guttering, etc.) would encroach on, under or across the boundary line with the adjoining property, as the requisite Notice has not been served on the owner of that property.  This permission does not confer any right of access onto land that is not under the developer's control. The developer should also be mindful of the obligations and requirements of the Party Wall Act (1996) details of which can be found at: http://www.communities.gov.uk/publications/planningandbuilding/partywall

  - The decision to grant planning permission follows the positive and proactive consideration of the issues raised by this planning application and issuing the decision within the eight week time period.