DELEGATED APPLICATION REPORT 				Target Date:	26/04/2016

Application No: 160375/HOU

Address: 8 Haywood Way, Reading

Proposal: Part two storey side/rear extension, single storey side/rear extension.

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a two storey semi-detached property with a rear conservatory and an attached side garage.  The adjoining property at 7 Haywood Way has a two storey side extension and part two storey, part single storey rear extensions.  The neighbouring property at 9 Haywood Way is at a higher level than the application site and has a single storey rear extension and a detached rear garage along the boundary.  The surrounding area is predominantly residential with similar style semi-detached properties with bungalows located at the top of the cul-de-sac.

Proposal:

The application is for a part two storey side and rear extension and single storey side and rear extension.  The proposal includes a single storey front extension and ‘open’ porch.  The proposed side extension at ground floor level measures 3m in width (2.7m from the side elevation of the existing property) and extends 12.6m in length.  It is 0.15m from the boundary with the neighbouring property at 9 Haywood Way and extends 0.5m forward of the existing front elevation.  This element of the proposal has been amended and now has a mono pitch roof to the side elevation measuring 2.4m in height to eaves level and 3m in height at its highest point.  The proposed ‘open’ porch attaches to the front extension although it has an open side and front with a matching roof.  The proposed single storey element of the proposal wraps around the rear of the property and extends 5.3m in depth from the rear elevation.  The proposed single storey rear extension would be 0.15m from the boundary with the neighbouring property at 7 Haywood Way and would have a flat roof measuring 3m in height.

The proposed first floor side element of the proposal is set in from the boundary with the neighbouring property at 9 Haywood Way by 0.75m and set back from the existing front elevation of the property by 0.4m.  The proposal measures 2.1m in width and extends 10m in length.  The proposal is set down from the main roof of the existing property by 0.15m.  The proposal has the same eaves height and roof design as the existing property.  The proposed first floor element of the proposal wraps around the rear of the property and extends 3.6m in depth and has a hipped roof and is set down from the main roof by 0.4m.  This element of the proposal is set off the boundary with the neighbouring property at 7 Haywood Way by 2.3m.

The proposal has front and rear windows and doors with two first floor side windows facing 9 Haywood Way.  The amended drawings show these are obscure glazed and fixed up to 1.7m in height above internal floor level, this will be conditioned.  No ground floor side windows are proposed.  Two large roof lights are proposed to the single storey rear element of the proposal.

The existing property is brick to the ground floor and white render to the first floor.  The proposed materials consist of white rendered blockwork and brown concrete tiles.

Plans Considered:

The agent submitted the following drawings, received on 1st March 2016:

Drawing No: 01 – Existing Plans & Elevations

Drawing No: 02 – Proposed Plans & Elevations

Design and Access Statement

The agent submitted the following amended drawing, received on 23rd April 2016:

Drawing No: 02 Rev A – Proposed Plans & Elevations

Relevant Planning History:

None.

Consultation Responses:

Ecology raised no objection.

Transport raised no objection.

Properties at 7 and 9 Haywood Way and 20 and 22 Fontwell Drive were consulted.  Two letters of representation have been received with regards to:

- Loss of light to side elevation of 9 Haywood Way and rear of 7 Haywood Way.

- The proposal will create a wind tunnel.  Officer Note: there is a distance of approximately 4m between the boundary with the application site and 9 Haywood Way and there is an existing single storey garage along the boundary with this neighbouring property. Due to this separation distance and as there is an existing structure, albeit single storey, along the boundary the proposal at two storey is not considered to create a wind tunnel that would be harmful to this neighbouring property.

- All other properties seem to be only one storey from the back of the original house.  Officer note: there are two storey rear extensions at 5, 6, 7 and 32 Haywood Way which are within close proximity to the application site but notwithstanding this each application is taken on its own merits and planning decisions made against relevant planning policy and guidance.

- Proposed side windows will overlook neighbouring property.

- The proposed rear extension will extend at least 2m further than the neighbouring extension which will be excessive and overbearing.

- Concerns that foundations will be detrimental to the stability of the neighbouring decking and fence panels.  Officer Note:  this is a Party Wall matter and the applicant will be advised of their obligation under the Party Wall Act.

- Proposals for extending forward of 7 Haywood Way have previously been refused but this previous proposal is similar to that proposed at 8 Haywood Way.  Officer Note: each application is taken on its own merits however, having looked briefly at the previous proposals at 7 Haywood Way there are clear differences in the proposals and the context of each site.  7 Haywood Way is at a higher level than its neighbouring property at 6 Haywood Way whilst 8 Haywood Way is at a lower level than its neighbouring property at 9 Haywood Way and therefore the impact on the neighbouring properties is different.

Summary of discussions with applicant/agent:

The agent was advised in an email dated 22nd April 2016 that there was no objection to the proposal in principle however a concern was raised with regards to the single storey side element.  This part of the proposal had a flat roof that extended for approximately 12m at 3m in height.  This part of the proposal would be clearly visible from the road and it was felt that a mono pitch roof with a lower eaves height would be more acceptable and less overbearing on the neighbouring property at 9 Haywood Way.  It was also suggested that the height of the porch roof match the height of the single storey front extension so it is seen as one and therefore better in terms of appearance.  Amended drawings were submitted on 23rd April 2016.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

CS36 (Biodiversity and Geology)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, the proposal meets general policy and guidance as the first floor side element of the proposal is set back from the existing front elevation of the property and the extensions are set down from the main roof.  Although the rear extension measures 5.3m in depth which is larger than the 4m recommended in the Council’s SPG it is considered that the property and garden are capable of taking an extension of a longer depth.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; There may be some loss of direct sunlight to the side elevation of the neighbouring property at 9 Haywood Way due to the orientation of the properties however, 9 Haywood Way is at a higher level than the application site and there is a gap of approximately 4m between the side boundary and the side elevation of this neighbouring property and the loss of light is not considered to significantly reduce the amount of light to this neighbouring property.  The proposed drawing also shows a line taken at 45 degrees from the closest window at 7 Haywood Way and the proposed two storey rear extension does not encroach on this line which protects the light to the rooms in this neighbouring property.  A concern has been raised with regards to loss of light to the rear of 7 Haywood Way however the proposed rear extension is approximately 1.6m deeper than the extension at this neighbouring property and due to the orientation of these properties there is not considered to be any significant loss of light to the rear of this neighbouring property.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO, although two first floor side windows are proposed facing 9 Haywood Way a condition will be imposed which requires these windows to be obscure glazed and fixed up to 1.7m in height above internal floor level.

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO, see above.

- Access to sunlight and daylight; NO, see above.

- Visual dominance and overbearing effects of a development; NO, although the neighbouring properties at 7 and 9 Haywood Way will notice the addition height and depth of the extensions the proposal is not considered to be visually dominant or overbearing.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time limit

- Materials

- Approved plans

- First floor side windows to be obscure and fixed up to 1.7m in height.

Informatives relating to:

- Terms and conditions

2.	Building regulations

3.	Construction and demolition

4.	Encroachment and Party Wall

5. 	Positive and proactive