DELEGATED APPLICATION REPORT 				Target Date:	19/07/2016

Application No: 160954/HOU

Address: 6 Clevedon Road, Reading

Proposal: 2 storey rear extension and single storey rear extension. Front porch

and bay window. Loft conversion with 2 rear dormer windows and a

velux window.

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a two storey detached property with a two storey front projection.  The main entrance to the property is along the side elevation and historically the property had been divided into two flats.  The property has a large rear garden.  The property is set back from the neighbouring property at 10 Clevedon Road and the neighbouring property at 4 Clevedon Road is located at an angle on its corner plot.  The surrounding area is predominantly residential of mixed styles and designs.

Proposal:

The proposal involves a single storey front extension to the side of the two storey front projection to create a new front entrance and a bay window to the ground floor family room.  There are a few alterations to windows and doors along the side elevation along with a two storey rear extension, a single storey rear extension, 2 rear pitched roof dormer windows and a roof light.

The materials will consist of brick and tiles to match the existing property.

Plans Considered:

The applicant submitted the following plans and drawings, received on 20th May 2016:

Sheet 2 of 4 – Plan: Existing And Proposed Plans

Sheet 3 of 4 – Existing Roof Plan And Elevations

Sheet 4 of 4 – Proposed Elevations

The applicant submitted the following plan, received on 24th May 2016:

Sheet 1 of 4 – Site Location & Site Block Plan

The applicant submitted the following information, received on 5th July 2016:

Initial Bat Assessment, by eaves ecology, dated 01/07/2016, ref: 16-005-IBA.

Relevant Planning History:

94/00492/FUL (Civica Ref: 940729) - New chimney on west elevation.  Permitted 11/07/1994.

160187/PREAPP – Pre-application advice for single storey front extension, single storey and two storey rear extensions, rear dormer window and various alterations and additions to windows and doors.  Observations sent 04/03/2016.

Consultation Responses:

Ecology raised no objection following the submission of a Bat Survey.

Transport raised no objection.

Properties at 4, 10 and 12 Clevedon Road, 2 Carlisle Road and 995A, 995B, 997 and 999 Oxford Road were consulted.  One concern was raised by telephone with regards to overlooking from the proposed dormer windows.

Summary of discussions with applicant/agent:

None.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

CS36 (Biodiversity and Geology)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal

The main issues to be considered are:

- Design and impact on the character of the area

- Impact on neighbours

a) Design and impact on the character of the building and area

The proposed front extension would be located to the side of the two storey projection and measures 2m in width and 2.6m in depth.  The proposal would attach to an existing single storey front element and the hipped roof would remain as existing.  This element of the proposal would become a new front entrance porch.  A front bay window is also proposed to the existing family room measuring 0.5m in depth.  These proposals are subservient to the existing property and are not considered to detract from the character and appearance of the property and are considered acceptable.

The proposed two storey rear extension measures 1m in depth and 2.9m in width.  The proposal would be located to the left hand side of the rear elevation when viewed from the garden.  The proposal would have a hipped roof and would be set down from the existing roof by 1.7m.  The proposed single storey rear extension would be located to the right hand side of the rear elevation when viewed from the garden and measures 1m in depth and 2.8m in width.  The two storey rear extension follows general policy and guidance and the depth of both rear extensions are such that they are not considered to detract from the character and appearance of the property and are considered acceptable.

The proposed pitched roof rear dormer windows are set within the roof slope which meets the Council’s guidance in the Design Guide to House Extensions SPG.  The dormer windows line up with the windows in the rear elevation of the property and they would appear to be permitted development.  The proposed dormer windows are not considered to detract from the character and appearance of the property and are considered acceptable.

b) Impact on neighbouring properties

The neighbouring property at 10 Clevedon Road is at a higher level than 6 Clevedon Road and the proposals are not considered to impact on the residents of this neighbouring property in terms of loss of light, visual dominance or overbearing effects.  The neighbouring property at 4 Clevedon Road will notice the additional height and depth of the two storey extension however there is sufficient distance between both properties and the proposed two storey extension would be adjacent to the side elevation of this property.

A concern has been raised with regards to overlooking from the rear dormer windows however the application property extends 6.5m further back from the rear elevation of the neighbouring property at 10 Clevedon Road and is at a lower level.  Although the proposed dormer windows will allow views to the rear there is already some degree of overlooking from the first floor rear windows and taking into account that 6 Clevedon Road is at a lower level and further back on the site than 10 Clevedon Road the proposed dormer windows are not considered to cause a significant detrimental impact to the living environment of the residents at 10 Clevedon Road in terms of overlooking and loss of privacy.

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time limit

- Materials

- Approved plans

Informatives relating to:

- Terms and conditions

2.	Building regulations

3.	Construction and demolition

4.	Encroachment and Party Wall

5. 	Positive and proactive