DELEGATED APPLICATION REPORT 			Target Date:	14/09/2016

Application No: 161334/HOU

Address: 21 Greenfields Road, Reading

Proposal: Single storey front and rear extensions

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a two storey semi-detached property with a projecting two storey rear element and two single storey rear extensions adjacent to each other.  The surrounding area is predominantly residential.

Proposal:

The application involves a single storey front extension to create a porch and a single storey rear extension that replaces one of the existing extensions and which extends from the rear of the other existing extension.

The proposed porch measures 1.5m in depth, 3.5m in width, 2.3m in height to eaves level and 3.2m in height at its highest point.  The proposed porch would have a mono pitch roof with a door and window to the front.  There are other examples of similar porches within the road.

The proposed single storey rear extension would measure 4.7m in depth adjacent to the side boundary with 19 Greenfields Road and an additional 3m in depth from the existing extension along the boundary with 23 Greenfields Road.  The proposal would have a mono pitch roof and measures 2.3m in height to eaves level and 3.7m in height at its highest point.

The materials will consist of brick and tiles to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 17th July 2016:

Drawing No: P/16/112/002 – Existing Plans

Drawing No: P/16/112/003 – Proposed Plans

The agent submitted the following plan, received on 18th July 2016:

Drawing No: P/16/112/001 – Location, Block and Site Plan

Relevant Planning History:

90/00266/FUL (Civica Ref: 900919) - New entrance lobby and porch. 1st floor extension over existing ground floor extension and new two storey extension at rear. Replacement of existing garage.  Permitted 26/06/1990.

Consultation Responses:

Transport raised no objection subject to an informative.

Properties at 19, 23 and 44 Greenfields Road were consulted and no comments have been received.

Summary of discussions with applicant/agent:

None.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7: Design and the Public Realm

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, although the rear extension exceeds the Council’s guidance of 4m the neighbouring property at 19 Greenfields Road has an extension to a similar depth and there is a footpath between both properties.  The proposed rear extension is approximately 3m deeper than the extension at 23 Greenfields Road.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO, the neighbouring property at 23 Greenfields Road has an existing extension along the boundary and the proposed extension will extend approximately 3m deeper which is considered acceptable in terms of its impact on this neighbouring property.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO

- Visual dominance and overbearing effects of a development; NO, the neighbouring properties will notice the additional height and depth of the proposals however the extensions are not considered visually dominant or overbearing.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time limit

- Materials

- Approved plans

Informatives relating to:

- Terms and conditions

2.	Building regulations

3.	Construction and demolition

4.	Encroachment and Party Wall

5.	Damage to the highway

6. 	Positive and proactive