DELEGATED APPLICATION REPORT - Application No: 152217

Address: 23 St Peters Avenue

Proposal: Single storey rear extension

Case officer: Jasmine Singh

Site Description and Surrounding Area: The application relates to 23 St Peters Avenue.

The property has been extended previously and benefits from wider plot area with scope to facilitate additional extension. The properties in the vicinity vary in terms of design and architectural composition.

Proposal:

Permission is sought for erection of ‘L’ shape single storey rear extension measuring (side infill) 5.6m deep, 7.3m wide, 1.7deep ( towards the rear), 3.4m high (max) and 2.8 eaves level with pitched roof.

Plans Considered:

Site Plan

Location plan

Existing floor plans: 31415-01(received by LPA on 10 December 2015)

Existing elevation: 31415-02(received by LPA on 10 December 2015)

Proposed floor plan: 31415-03(received by LPA on 14 December 2015 with amended title)

Proposed elevation: 31415-04 (received by LPA on 10 December 2015)

Relevant Planning History:

01-01344-FUL: Two storey rear extension: Application permitted: 6/12/2001

01-00652-FUL: Single storey rear extension: Application permitted: 28/ 6/2001

Consultation Responses:

Neighbours: No representation received.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise.  Material considerations include relevant policies in the National Planning Policy framework (NPPF) - among them the 'presumption in favour of sustainable development'.  However the NPPF does not change the statutory status of the development plan as the starting point for decision making (NPPF paragraph 12).

In this regard, the NPPF states that due weight should be given to the adopted policies of the Local Development Framework (LDF) (Core Strategy and Sites and Detailed Policies Document) according to their degree of consistency with the NPPF (the closer the policies in the plan to the policies in the NPPF, the greater the weight that may be given).

Accordingly, the National Planning Policy Framework and the following development plan policies and supplementary planning guidance are relevant:

### Reading Borough LDF Core Strategy (Adopted January 2008) (Altered 2015)

CS7     Design and the Public Realm

Sites and Detailed Policies Document – (Adopted October 2012) (Altered 2015)

DM4    Safeguarding Amenity

DM9    House Extensions and Ancillary Accommodation

### Supplementary Planning Guidance/Documents

SPG – A Design Guide to House Extensions

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building;  NO

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

The details and of the proposed extension in terms of design, style, materials and fenestration pattern is in keeping with the host dwelling.

The proposed extension would be in keeping with character of the host dwelling

and it would be indiscernible from the streets scene.

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

The proposed extension is set in from the boundaries of the adjoining properties and height of the proposed structure would balance with existing boundary treatment and therefore it is not considered to have detrimental impacts on the adjoining properties.

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO

- Visual dominance and overbearing effects of a development; NO

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fume; NO

- Smell; NO

The fenestration pattern with rear openings does not afford view into neighbouring properties.

RECOMMENDATION:

GRANT

The proposed development would not be harmful to the character or appearance of the site or its surroundings and would not have a detrimental impact on the original building or neighbouring properties.  As such the proposed works are considered to be in accordance with CS7 (Design and the Public Realm) of the Reading Borough Local Development Framework Core Strategy (2008), DM4: Safeguarding Amenity and DM9: House Extensions and Ancillary Accommodation of the Sites and Detailed Policies Document (Oct 2012) and the Councils Supplementary Planning Guidance A Design Guide to House Extensions (2003).

Conditions:

- The development to which this permission relates must be begun not later than the expiration of three years beginning with the date on which the permission was granted.

Reason: to prevent an accumulation of unimplemented planning permissions and in accordance with Section 91 of the Town & Country Planning Act 1990 as amended by the Planning and Compulsory Purchase Act 2004.

- The materials to be used in the construction of the external surfaces of the development shall match those used on the existing building.

Reason: as no details of the materials have been submitted and in the interests of ensuring that the new development appears in keeping with the existing building and the character of the surrounding area, in line with Policy CS7 of the Reading Borough LDF Core Strategy 2008 (Altered 2015) and Policy DM9 of the Sites and Detailed Policies Document 2012 (Altered 2015).

- The permission relates only to the following plans and

Site Plan

Location plan

Existing floor plans: 31415-01(received by LPA on 10 December 2015)

Existing elevation: 31415-02(received by LPA on 10 December 2015)

Proposed floor plan: 31415-03(received by LPA on 14 December 2015 with correct title )

Proposed elevation: 31415-04 (received by LPA on 10 December 2015)

Reason: to clarify which plans have been approved and in the interests of proper      planning.

Informatives:

- These works will require separate approval under the Building Regulations.  Please contact the Council's Building Control service for further advice on (0118) 937 2449.

- Your attention is drawn to the terms and conditions of this permission. Any development which is carried out but differs materially from the approved plans and details, or does not comply with any condition (s) or planning obligation (s) attached to the permission may result in the Council taking action to remedy the breach of planning control. If you are in any doubt please contact the council.