Delegated Application Report 						Target Date: 29/08/16

Address: 132 St Michael’s Road

Application No: 161234/CLP

Proposal: Certificate of Lawfulness for proposed hip-to-gable roof alteration and rear box dormer

Case Officer: Tom Hughes

Introduction and Proposal

The application site is a semi-detached property with a hipped roof and a two storey front gable. A chimney is located of the rear roof pitch. At the rear of the property is a single storey gable with a mono pitch roof. There is gated access from the rear garden onto The Meadway. The surrounding area is predominantly residential.

The applicant proposes a hip-to-gable roof alteration and a rear box dormer with a depth of 3.5m. The dormer would extend across the width of the property by 5.2m, set in from the proposed side elevation by 0.4m. The dormer would be set down from the existing ridgeline by 1.2m and set back from the eaves by 0.3m.

The proposal would include two skylights on the front elevation.

Plans Submitted

The agent supplied the following drawings, received 3rd July 2016:

Drawing No: 1 – Proposed Ground Floor Plan

Drawing No: 2 – Proposed First Floor Plan

Drawing No: 3 – Proposed Loft Plan

Drawing No: 4 – Proposed Elevations

Drawing No: 5 – Building Section

Site Location

Block Plan

Relevant Planning History

None.

Summary of Consultations

No consultations were undertaken as it is not required for an application of this type.

Relevant Planning Policy and Guidance

Annex 8 "Lawfulness and the Lawful Development Certificate" of Planning Circular 10/97 'Enforcing Planning Control: Legislative Provisions and Procedural Requirements" sets out the procedure for assessing applications for certificates of lawfulness:

Para 8.26 states, "if the LPA are supplied with information satisfying them that the operations described in the application would be lawful, if instituted or begun at the time of the application, they shall issue a certificate to that effect; and, in any other case, they shall refuse the application." "The burden of proof in a LDC application is firmly on the applicant."

The Town and Country Planning (General Permitted Development) (England) Order 2015 (GPDO)

Permitted development rights for householders – Technical Guidance April 2016

A Design Guide to House Extensions (Supplementary Planning Guidance)

Assessment

A Certificate of Lawfulness for a proposed development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

The proposed hip to gable roof alteration and rear box dormer must be assessed to ensure that it complies with the conditions of Class B and Class C, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015, which relates to the ‘enlargement of a dwellinghouse consisting of an addition or alteration to its roof’ and ‘any other alteration to the roof of a dwellinghouse’.

The relevant conditions of the GPDO for a hip to gable roof alteration and rear box dormer, which is not listed or in a conservation area, are as follows:

B.1 Development is not permitted by Class B if:

- B.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); complies

- B.1 (b) any part of the dwellinghouse would, as a result of the works, exceed the height of the highest part of the existing roof; complies

- B.1 (c) and part of the dwellinghouse would, as a result of the works, extend beyond the plane of any existing roof slope which forms the principle elevation of the dwellinghouse and fronts a highway; complies

- B.1 (d) the cubic content of the resulting roof space would exceed the cubic content of the original roof space by more than (i) 40 cubic metres in the case of a terrace house, n/a or (ii) 50 cubic metres in any other case; complies, the calculated volume would be 36.63 cubic metres

- B.1 (e) it would consist of or include (i) the construction or provision of a verandah, balcony or raised platform, complies or (ii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe; complies or

- B.1 (f) the dwellinghouse is on article 2(3) land. complies

B.2 Development is permitted by Class B subject to the following conditions:

- B.2 (a) the materials used in any exterior work must be of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse; complies

- B.2 (b) the enlargement must be constructed so that (i) other than in the case of a hip-to-gable enlargement or an enlargement which joins the original roof to the roof of a rear or side extension (aa) the eaves of the original roof are maintained or reinstated; n/a and (bb) the edge of the enlargement closest to the eaves of the original roof is, so far as practicable, not less than 0.2 metres from the eaves; n/a and (ii) other than in the case of an enlargement which joins the original roof to the roof of a rear or side extension, no part of the enlargement extends beyond the outside face of any external wall of the original dwellinghouse; complies and

- B.2 (c) any window inserted on a wall or roof slope forming a side elevation of the dwellinghouse must be (i) obscure-glazed, complies and (ii) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed. complies

C.1 Development is not permitted by Class C if:

- C.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); complies

- C.1 (b) the alteration would protrude more than 0.15m beyond the plane of the slope of the original roof when measured from the perpendicular with the external surface of the original roof; complies

- C.1 (c) it would result in the highest part of the alteration being higher than the highest part of the original roof; complies or

- C.1 (d) it would consist of or include (i) the installation, alteration or replacement of a chimney, flue or soil and vent pipe, complies or (ii) the installation, alteration or replacement of solar photovoltaics or solar thermal equipment. complies

C.2 Development is permitted by Class C subject to the condition that any window located on a roof slope forming a side elevation of the dwellinghouse must be:

- C.2 (a) obscure glazed; complies and

- C.2 (b) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed. complies

At the time of the site visit on 22nd July 2016, works to remove the existing roof had begun.

Recommendation

Sufficient evidence has been provided to demonstrate that a Certificate of Lawfulness should be granted for the proposed hip-to-gable roof alteration and rear box dormer with a maximum depth of 3.5m and width of 5.2m, with a height of 2.2m and set down from the existing ridgeline by 1.2m and set back from the existing eaves by 0.3m.

Conditions

- The proposal has been assessed against the Town and Country Planning Act 1990 (as amended) and Class B and Class C, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015. It can be confirmed that the proposed operation would be lawful for planning purposes on the basis of the drawings received on 3rd July 2016.

Drawing No: 1 – Proposed Ground Floor Plan

Drawing No: 2 – Proposed First Floor Plan

Drawing No: 3 – Proposed Loft Plan

Drawing No: 4 – Proposed Elevations

Drawing No: 5 – Building Section

Site Location

Block Plan

Therefore the Local Planning Authority can issue the applicant with a Certificate of Lawfulness for the proposed development.

Informatives

1. The applicant should check if submission of a Building Regulations application is required for the proposed development. Please contact Building Control on 0118 937 2449 for advice.

- Householder Prior approval was granted under application reference 161234  (03/07/16) for a hip-to-gable roof alteration and rear box dormer with a maximum depth of 3.5m, a maximum height of 2.2m and a maximum width of 5.2m. The box dormer would be set down from the existing ridgeline by 1.2m and set back from the existing eaves by 0.3m. This CLP is obtaining confirmation that that proposal is lawful.

- During the course of construction, the developer should take care to ensure that no part of this development (foundations, eaves, guttering, etc.) would encroach on, under or across the boundary line with the adjoining property, as the requisite Notice has not been served on the owner of that property.  This permission does not confer any right of access onto land that is not under the developer's control. The developer should also be mindful of the obligations and requirements of the Party Wall Act (1996) details of which can be found at:

www.communities.gov.uk/publications/planningandbuilding/partywall

- The construction (and demolition) of the development will be subject to environmental health law and the applicant is referred to:

http://beta.reading.gov.uk/environmentalprotection