DELEGATED APPLICATION REPORT - Application No.: 151439

Address: 16 Amity Road

Proposal: Construction of double storey extension to the side and single storey

extension to the rear

Case officer: Alison Amoah

Site Description and Surrounding Area:

The site comprises an end of terrace property currently rented to a family.  It has an existing side/ rear lean-to perspex extension, with the front door within it, which is to the boundary with no. 14.  The entrance door is set back from the front elevation by 6.3m.  There is also an existing single storey brick built rear extension which forms the bathroom.

Proposal: Removal of the existing lean to and the erection of a two storey side extension at the same height as the existing dwelling at a width of 1.8m wide, set back by 0.2m, set off the boundary to no. 14 by 0.15m, and with an overall depth of 8.8m.  Beyond this a single storey extension, which extends beyond the existing rear wall of the application property by 1.1m.  This attaches to a proposed single storey rear extension of 5.8m deep, and 2.4m wide, i.e. filling in the existing ‘l’ area.  This would be set off the boundary to no. 14 by 1.2m.  This is proposed to be flat roofed at a height of 2.3m.

The proposed development would create one additional bedroom at ground floor (total of 4 bedrooms) in place of the existing lounge, an enlarged front ground floor bedroom and ensuite, two enlarged bedrooms at first floor, both with the addition of en-suites, a shower room at ground floor and enlarged kitchen/dining area.

A roof light is proposed over ground floor rear bedroom (no.2) and above the kitchen.  At the rear one first floor and three ground floor windows are proposed.

Materials proposed are to match existing.

Plans considered:

Site Location Plan and Block Plan – Drawing no: A-1000 Rev A, received 12th August 2015

Existing and Proposed Elevations – Drawing no: A-1020 Rev B, received 19th January 2016

Existing and Proposed Floor and Roof Plans – Drawing no: A-1010 Rev B, received 9th October 2015

Relevant Planning History:

None

Consultation Responses:  Nos. 14 & 18 Amity Road were consulted.  Two responses received with objections as follows:

- Communal area is limited

- Increased noise levels

- Parking is already problematic

- Number of residents could cause a greater load on public transport

- Issues in the past with safe disposal of rubbish

- Considering the traditional family occupancy of other houses in this neighbourhood concern that the development will be of detriment to the area.

- Loss of gap will be to the detriment of local character

- The side extension changes the character of the road

- Maintenance of with side wall (of no14, and no. 16) will be impossible

- The rear extension is sprawling and out of character with other extensions along the road reducing their garden and light to no. 14.

- The length of the extension is in excess of the Council’s guidance of 4m and ignores the guidelines that rear extensions should be located as far away as possible from side boundaries.

- Light to no. 14’s basement bedroom will be adversely affected, as well as the lounge and kitchen

- Privacy at no. 14 will  be reduced by the two storey extension adding a first floor window, which will be much closer to no. 14

- There will be disruption from building work and concerns as to the structural integrity of no. 14 being compromised

Transport and Ecology had no objections.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise.  Material considerations include relevant policies in the National Planning Policy framework (NPPF) - among them the 'presumption in favour of sustainable development'.

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7: Design and the Public Realm

Sites and Detailed Policies Document (2012)

DM9: House Extensions and Ancillary Accommodation;

DM4: Safeguarding Amenity

Supplementary Planning Guidance/ Documents:

A Design Guide to House Extensions (2003)

Appraisal Checklist:

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- Detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; No – although the proposal has no set down, the view was taken that retaining the ridge height for the side extension, which would only be 1.8m in width would have a better appearance within the street scene.  The proposal would reintroduce the traditional location of a front door.

A concern was raised that length of the extension is in excess of the Council’s guidance of 4m.  Although the proposed extension would be 5.7m in depth, it would essentially fill in the area adjacent to the existing rear extension, and would not extend further than this.  Through an amended plan it has been pulled off the boundary by 1.2m which is considered a suitable set off the boundary for the proposed single storey element of the scheme.

- Reduce the amount of usable amenity space for the property to an unacceptable degree; No

- Require the loss of trees in the garden or put significant trees at risk; No

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- Significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; No – Objections have been raised by no. 14 that the proposal would reduce natural daylight to no. 14 and in particular the basement room.  The two storey extension does not extend beyond the rear wall of no. 14.  The majority of the single storey element would be set off the boundary to no. 14 by 1.2m at a height of 2.3m.  This compares to an existing lean to conservatory which is to the boundary.  It is not considered that this element of the proposal would have a significant effect on the light to no. 14 and especially since the application site is to the north of no. 14.  The officer’s contention is that the basement room would already have limited natural daylight entering it and that the proposal would not create any significant change to this.

- Invade the privacy of neighbouring properties through overlooking from windows or balconies; No – Objection has been raised that the two storey extension will have a first floor window closer to no. 14, which would affect privacy.  Although the proposed window would be closer, the view possible from this window as compared to the existing is not considered so significantly different as to warrant refusal on this basis.

- Detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; No – Neighbours are concerned that the rear extension would be sprawling and out of character with other extensions along the road.  The officer’s view is that the proposed extension is of an appropriate scale, which essentially fills in an area to the rear within the existing ‘l’ and does not encroach any further into the garden than the existing built form.

There are very few gaps between properties in the road and the loss of this gap is not considered to be detrimental to the overall character of the road which is largely comprised of a continuous row of terraces.  The side extension is of a limited scale, which is not considered will change the character of the road.

- Detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; No

Other matters raised through consultation

- Communal area is limited – the proposal will create what appears to be a C4 small HMO, for which there is no specific policy requirement for a particular size of communal area and indeed the change of use of the existing dwelling from a C3 dwelling to C4 small HMO would be permitted development in this specific location (if indeed this hasn’t already occurred).  The proposed kitchen/dining area is 21m2 which is considered adequate to serve the proposed increase to four bedrooms.

- Increased noise levels – The proposal would increase the number of bedrooms by one.  It is not considered that this would have a significant impact on the level of noise as compared to a fully occupied family home/ rented family home and an increase in the numbers of occupants could occur without the requirement for planning permission.

- Parking is already problematic – There is a parking permit scheme in operation on the street and the number of permits available to each property would be determined by the parking team of the Council, which generally would be two on a first come first served basis. Therefore any changes to the number of people should have no significant impact on the level of parking.

- Number of residents could cause a greater load on public transport – The additional number of residents would be negligible in this regard.

- Issues in the past with safe disposal of rubbish – This is not a planning matter

- Considering the traditional family occupancy of other houses in this neighbourhood concern that the development will be of detriment to the area – There could be an increase in numbers occupying the existing property without any requirement for planning permission.

- Maintenance of with side wall (of no14, and no. 16) will be impossible – This is not a planning matter. An informative referring the developer to the Party Wall Act is recommended.

- There will be disruption from building work and concerns as to the structural integrity of no. 14 being compromised – an informative is included regarding construction and demolition.

RECOMMENDATION:

APPROVE

- The development to which this permission relates must be begun not later than the expiration of three years beginning with the date on which the permission was granted.

Reason: to prevent an accumulation of unimplemented planning permissions, and in accordance with Section 91 of the Town & Country Planning Act 1990 as amended by the Planning and Compulsory Purchase Act 2004.

2)	This permission relates only to the following plans:

Site Location Plan and Block Plan – Drawing no: A-1000 Rev A, received 12th August 2015

Existing and Proposed Elevations – Drawing no: A-1020 Rev B, received 19th January 2016

Existing and Proposed Floor and Roof Plans – Drawing no: A-1010 Rev B, received 9th October 2015

3)	The materials to be used in the construction of the external surfaces of this development shall be as stated in the application dated 6th August 2015.

Reason: To ensure that the new development appears in keeping with the existing building.

Adopted Core Strategy 2008:

CS7 – Design and Public Realm

LDF Sites and Detailed Policies Document (2012) policies:

DM9 - House Extensions and Ancillary Accommodation

Informatives:

Your attention is drawn to the terms and conditions of this permission. Any development which is carried out but which differs materially from the approved plans and details, or does not comply with any condition(s) or planning obligation(s) attached to the permission may result in the Council taking action to remedy the breach of planning control. If you are in any doubt please contact the Council.

The applicant’s attention is drawn to the requirement for submission of a Building Regulations application for the proposed development.  Please contact Building Control on 0118 937 2449 for advice.

During the course of construction, the developer should take care to ensure that no part of this development (foundations, eaves, guttering, etc.) would encroach on, under or across the boundary line with the adjoining property, as the requisite Notice has not been served on the owner of that property.  This permission does not confer any right of access onto land that is not under the developer's control.  The developer should also be mindful of the obligations and requirements of the Party Wall Act (1996) details of which can be found at http://www.communities.gov.uk/publications/planningandbuilding/partywall

The construction and demolition of the development will be subject to environmental health law and the applicant is referred to

http://www.reading.gov.uk/businesses/EnvironmentalProtection/construction-demolition

Occupation of the building by between 3 and 6 unrelated people sharing one or more basic facilities would be a change from Class C3 to C4. This may be permitted development in this instance, although to receive formal determination of this would require the submission of an application for a Lawful Development Certificate.  Should the number of occupants exceed six persons then this would be classed as a large ‘Sui Generis’ HMO and would require planning permission.

The decision to grant planning permission follows the positive and proactive consideration of the issues raised by this planning application.  This included an amended design to reduce the scale of the rear extension.