DELEGATED APPLICATION REPORT 				Target Date:	 18/05/2016

Application No: 160561/HOU

Address: 8 Shakespeare Close

Proposal: Single storey side extension

Case officer: Heather Banks

Site Description and Surrounding Area:

The application relates to a two storey semi-detached dwelling which fronts Shakespeare Close.  Surrounding dwellings are residential in nature and characterised by semi-detached pairs and short terraces with a similar architectural style to the subject property.

Proposal:

The application is for a single storey side extension.

It is proposed that the side extension will have a width of 3.3m and a depth of 9.3m. It will extend the full depth of the property and project forward of the existing front elevation of the property at this side to be level with and join to the existing front elevation of the porch and kitchen. It is proposed that the extension will have a hipped roof with an eaves height of 2.3m, to match that of the existing porch and kitchen front projection, and a maximum height of 3.6m. The pitch of the front facing roof slope will match that of the existing single storey front projection. It is proposed that there will be a window to the front elevation of the extension, to match the existing front facing kitchen window, and glazed doors to the rear elevation. It is also proposed that there will be two skylights fitted in the side facing roof slope.

It is proposed that the materials to be used will match those of the existing property.

Plans Considered:

The agent submitted the following drawings, received on 28th June 2016

Drawing No: CANDISH-0316-05 – Block and Location Plans

Drawing No: CANDISH-0316-03 rev. B – Existing Plans and Elevations

Drawing No: CANDISH-0316-01 rev. B – Proposed Ground floor plan

Drawing No: CANDISH-0316-02 rev. B – Proposed Elevations

Drawing No: CANDISH-0316-04 rev. B – Section A-A

Relevant Planning History:

152258/PREAPP – Two storey side extension. Response provided 08/03/2016

Consultation Responses:

Properties at 7 Shakespeare Close and 10 Redberry Close were consulted and one response was received.

Originally this application was for a two storey side extension of the same width and slightly reduced depth. An objection was made expressing concerns that such a two storey extension would reduce the light into the property at 10 Redberry Close, would be overbearing on this property and would be located unreasonably close to the boundary with this property.  The case officer agreed with these concerns.   Following the amendment of this development to be a single storey extension, the objector has been notified and no further objections have been raised.

Summary of discussions with applicant/agent:

On the 5th May the applicant was advised, by email, that the application, which at this point was for a two storey side extension, would be likely to be refused due to the unacceptable overbearing effects it would have on the neighbouring property at 10 Redberry Close. Following discussions with the applicant over email a meeting was held at the application property on the 27th May. Following this meeting plans were submitted for consideration for a reduced, single storey extension. The final iteration of these drawing was received on 8th June. On the 23rd June the agent was requested to supply an amended block plan for this revised scheme, as one had not yet been submitted.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy, 2008 (altered 2015)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document, Oct 2012, (altered 2015)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; It is considered that the hipped nature of the proposed roof would be some-what out of character with the pitched roofs of the application property and its neighbours. However, given that the front and rear faces of the extension will be of the same pitch as the roof of the main house, the materials of the extension will match the main property and the property is set back from the road, it is not considered that this would harm the appearance of the application property to a degree that would warrant refusal.

- reduce the amount of usable amenity space for the property to an unacceptable degree; No

- require the loss of trees in the garden or put significant trees at risk; No

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; No.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; No

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; No. The single storey nature of the extension and its hipped roof form reduce the impact of the extension on the property at 10 Redberry Close

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing;

No. It is considered that given the single storey and subservient nature of the proposed extension, and that it will match existing materials and roof pitches, it is not considered that the uncharacteristic hipped roof will not substantially detract from the appearance of the street.

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking;

No.

- Access to sunlight and daylight;

No

- Visual dominance and overbearing effects of a development;

No. Although the proposed side extension will be located close to the boundary with 10 Redberry Close and will be located only 8.5m from the rear elevation of that property, it is considered that by virtue of its single storey nature and hipped roof form, the proposed extension will not appear visually dominant and overbearing to the occupiers of 10 Redberry Close.

- Noise and disturbance;

No

- Artificial lighting;

No

- Vibration;

No

- Dust and fumes;

No

- Smell;

No

- Crime and safety;

No

RECOMMENDATION:

Grant

Conditions:

- The development to which this permission relates must be begun not later than the expiration of three years beginning with the date on which the permission was granted.

Reason: to prevent an accumulation of unimplemented planning permissions and in accordance with Section 91 of the Town & Country Planning Act 1990 as amended by the Planning and Compulsory Purchase Act 2004.

2. 	The materials to be used in the construction of the external surfaces of the development shall match those used on the existing building as specified in the submitted documentation for this application.

Reason: To ensure that the new development appears in keeping with the existing building, in the interests of the character of the area, in accordance with Policy CS7 of the Reading Borough Local Development Framework Core Strategy 2008 (altered 2015) and Policy DM9 of the Sites and Detailed Policies Document 2012 (altered 2015).

- The permission relates only to the following plans and drawings:

Drawing No: CANDISH-0316-05 – Block and Location Plans

Drawing No: CANDISH-0316-03 rev. B – Existing Plans and Elevations

Drawing No: CANDISH-0316-01 rev. B – Proposed Ground floor plan

Drawing No: CANDISH-0316-02 rev. B – Proposed Elevations

Drawing No: CANDISH-0316-04 rev. B – Section A-A

Received 28th June 2016

Reason: To clarify which plans have been approved and in the interests of proper planning.

Informatives relating to:

  - Your attention is drawn to the terms and conditions of this permission. Any development which is carried out but which differs materially from the approved plans and details, or does not comply with any condition(s) or planning obligation(s) attached to the permission may result in the Council taking action to remedy the breach of planning control. If you are in any doubt please contact the Council.

  - The applicant’s attention is drawn to the requirement for submission of a Building Regulations application for the proposed development.  Please contact Building Control on 0118 937 2449 for advice.

  - The construction (and demolition) of the development will be subject to environmental health law and the applicant is referred to:

http://beta.reading.gov.uk/environmentalprotection

  - During the course of construction, the developer should take care to ensure that no part of this development (foundations, eaves, guttering, etc.) would encroach on, under or across the boundary line with the adjoining property, as the requisite Notice has not been served on the owner of that property.  This permission does not confer any right of access onto land that is not under the developer's control. The developer should also be mindful of the obligations and requirements of the Party Wall Act (1996) details of which can be found at: http://www.communities.gov.uk/publications/planningandbuilding/partywall