DELEGATED APPLICATION REPORT 				Target Date:	22/12/2015

Application No: 151910/HOU

Address: 15 Pierces Hill, Reading

Proposal: First floor side and rear extension and single storey front and rear

extensions

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a two storey semi-detached property linked to the neighbouring semi-detached property by single storey attached garages.  This is characteristic of the properties within the row.  The property is at a lower level than the road and has a single storey front projection.  The property has a small rear garden with differing levels at the rear.  The surrounding area is predominantly residential of mixed styles and designs.

Proposal:

The proposal involves a first floor side extension, a two storey rear extension and a single storey front extension.

The proposed first floor side extension extends directly above the attached garage.  The proposed first floor extension is set back from the existing front elevation by 0.5m and set down from the existing roof by 0.2m.  The proposal extends along the boundary with the attached garage at 11 Pierces Hill.

The proposed two storey rear extension measures 1.5m in depth and 3.8m in width at ground floor.  The proposed first floor element is set in from the boundary with the neighbouring property at 11 Pierces Hill by 0.8m.  The two storey rear extension has a pitched roof and a rear bathroom window.

The proposed single storey front extension replaces a small flat roof porch and measures 1.2m in depth and 4m in width.  The proposal has a flat roof with a front facing door and window.

The materials will consist of yellow/brown multi bricks and brown concrete tiles to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 26th October 2015:

Drawing No: 15.16.01 – Existing Plans and Elevations

Drawing No: 15.16.02 Rev A – Proposed Plans and Elevations

Relevant Planning History:

None.

Consultation Responses:

Transport raised no objection as two parking spaces remain on the driveway.

Ecology raised no objection.

Properties at 11 and 17 Pierces Hill and 57 and 58 Fircroft Close were consulted and no comments have been received.

Summary of discussions with applicant/agent:

None.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

CS36 (Biodiversity and Geology)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, the proposal meets general policy and guidance.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO, the submitted drawings show that the proposed rear extension does not encroach on a line taken at 45 degrees which protects light reaching the neighbouring property and there is sufficient distance between the proposed side extension and the first floor side window at the neighbouring property.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO, no side windows are proposed.

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO, a condition will be imposed to restrict side facing windows.

- Access to sunlight and daylight; NO

- Visual dominance and overbearing effects of a development; NO, the neighbouring property at 11 Pierces Hill will notice the additional height and depth of the proposal however, the proposal is not considered visually dominant or overbearing.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time Limit

- Materials

- Approved plans

- No first floor side windows

- Obscure glazed and fixed first floor rear window

Informatives relating to:

- Terms and conditions

- Building Regulations

- Construction and demolition

- Encroachment and Party Wall

- Positive and proactive