METHOD STATEMENT FOR NOISE MITIGATION

Location:	Reading Road, Hook

Activities Likely to Produce Noise:

- Deliveries to site

- Vehicles leaving site

- Vehicle movements on site

- Refilling mortar silos

- Cutting of materials with mechanical saws

- Mixing of mortars and other powdered material

- Piling

- General site activities

Timetable of Activities:

Working times on site are 8am – 6pm from Monday to Friday, and 8am to 1pm on Saturdays.  No work will be carried out on Bank Holidays or Sundays.

Control Measures to be Used:

- Deliveries to site are not permitted until 8am and no later than 5pm.

- Vehicles leaving site are to be controlled by the project gateman.

- Vehicles on site are to be parked in the site car park and engines to be switched off and radios switched off.  Deliveries are to be unloaded promptly and delivery vehicles to be off site asap.

- Mortar refills are to be done during the working day to minimise noise on site.

- Mixing off mortar to be kept to foundation mortar only to reduce the noise from the mixing area.

- Piling works are to be done in stages of build release.  All piling works are to be monitored for sound readings at specific points on the project and all readings are kept on file within the project office.

- General site activities are controlled to prevent any works being carried out outside the site working hours.

Methodology:

Deliveries to Site

All deliveries to site will be delivered to site during the site hours.  In the event that any deliveries arrive to site before 8am, the vehicles are allowed on to site but must switch off their engine and remain parked until 8am.

Vehicles leaving Site

All vehicles leaving site will be controlled by the project gateman.  All vehicles are to leave the scheme in a controlled manner to prevent any undue noise to the surrounding areas.

Refilling Mortar silos

The refilling of mortar silos will be carried out during the working hours for the scheme.

Piling

Piling to be done in build stages to prevent the impact on the surrounding properties.  All piling works are to be monitored twice a day and all readings are to be retained within the project office.

General Site Issues

All contractors are inducted prior to starting on site and it is explained within the site induction that all noise is to be kept to a minimum.  Loud radios are not permitted on site at any time.  In the event of any one breaking the site rules, they will be removed from site.