DELEGATED APPLICATION REPORT 				Target Date: 21/09/2015

Application No: 151328/HOU

Address: 222 Thirlmere Avenue, Tilehurst, Reading

Proposal: Proposed two storey side extension and loft conversion with rear dormer.

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a two storey end of terrace property within a row of four properties.  The property is raised up from the road and there are steep levels at the rear.  The property has a conservatory and a large outbuilding at the rear.  The property is at a higher level than its neighbouring property at 224 Thirlmere Avenue.  The surrounding area is predominately similar style residential properties.

Proposal:

The application is for a two storey side extension and a rear dormer window.  The proposed side extension measures 1.7m in width and 7m in depth.  The proposal is set back from the existing front elevation by 0.6m and set down from the existing roof by 0.3m.  The proposal is 0.7m away from the side boundary with 224 Thirlmere Avenue and would have a ground floor and first floor front and rear window, a side ground floor window and a roof light to the side roof slope.

The proposed rear dormer window extends across the existing roof and the roof of the proposed side extension and measures 3.3m in width, 3.1m in depth and 2.6m in height.  The dormer is set back from the eaves of the main roof by 0.7m and set down from the main roof by 0.5m.

The materials will consist of red/brown brick and tiles to match the existing property and red/brown plain tile hanging to dormer walls.

Plans Considered:

The agent submitted the following drawings, received on 27th July 2015:

Drawing No: TA/PL/01 A

Drawing No: TA/PL/02

Bat Presence/Likely-absence Survey, by ARBTECH, dated 29/05/2015

The agent submitted the following amended plan, received on 11th September 2015:

Drawing No: TA/PL/01 B

Relevant Planning History:

141623 - Proposed two storey side extension and loft conversion with dormers.  Withdrawn 01/12/2014.

Consultation Responses:

Transport raised no objection subject to a condition and informatives.

Ecology raised no objection following the submission of a Bat Survey.

Properties at 220 and 224 Thirlmere Avenue were consulted and no comments have been received.

Summary of discussions with applicant/agent:

The previous application submitted in 2014 proposed a side dormer window attached to the proposed two storey side extension.  This was considered out of keeping with the existing property and the agent was advised the side dormer should be removed.  The current application was initially submitted with the side dormer proposed and the agent was advised this was not considered acceptable.  Amended plans were submitted on the 11th September 2015 which removed the side dormer.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

CS36 (Biodiversity and Geology)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, the proposed side extension is set down from the main roof and set back from the existing front elevation which complies with policy.  The proposed dormer, although flat roofed, is set within the roof slope and does not extend the full width.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO, although the neighbouring property at 224 Thirlmere Avenue has a first floor and ground floor side window adjacent to the proposal a 2.1m gap will remain.  The orientation of the properties also means the proposal would not significantly reduce the amount of light to this neighbouring property.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO

- Visual dominance and overbearing effects of a development; NO, although the neighbouring property at 224 Thirlmere Avenue is at a slightly lower level than the application site the residents of this property will notice the additional height and depth of the proposal but there is a 2.1m gap between the proposed extension and the side of this neighbouring property and the proposal is not therefore not considered visually dominant or overbearing.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time Limit

- Materials

- Approved plans

- Access constructed before development

Informatives relating to:

- Terms and conditions

- Building Regulations

- Construction and demolition

- Encroachment and Party Wall

- Access construction

- Damage to the highway

- Works affecting the highway

- Positive and proactive