DELEGATED APPLICATION REPORT 			Target Date: 09/06/2016

Application No: 160663/HOU

Address: 15 Newlands Avenue

Proposal: Two storey side extension and single storey rear extension

Case officer: Heather Banks

Site Description and Surrounding Area:

The application property is a two storey, red brick, hipped roof, semi-detached property on the south western side of Newlands Avenue. The existing property has a detached garage, which sits level with the front façade of the property, along the southern boundary of the site. It also has a single storey, flat roof extension to the rear of the property. The road and application site slope up towards the north. The surrounding area is residential in nature and characterized by a mix of semi-detached and detached properties, with off road parking. The application property is part of one of two semi-detached pairs of the same design, which are grouped together at this end of the road.

Proposal:

The proposal involves the demolition of the existing detached garage and rear extension and the construction of a two storey side extension and single storey rear extension.

It is proposed that the two storey side extension will have a width of 3.4m and will run the full depth of the existing property. The roof is to be hipped and will have a ridge height, eaves height and pitch to match roof of the existing property. This roof will have an eaves height of 2.55m and a maximum height of 3.3m. At first floor level it is proposed to have a centrally located small bay window to match the small bay window at first floor level on the existing property. To the rear of the extension it is proposed to have a centrally located window at first floor level to match the first floor window closest to the extension on the existing property. At ground floor level the two storey side extension will join the proposed single storey rear extension. To the side elevation of the side extension it is proposed to have two doors and two windows, one of which is to be obscure glazed, at ground floor level. It is proposed to maintain a gap of 1m between the extension and the boundary with 13 Newlands Avenue.

It is proposed that the single storey rear extension will have a depth of 3.85m and a width of 9.9m. This will mean that the rear extension will run the full width of the existing property, plus the width of the proposed side extension. It will be pulled slightly, 0.15m, off of the boundary with 17 Newlands Avenue. The rear extension will have a mono-pitch roof with an eaves height of 2.85m and a maximum height of 4.15m. To the rear elevation of the extension it is proposed to have two full height glazed openings, one of width 4.8m and the other of width 2.45m. It is also proposed to have four velux windows fitted in the roof of the extension.

It is proposed that the materials of the extension will match those of the existing property.

Plans Considered:

The agent submitted the following drawings, received on 11th April

Site Plan

And the following, received on 14th April

Drawing No: 267/01 rev. A – Existing Plans and Elevations

Drawing No: 267/02 rev. A – Proposed Plans

Drawing No: 267/03 rev. A – Proposed Elevations

Drawing No: 267/04 rev. A – Existing Block Plan

Drawing No: 267/05 rev. A – Proposed Block Plan

Drawing No: 267/06 rev. A – Proposed Sections

Relevant Planning History:

None

Consultation Responses:

The RBC Transport Strategy department stated that the proposed development would increase the size of the property from 3-bed to 4-bed.  In accordance with the adopted Parking Standards and Design SPD, a 3 or 4-bed dwelling would be required to provide a parking provision of 2 spaces. It is considered that the dimensions of the proposed integral garage would not be large enough to accommodate a vehicle. The proposal therefore reduces the parking provision at this property from 2 spaces to 1 space. If it cannot be demonstrated that sufficient parking can be accommodated on site it is recommended that this application be refused as a short fall of off road parking is proposed.

The properties at 13 and 17 Newlands Avenue and 20 and 22 The Ridgeway were consulted and no comments have been received.

Summary of discussions with applicant:

On the 2nd June 2016 the applicant was advised by email that the application would be recommended for refusal, due to the harm the proposals, predominantly the side extension, would have on the character of the street scene and the character of the application property. It was recommended that if the side extension was reduced in width and height and stepped back from the existing front elevation of the property, these concerns could be overcome. The applicant was advised that the application could be extended to allow submission of revisions or the application could be withdrawn. Following a phone conversation where the reasons for refusal where discussed an email was received from the applicant stating that they would like the application determined with no revisions and that they would then begin the appeal process.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise.  Material considerations include relevant policies in the National Planning Policy framework (NPPF) - among them the 'presumption in favour of sustainable development'.  However the NPPF does not change the statutory status of the development plan as the starting point for decision making (NPPF paragraph 12).

In this regard, the NPPF states that due weight should be given to the adopted policies of the Local Development Framework (LDF) (Core Strategy and Sites and Detailed Policies Document) according to their degree of consistency with the NPPF (the closer the policies in the plan to the policies in the NPPF, the greater the weight that may be given).

Accordingly, the National Planning Policy Framework and the following development plan policies and supplementary planning guidance are relevant:

Reading Borough Local Development Framework Core Strategy, 2008 (altered 2015)

CS7 (Design and the Public Realm)

CS24 (Car and Cycle Parking)

Sites and Detailed Policies Document, Oct 2012 (altered 2015)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Document - Revised Parking Standards and Design (2011)

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal

The main issues are considered to be:

(i) Impact on the character the application property

(ii) Cumulative impact of development on the character of the street scene

(iii) Car parking

(iv) Impact on amenity of neighbouring properties

Impact on the character of the street scene and surrounding properties

Policy DM9 of the Sites and Detailed Policies Document, Oct 2012 (altered 2015) states that an extension to a house will be acceptable where it respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design.

Given that the proposed extension will be set 1m away from the boundary with 13 Newlands Avenue, the existing side extension at No. 13 is set 1.5m away from the boundary at first floor level and there is a change in level between the two properties, these attributes will serve to lessen a visual ‘terracing effect’ with the neighbouring property, as advised against in the SPG. Also, as the materials to be used in this proposal are to match those of the existing property, it is not considered that choice of materials will have a negative effect on the street scene and character of the area.

However, the character of this part of Newlands Avenue is considered to be one of spaciousness. Properties, which are a mix of detached and semi-detached, two storey and bungalow, are well spaced from each other and there is a large set back between the properties and the road, with a large grass verge, followed by a pavement and then off road parking to the front of the properties. It is considered that the development which has already occurred at 11 and 13 Newlands Avenue has already removed some of this spaciousness and created a greater feeling of enclosure as one passes these properties. It is considered that the current proposal, in combination with the development that has already occurred at 11 and 13, would further reduce the spacious character of the area to an unacceptable degree. The combination of the width, height, depth and location of the proposed side extension would contribute to creating a very wide front façade with no subservient element to reduce the impact of the scale of the development on the street scene. This area of the road would appear uncharacteristically built up, creating a feeling of enclosure rather than spaciousness which is considered at odds with the character of Newlands Avenue. It is therefore considered that the proposed side extension would be contrary to Policy DM9 of the Sites and Detailed Policies Document, Oct 2012 (altered 2015).

Impact on the character of the application property

DM9: House Extensions and Ancillary Accommodation of the Sites and Detailed Policies Document, Oct 2012 (altered 2015), states that an extension to a house will be acceptable where it respects the character of the house in terms of scale, location, materials and design.

The Supplementary Planning Guidance - A Design Guide to House Extensions (2003) states that two storey side extensions should normally be designed to be smaller in scale than the main house. This can be achieved by setting the proposed front elevation further back than the front elevation of the original house, and maintaining a gap between neighbouring properties by bringing the upper floors away from the side boundary. These devices should result in the ridgeline of the roof being significantly lower than that on the original house and the overall effect of an extension in proportion with the original house.

As the choice of materials to be used in this proposal are to match those of the existing property, it is not considered that choice of materials will have a negative effect on the application property.  It is also considered that by virtue of its depth and single storey nature the proposed rear extension would appear as subservient and in proportion to the existing property.

However, it is considered that by virtue its continuation of the ridge and eaves height of the existing building, the proposed side extension would result in unacceptable harm to the character of the application property. It is considered that the side extension would not appear as an extension to the application property, but rather would appear as if it was part of the main property, changing the character of the property to that of a much wider house. The proposed side extensions would constitute a dominant rather than subservient addition and would not preserve the existing house as the main element of the property. It would also result in the loss of prominence of the two storey bay as the focal element of the front façade, as this would be overwhelmed by the width of rest of the new front façade.

Although the properties at 11 and 13 Newlands Road appear to have constructed similar side extensions, it should be noted that these provide examples of where development has had an adverse effect on the character of the property. There properties appear overly wide and the character and proportions of the original property has been lost. The side extension at 11 Newlands Avenue was granted permission in 1988 under application 87/TP/1317. As this occurred a significant time prior to the introduction to the current Reading Borough Council policies and guidance regarding the extensions of houses, the granting of this permission should not be considered to be a reason to grant permission for the current scheme. The side extension at 13 Newlands Avenue was granted permission in 2008 under application 08-00238-FUL. It should be noted that the design of this extension, although not set down from the ridge line of the original property, or set back from its front façade, was considered acceptable as it would restore symmetry to the semi-detached pair, previously unbalance by the 1988 extension at No. 11. As the application property is part of a separate semi-detached block, it is not considered that the proposals similarity with the extensions at 11 and 13 Newlands Avenue should be seen as a reason to grant approval. Rather, it is considered that any side extensions to the application property and its adjoined neighbour, No. 17, should not perpetuate the poor design of the neighbouring semi-detached pair created by the 1988 planning application, as it would fail to preserve the balance/duality of the pair.

It is considered that the proposed two storey side extension would not preserve the character of the existing property as it would constitute a dominant rather than subservient addition. It is considered that the proposed side extension would be harmful to the character of the application property and contrary to the advice given in A Design Guide to House Extensions (2003) and contrary to Policy DM9 of the Sites and Detailed Policies Document, Oct 2012 (altered 2015).

Parking

It is considered that the dimensions of the proposed integral garage would not be large enough to accommodate a vehicle. The proposal therefore reduces the parking provision at this property from 2 spaces to 1 space. The adopted Parking Standards and Design SPD states that a 3 or 4-bed dwelling is required to provide a parking provision of 2 spaces; this has not been provided in the submitted proposal. However, it is possible that two parking spaces could be provided within the curtilage of the property and this could be required to be supplied, by condition.

Impact on amenity of adjoining properties

DM4 of the Sites and Detailed Policies Document, Oct 2012 (altered 2015) states development should not cause a significant detrimental impact to the living environment of existing of new residential properties in terms of overshadowing, privacy and overlooking and visual dominance and overbearing effects of a development.

There is a drop in ground level between the application property and its neighbour at 13 Newlands Avenue. This can often cause issues of overbearing and/or overlooking with side and rear extensions. However, it is considered that due to the depth and location of the existing single and two storey extensions at 13 Newlands Avenue, the proposed extensions will not have a detrimental impact on the occupiers of that property with regards to the issues mentioned above. Due to the existing rear extension and high boundary wall immediately to the rear of 17 Newlands Avenue it is not considered that the proposed rear extension would have any overshadowing or overbearing impacts on the occupiers of this property. It is considered that the proposed extensions would be in accordance with Policy DM4 of the Sites and Detailed Policies Document, Oct 2012 (altered 2015).

Conclusion:

Due to the unacceptable impact on the house itself and the duality/balance of the semis, the design would be harmful to the streetscene.  The applicant is aware of these concerns, but has decided not to withdraw the application.

RECOMMENDATION:

REFUSE

By virtue of its width, its lack of set-back to the front façade, and failure to set down the ridge height of the existing property, it is considered that the proposed side extension would cause unacceptable harm to the character of the application property and the duality/balance of the pair of semis, through failing to provide a subservient addition.  Furthermore, given the existing developments at 11 and 13 Newlands Avenue which also continue the front façade and ridge height of their host properties, the cumulative effect created by the addition of this proposal to the street scene would result in unacceptable harm to the spacious character of this part of Newlands Avenue.  It is therefore considered that the extension is contrary to Policy CS7 (Design and Public Realm) of the Reading Borough LDF Core Strategy (2008, as amended 2015) and policy DM9 (House Extensions and Ancillary accommodation) the Reading Borough LDF Sites and Detailed Policies Document (2012) and the Council's adopted Supplementary Planning Guidance, 'A Design Guide to House Extensions' (2003).

Informatives

- The plans considered in this planning application are as follows:

Site Plan

Received by the Local Planning Authority on 11th April 2016, and:

Drawing No: 267/01 rev. A – Existing Plans and Elevations

Drawing No: 267/02 rev. A – Proposed Plans

Drawing No: 267/03 rev. A – Proposed Elevations

Drawing No: 267/04 rev. A – Existing Block Plan

Drawing No: 267/05 rev. A – Proposed Block Plan

Drawing No: 267/06 rev. A – Proposed Sections

received by the Local Planning Authority on 14th April 2016

- The Local Planning Authority has worked with the applicant in a positive and proactive manner. The applicant was made aware of the issues relating to the refusal of this application and advice was provided to overcome the issues raised.  The applicant decided not to respond to the advice given and so has not been possible to overcome these issues, hence the issuing of this notice of refusal of permission.

- This planning application was submitted without first having sought pre-application advice, which can often avoid the need to refuse planning permission.  The council's pre-application advice service is here:

http://www.reading.gov.uk/media/1190/Pre-Application-Enquiry-Form/pdf/Pre-app_1_Revised_Nov_2015.pdf