DELEGATED APPLICATION REPORT - Application No: 152010/HOU

Address: 12 Sheridan Road, Reading.

Proposal: Proposed erection of a single storey extension to both side and rear of existing dwelling.

Case officer: John Gregory

Site Description and Surrounding Area: The application relates to a two storey semi-detached dwelling on the south side of Sheridan Avenue, Reading. The surrounding area is predominantly residential and of mixed character.

Proposal: The application seeks planning permission for a single storey side and rear extension incorporating a pitched roof extending rearwards at a height of 3.75m. The side extension running the full length of the existing host dwelling at 1.2m wide incorporating the continuation of the rear elements pitched roof.

The gable ended rear element of the proposal to be 5.745m deep is intended to be full width with the addition of the 1.2m wide side extension. Consequently 6.5m wide with rear extension following demolition of existing detached rear garage. Three velux windows on either side of new rear extension pitched roof. New rearward concertina patio doors to rear extension. Proposal to provide sitting/family area following a series of associated internal alterations.

Plans Considered:

Drawing no. 1499 -01 – Existing location and Block plan

Drawing no. 1499 -02 B – proposed location and Block plans received 18th December 2015

Drawing no. 1499 -01 – Existing Plans & Elevations

Drawing no. 1499 -02 B – Proposed Plans & Elevations received 18th December 2015

Received on 10th November 2015

Relevant Planning History:

None.

Consultees:

Transport – No objection. Expressed concern over the loss of a garage but, that the existing side entrance was too narrow for car access. A single car park space is acceptable.

Notifications:

No10 and No14 Sheridan Avenue

No 2 Valley Close

Consultation Responses:

Objection 16th December 2015.

Object over the size of extension and building foundations. Loss of light to rear windows and effect on the character of rear garden.

The gardens face north and so have reduced light. Our garden is in addition to the east side of No12 Sheridan Avenue. We only get evening sun in the back garden. Concerned about the changes to the character of our garden.

Overall concern is the size of the proposed extension.

Objection 25th January 2016 amended drawings 1499 -02 B

Object to the size of the side extension.

Also object to the reduction of parking space at the property.

Also an error in the drawing where the parking is described as shared. This is not the case.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

Section 38(6) of the Planning and Compulsory Purchase Act 2004 requires that proposals be determined in accordance with the development plan unless material considerations indicate otherwise.  Material considerations include relevant policies in the National Planning Policy framework (NPPF) - among them the 'presumption in favour of sustainable development'.  However the NPPF does not change the statutory status of the development plan as the starting point for decision making (NPPF paragraph 12).

In this regard, the NPPF states that due weight should be given to the adopted policies of the Local Development Framework (LDF) (Core Strategy and Sites and Detailed Policies Document) according to their degree of consistency with the NPPF (the closer the policies in the plan to the policies in the NPPF, the greater the weight that may be given).

Accordingly, the National Planning Policy Framework and the following development plan policies and supplementary planning guidance are relevant:

### Reading Borough LDF Core Strategy (Adopted January 2008)

CS7     Design and the Public Realm

Sites and Detailed Policies Document – (Adopted October 2012)

DM4    Safeguarding Amenity

DM9    House Extensions and Ancillary Accommodation

### Supplementary Planning Guidance/Documents

SPG – A Design Guide to House Extensions

Assessment.

From the original submitted design the rear extension was proposed at 5.745m deep, full width 6.5m and a ridge to 3.7m. The side extension element to match the existing front building line.

This proposal was considered to be too large following ASG (Application Steering Group) on Tuesday 1st December 2015. A revision was requested. Reduce the overall length, height of the rear extension and cut back from the common boundary with No10.

On the 10th December a revised design showed a cutback of the rear extension to 5m deep, and a lowering of ridge to 3.6m. Four velux windows as opposed to 6. Reduction of velux windows from 6 to 4.

Cut back from both neighbouring common side boundaries

In addition the side element cut back from the front building line by 0.5m. A suggestion to cut back the rear extension from the common boundary and create or move the WC side window to the front elevation, to create a feature, facing Sheridan Avenue were ignored.

For a second time the proposal was considered by ASG 11th December 2015 to be too large.

On 18th December a second revision arrived with a further reduction in the overall length to 4.00m and cut back the extension from the common boundary with No10 Sheridan Avenue by 0.5m ridge to 3.55m.

Objection.

The objection above was received in association with the original scheme at 5.745m depth and not the subsequent efforts to amend. Revised drawings given “B”

Objection No2 with second revised scheme of 18th Dec 2016 Drawing 1499-02 B

A second objection was received in association with the revised scheme. Despite these it is considered that following the substantial reduction is the mass and bulk of the proposed extension from the original scheme that the size of the side extension would not present an unacceptable loss of residential amenity to the adjacent neighbours.

Highways.

Expressed concern as did the objection above but transport considered No Objection to the loss of the side access and retaining of one car park space.

Conclusion.

Revision be is considered not to raise objection in terms of overall bulk, mass, shadowing,  sense of enclosure, loss of privacy to the residential amenity of neighbours.

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building;  NO

- reduce the amount of usable amenity space for the property to an unacceptable degree;     NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO

- Visual dominance and overbearing effects of a development; NO

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fume; NO

- Smell; NO

RECOMMENDATION:

GRANT

The proposed development would not be harmful to the character or appearance of the site or its surroundings and would not have a detrimental impact on the original building or neighbouring properties.  As such the proposed works are considered to be in accordance with CS7 (Design and the Public Realm) of the Reading Borough Local Development Framework Core Strategy (2008), DM4: Safeguarding Amenity and DM9: House Extensions and Ancillary Accommodation of the Sites and Detailed Policies Document (Oct 2012) and the Councils Supplementary Planning Guidance A Design Guide to House Extensions (2003).

Conditions:

- The development to which this permission relates must be begun not later than the expiration of three years beginning with the date on which the permission was granted.

Reason: to prevent an accumulation of unimplemented planning permissions and in accordance with Section 91 of the Town & Country Planning Act 1990 as amended by the Planning and Compulsory Purchase Act 2004.

- The permission relates only to the following plans and drawings:

Drawing no. 1499 -01 – Existing location and Block plan received 18th December 2015

Drawing no. 1499 -02 – proposed location and Block plans received 18th December 2015

Drawing no. 1499 -01 – Existing Plans & Elevations

Drawing no. 1499 -02 B – Proposed Plans & Elevations received 18th December 2015

Received on 10th November and 18th December 2015

Reason: for the avoidance of doubt and in the interests of proper planning.

3. 	The materials to be used in the construction of the external surfaces of the development shall match those used on the existing building.

Reason: To ensure that the new development appears in keeping with the existing building, in the interests of the character of the area, in accordance Policy DM9 of the Sites and Detailed Policies Document 2012.

Informatives:

- These works will require separate approval under the Building Regulations.  Please contact the Council's Building Control service for further advice on (0118) 937 2449.

- Your attention is drawn to the terms and conditions of this permission. Any development which is carried out but differs materially from the approved plans and details, or does not comply with any condition (s) or planning obligation (s) attached to the permission may result in the Council taking action to remedy the breach of planning control. If you are in any doubt please contact the council.

- The determination of this planning application required no issues to be resolved so officers have worked positively and proactively to provide a recommendation for approval to be made within the target timescale for householder planning applications.