DELEGATED APPLICATION REPORT 			Target Date: 21/01/2016

Application No: 152116/FUL

Address: 189 Southampton Street, Reading

Proposal: Change of use of existing 7 bedroom House in Multiple Occupation to 9

bedroom HMO to include a two storey rear extension and replacement front and rear dormer windows.

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a three storey end terrace property that is used as a 7 bedroom large house in multiple occupation (HMO).  The property has a shallow pitched roof with a small pitched roof dormer to the front roof slope and a small flat roof dormer to the rear roof slope and an existing single storey rear extension.  The surrounding area is predominately residential.

The application site falls within an Article 4 Direction Area which removed permitted development rights for the change of use from a C3 dwellinghouse to a C4 small HMO.

Proposal:

The proposal is for the change of use of the existing 7 bedroom HMO to a 9 bedroom HMO which involves the demolition of the existing single storey rear extension that measures 2.2m in width, 4.9m in depth, 2.2m in height to eaves level and 3.2m in height at its highest point and the construction of a two storey rear extension and the replacement of the existing front and rear dormer windows.

The existing small pitched roof front dormer and small flat roof rear dormer are proposed to be replaced with two flat roof dormers to the front roof slope and a larger flat roof dormer to the rear roof slope.  The proposed front dormers remain small in size and sit within the existing roof slope and are positioned in line with the existing ground and first floor windows.  The proposed rear dormer, although larger than the existing, is also still small in size and sits within the rear roof slope.

The ground floor element of the proposed two storey rear extension measures 5.4m in width (the same width as the existing property) and 5.2m in depth which is the same as the previously approved planning permission for a single storey rear extension (151670/HOU).  The proposed first floor element of the extension introduces a cat slide roof with 2 first floor flat roof rear dormer windows.  The height to eaves level is approximately 3.5m.  The proposal adjoins both neighbouring boundaries.  The neighbouring property at 191 Southampton Street is at a higher level and has an existing single storey rear extension that extends along the boundary with the application site.  The proposed extension would be 0.5m shorter than this neighbouring extension.  The property at 185 Southampton Street contains 5 flats and is at a lower level than the application site and this property is set 2m further back from the rear of the application site.

The proposed development would require alterations to the internal arrangement of the property and includes a shared kitchen and dining room on the ground floor.

The materials will consist of red brick and slate tiles to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 26th November 2015:

Drawing No: 01 Rev P1 – Site Location Plan

Drawing No: 02 Rev P1 – Site Plan

Drawing No: 03 Rev P3 – Existing Plans

Drawing No: 04 Rev P2 – Existing Elevations

Drawing No: 07 Rev P4 – Proposed Plans Option 2

Drawing No: 08 Rev P2 – Proposed Elevations

Drawing No: 09 Rev P1 – Existing RH Section / Elevation

Drawing No: 11 Rev P1 – Proposed RH Section / Elevation Option 2

Drawing No: 8973/Opt 2/02 – Tree Protection Plan

Arboricultural Report by Ian Keen Limited, Ref: AP/8973 Opt 2/SO, dated 24th November 2015.

Relevant Planning History:

151670/HOU - Demolition of existing single storey rear extension and existing front dormer and construction of a new single storey rear extension and front and rear dormer windows.  Permitted 13/11/2015.

141549/CLE - Certificate of Lawfulness Existing Use relating to a house in multiple occupation, 7 bedsits.  Permitted 12/12/2014.

Consultation Responses:

Transport raised no objection subject to conditions and informatives.

Natural Environment Trees raised no objection subject to a condition implementing tree protection measures.

Environmental Protection raised no objection subject to conditions on the control of noise and dust, hours of working and bonfires.

Properties at 185 (all), 189 and 191 Southampton Street and 62 and 64 Mount Pleasant were consulted and no comments have been received.

Summary of discussions with applicant/agent:

The agent was advised in an email dated 15th January 2016 that there were a number of concerns with the proposal with regards to the design and appearance of the proposed extension, the impact of the extension on the neighbouring property at 185 Southampton Street and the application did not demonstrate that there was sufficient communal space available for a 9 bedroom HMO.  At the time of writing the applicant had not advised how they wished to proceed and the application has therefore been refused.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

National Planning Policy Guidance

National Planning Policy Framework (NPPF) - Part 6 – Delivering a wide choice of high quality homes

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

CS18 (Residential Conversions)

CS36 (Biodiversity and Geology)

CS38:  Trees, Hedges and Woodlands

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM8 (Residential Conversions)

DM9 (House Extensions and Ancillary Accommodation)

DM10 (Private and Communal Outdoor Space)

DM12 (Access, Traffic and Highway-Related Matters)

Supplementary Planning Guidance/ Documents:

Residential Conversions SPD (2013)

Revised Parking Standards and Design SPD (2011)

A Design Guide to House Extensions SPG (2003)

Appraisal

The main issues are:

- Policy background

- Principle of the change of use

- Impact of the extension on amenity of existing and future residents of the property

- Design and impact on the character of the surrounding area

- Impact on neighbours

- Traffic generation and parking

- Community Infrastructure Levy

- Policy background

Residential conversions have an important role to play in housing supply in Reading by adding a valuable supply of reasonably affordable private rented accommodation.  However in some circumstances, residential conversions can have a harmful impact on the character of an area, providing poor or inadequate accommodation and leading to problems and issues for adjoining neighbours and the surrounding area.

Core Strategy Policy CS18: Residential Conversions sets out the strategic approach to converting residential properties into HMOs covering aspects such as impact on the amenity and character of the area, loss of privacy and ensuring adequate parking.  Policy DM8: Residential Conversions of the Sites and Detailed Policies Document sets out detailed matters for consideration such as unduly diluting or harming existing mixed and sustainable communities and providing a minimum size for converting properties to ensure that Sui Generis HMOs have an appropriate balance between communal and private areas.

The Residential Conversions SPD adds that adopted policies covering all residential conversions aim to find the right balance between:

-	Making a contribution towards identified housing needs;

-	Ensuring adequate standards of accommodation, which are suited to the needs of the growing number of smaller households;

-	Protecting existing residential amenity;

-	Providing acceptable levels of on-site amenity space and car parking;

-	Ensuring a mixed and sustainable community; and

-	Maintaining the supply of single family housing.

The application site has previously been granted a lawful development certificate for the existing use of the property as a 7 bedroom HMO however the intensification of the property to a 9 bedroom HMO needs to be considered against the criteria within the policies and guidance highlighted above.

- Principle of the change of use

As the property has already been converted into a large HMO and the use of the building will remain as Sui Generis HMO the principle of development has already been established.  However, the intensification of the building must satisfy other policy criteria as detailed in the report below.

- Impact of the extension on amenity of existing and future residents of the property

The Residential Conversions SPD identifies a number of checklist items which form the basis of the assessment of the proposal in terms of the impact of the extension on the amenity of existing and future residents of the property, the impact of the proposal on neighbouring properties and the surrounding area and parking provision.

With regards to the impact of the proposal on the amenity of the existing and future residents of the application property the SPD checklist makes reference to providing an appropriate level of amenity space whilst DM10: Private and Communal Outdoor Space states that residential conversions will be required to provide private outdoor space which should be no less than the gross floor area of the dwelling to which it relates.  The application site has a rear garden area that is available to existing occupiers of the property however it is not more than the gross floor area of the property.  Although this is contrary to the SPD as the property is already an existing large HMO the existing garden is considered an acceptable size in this instance.

The SPD also makes reference to minimum internal floorspace standards that relate specifically to house conversions where an intensified use of a building may result in a poor standard of development that may harm the residential amenity of existing and prospective residents.  The floor area for a shared kitchen shall not be less than 7 square metres and for bedrooms, if occupied by one person, must be of a minimum of 6.5 square metres and 10.5 square metres if occupied by two persons.  The submitted floor plans show a proposed kitchen on the ground floor with a floor area of 9.3 square metres.  The smallest bedroom in the property has a floor area of 8.8 square metres and the largest bedroom 17.8 square metres and 3 of the 9 rooms would be of a size for single occupancy only.  However, the documentation submitted with the application does not state whether the existing or proposed bedrooms are, or would be, single or double occupancy and it is not therefore possible to determine whether some of the rooms would be a suitable size if they were to be occupied by two people.  However, this matter could be dealt with by way of a condition that restricts the number of residents per room.

The SPD goes on to advise that habitable rooms such as living rooms and bedrooms should benefit from an external window and the provision of natural light and ventilation is a legal requirement under Environmental Health legislation.  All rooms within the property have an external window.  Internal layouts should avoid locating living rooms, bathrooms and kitchens next to, above, or below proposed and neighbouring bedrooms.  The proposed layout of the property has taken this into consideration and the majority of the rooms have been stacked where practical to do so and sound insulation will be addressed through Building Regulations as sound insulation is a requirement under Building Regulations for new extensions.  In this regard the proposal is acceptable.

As stated previously the documentation submitted with the application does not state whether the existing and proposed rooms are, or will be, used for single occupancy or double occupancy however, the size of the rooms on the submitted floor plans would seem to suggest 6 of the rooms could be double occupancy and 3 of the rooms single occupancy.  Potentially there could be 15 people living at the property.  The SPD states in Checklist 25 that a standard of one communal room for every 4-6 rooms (depending on the size of the bedroom) will be provided, which will have acceptable amenity standards and appropriate access for all residents.  The SPD (under point 4.9) states that it should be assumed that the provision of a communal room is an addition to the provision of a kitchen, particularly where the kitchen provides no seating area or space to relax and is purely a functional area.  Point 4.10 of this document states that where all bedrooms significantly exceed minimum room standards and the rooms are intended for single occupancy only, as a guide, one communal room per six bedrooms may be considered appropriate.

The application has not been submitted with this information and as such, the proposal is not considered acceptable in terms of its impact on existing and future occupiers as it has not been demonstrated that there is sufficient communal space available for a 9 bedroom HMO and therefore the increase from a 7 bedroom to a 9 bedroom HMO is contrary to the Council’s SPD, polices DM4 and DM8 of the Sites and Detailed Policies Document and CS18 of the Core Strategy.

- Design and impact on the character of the surrounding area

The proposed dormer windows and the ground floor element of the two storey rear extension has already been considered under the previous planning permission 151670/HOU.  The proposed dormer windows are small in size and located within the roof slopes and are considered in keeping with the existing property.  The proposed ground floor extension is no deeper than a neighbouring extension and although it exceeds the Council’s guidance of 4m in depth this does not detract from the appearance of the original building in this instance.

The main concern relates to the design and appearance of the two storey rear extension which introduces a cat slide roof with two first floor rear dormer windows.  This design is not considered in keeping with the existing property which has an existing shallow pitched roof.  DM9 of the Sites and Detailed Policies Documents states that extensions should respect the character of the house in terms of scale, location, materials and design whilst policy CS7: Design and the Public Realm of the Core Strategy states that all development must be of high design quality that maintains and enhances the character and appearance of the area.

Introducing a cat slide roof and first floor dormer windows does not reflect the design or appearance of the existing property nor does it complement the existing property.  The proposed extension is also considered to detract from the property’s original character and the character of the neighbouring properties.  The proposed extension is therefore contrary to Core Strategy Policy CS7 (Design and the Public Realm), Policy DM9: House Extensions and Ancillary Accommodation of the Sites and Detailed Policies Document and the Council’s Supplementary Planning Guidance – A Design Guide to House Extensions.

e) Impact on neighbouring properties

DM4: Safeguarding Amenity states that development should not cause a significant detrimental impact to the living environment of neighbouring properties in terms of privacy and overlooking; access to sunlight and daylight; visual dominance and overbearing effects of a development; noise and disturbance; artificial lighting; vibration; dust and fumes, smell; or crime and safety.  The impact on neighbouring properties therefore needs to take into account both the impact of the proposed extension and the intensification of the proposed change of use from a 7 bedroom HMO to a 9 bedroom HMO.

The application site is at a higher level than the neighbouring property at 185 Southampton Street and this neighbouring property is also set further back than the application site.  Although the submitted drawings show the proposal does not encroach on a line taken at 45 degrees, as this neighbouring property is at a lower level than the application site, extending at first floor level directly along the boundary is considered visually dominant and overbearing on the residents of this neighbouring property and in this respect the proposal is contrary to policies CS18 of the Core Strategy, DM4 and DM8 of the Sites and Detailed Policies Document and the Council’s SPD on Residential Conversions.

The proposal would also alter the outlook for the neighbouring property at 191 Southampton Street however, the proposal is not considered to impact on this neighbouring property in terms of being overbearing or visually dominant.

The proposed increase from a 7 bedroom HMO to a 9 bedroom HMO also has to be taken into consideration when assessing the impact on the neighbouring properties and the surrounding area in terms of noise and disturbance.  The existing property is used as a 7 bedroom HMO and the proposal will require the existing layout to be altered.  However, Building Regulations will be required which addresses sound within the new rooms of the extension and although this does not require additional insulation along the existing party wall this requirement will reduce noise impacts within the property which will in turn ensure there is no detrimental noise impacts on the neighbouring property which is in accordance with policies CS34 and DM4.

The nature of a HMO does mean that tenants are unlikely to reside at the property for long durations of time however there is no evidence to suggest that this would cause a significant detrimental impact on the living environment of the neighbouring properties in terms of noise and disturbance, crime or safety. In this respect the proposal is considered acceptable.

f) Traffic generation and parking

In accordance with the adopted SPD, the proposed development would be required to provide a parking provision of 0.25 spaces per bedroom equivalent to 2 on-site parking spaces for a 9 bedroom HMO.  However, it is not feasible to provide any additional on-site parking.

Given that the site is located within an area designated as a Residents Parking Permit Area Zone 10R where there are extensive parking controls preventing indiscriminate on-street parking, the proposed change of use from an existing 7 bedroom HMO to a 9 bedroom HMO is acceptable in transport terms.  However, there should be an assumption that any future occupants of the rooms would not be issued with resident parking permits which should be secured through conditions and informative.

g) Community Infrastructure Levy

No CIL is payable.

RECOMMENDATION:

REFUSE

Informatives relating to:

- Refused plans

- Positive and proactive