Design & Access Statement

115 Highgrove Street, Reading – January 2016

1. THE SITE

No. 115 Highgrove Street is a non-listed semi-detached residential property dating from the 1950’s set within a residential street of mixed-age properties. The property comprises two storeys and is constructed of simple engineering brick. Two large bay windows at ground and first floor level face onto the street. The roof is pitched and hipped.

The rear façade has a single storey wooden and glazed extension with plastic corrugated roof, which was added in the early 1990s. Windows within this face onto the adjacent properties (no’s 113 & 117) and the garden, which is predominantly laid to lawn. The garden backs onto private, flat roof, brick garages.

2. PLANNING HISTORY

No previous planning history exists that we are aware of.

3. DESIGN OBJECTIVE

This Design and Access statement has been prepared in respect of proposals to demolish and replace an existing wooden extension with a masonry, single storey extension at ground floor level.

4. PROPOSAL DESCRIPTION

The proposals comprise a single-storey rear extension, which, through some minor internal remodelling, will provide an enlarged kitchen/dining area, and allow the replacement of the existing small kitchen with storage and downstairs bathroom.

No clear building line exists to the rear of the properties on Highgrove Street due to varying house types and only one property, some distance away, has a similar rear extension (No.123).

5. PROPOSED DESIGN AND MATERIALS

The existing lean-to is very dilapidated and its removal will offer an immediate, significant improvement.

During design development height and visual prominence have been core considerations, as has the need to minimise overlooking of adjacent properties.

The glazing included within the proposals has been designed to minimise any overlooking of adjacent properties, with no windows proposed, only a large rooflight providing overhead natural light, and a canopied large bi-folding door opening directly onto the properties rear garden.

During the design phase a proposal utilising matching materials was carefully considered, but it was suggested that sourcing bricks to match the property would be difficult. The design therefore seeks to articulate the extension, by utilising a contrasting but muted palette of materials - white rendered blockwork, a zinc clad projecting canopy and roof parapets, a dark grey single-ply roofing membrane and an area of Siberian Larch timber cladding on the garden facing and southerly elevations. A 3D rendered model has been provided with the drawings, to articulate and aid assessment of these materials.

6. SCALE AND APPEARANCE

The proposals are held off the edge of the existing building by 250mm in order to remain subsidiary to it.

The single storey proposal extends 3.5m to the rear of the building with an added overhead projection taking the maximum extent from the building to 4.1m. It stands at a height of 3.0m to the eaves, with a projecting overhead roof light with upstands, taking the maximum height to 3.3m. This height mimics that of the garages to the rear of the property.

The proposals will extend 1.5m beyond the footprint of the existing lean-to structure and offer a circa 45% increase on the footprint of the property as a whole. Daylight reduction to the neighbouring property (No. 113) to the north was considered during the design phase and the single storey, flat roof proposal seeks to minimise any light reduction. No other properties are affected in terms of daylight, by the proposals.

The proposals include an external wall spanning the northern boundary line. This has been proposed in discussion with the occupants of No. 113 Highgrove Street, in order to offer them a usable wall and foundation, should they wish to extend in the future. A party wall agreement has been discussed with and agreed/signed by the owners of the adjacent property (113 Highgrove Street) to confirm they are happy with this.

7. ACCESS

Garden access is unaffected by the proposals.

8. SUPPORTING DOCUMENTATION

Site Plan @ 1.1250

Existing Block Plan @ 1:500

Existing Rear and Side elevations @ 1:50

Photos: Existing rear extension

Proposed Block Plan 1:200

Proposed Ground floor and First floor Plans 1:50

Proposed rear elevation and Section AA and BB @ 1:50

3D model of rendered finish