DELEGATED APPLICATION REPORT 				Target Date: 23/09/2015

Application No: 151342/HOU

Address: 2 Gairn Close, Tilehurst, Reading

Proposal: Single storey side extension

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a two storey end of terrace property.  The properties within the row are staggered in threes but unlike the rest of the properties within the row which have principal front elevations facing onto a grassed area the application property has a front elevation facing onto Gairn Close due to its corner position.  As such this is considered the principal elevation and any proposed rear extension would actually be for a side extension.  The surrounding area is predominately residential properties.

Proposal:

The application is for a single storey side extension that extends 4m in width, 5.2m in depth and 2.7m in height.  The proposal would have a flat roof with a roof light, bi-fold doors facing into the garden and a new window would be inserted into the front elevation.  As the property has a different principal elevation to the other properties within the row, what would normally be a rear extension is now a side extension however the depth of the proposal along the boundary with the adjoining property at 4 Gairn Close is 4m.  4 Gairn Close have an existing 2.5m depth conservatory that extends close to the boundary with the application site.  The proposal would extend directly along the front boundary where an existing boundary wall has been removed.

The materials will consist of brick to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 28th July 2015:

Drawing No: P1514-01 Rev A – Location Plan & Block Plan

Drawing No: P1514-02 – Existing Plan & Elevations

Drawing No: P1514-03 – Proposed Plans And Elevations

The agent submitted the following amended drawing, received on 22nd September 2015:

Drawing No: P1514-03 Rev A – Proposed Plans And Elevations

Relevant Planning History:

151047/HPA - Rear extension measuring 4.1m in depth, with a maximum height of 4m, and 3m in height to eaves level. Notification of the construction of an extension under Class A, Part 1 of Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015.  Withdrawn 14/07/2015 as the proposal was considered a side extension and not a rear extension.

Consultation Responses:

Transport raised no objection.

Properties at 1 and 4 Gairn Close were consulted and no comments have been received.

Summary of discussions with applicant/agent:

The agent was advised in an email dated 22nd September 2015 that the proposed elevations and floor plans measured 3m in width but the Block Plan measured 4m in width.  The agent confirmed the width was 4m and amended plans were submitted on 22nd September 2015.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO, the orientation of the properties means the proposal will not significantly reduce the amount of light to the neighbouring property at 4 Gairn Close.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO, the proposal replaces part of a brick boundary wall and the appearance from the street will not be significantly different.

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO

- Visual dominance and overbearing effects of a development; NO, the neighbouring property at 4 Gairn Close will notice the additional height and depth of the proposal but this property has an existing 2.5m depth rear conservatory.  The proposed extension is an extra 1.5m deeper than the neighbouring conservatory which is considered acceptable and the proposal is not considered visually dominant or overbearing.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time Limit

- Materials

- Approved plans

Informatives relating to:

- Terms and conditions

- Building Regulations

- Construction and demolition

- Encroachment and Party Wall

- Positive and proactive