DELEGATED APPLICATION REPORT

Target Date:  10/08/2015 			Extended Target Date: 11/09/2015

Application No: 151042/HOU

Address: 14 Morgan Road, Reading

Proposal: Part two storey and single storey side and rear extension

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a three storey semi-detached property with a three storey rear gable, a single storey side element and a single storey rear extension and conservatory.  The surrounding area is predominantly residential.

Proposal:

The existing property has a single storey flat roof element to the rear of the existing gable which matches the adjoining property at 16 Morgan Road and measures approximately 3.4m in depth.  The property has a conservatory to the rear of this single storey element that measures 2.5m in depth and 3.6m in width.  The property also has an existing single storey side element that does not extend past the main part of the property which extends along the boundary with 12 Morgan Road.

The proposal involves the demolition of the existing conservatory and the construction of a single storey side and rear extension that wraps around the existing gable.  The single storey element of the proposal extends to the rear of the existing side extension by a further 4m and then steps in from the boundary by 1.8m.  The single storey extension then extends a further 6.8m to the rear.  The proposal will extend a further 1m to the rear than the depth of the existing conservatory but increases in width to 6.1m.  The proposed single storey would have a parapet wall measuring 3m in height with 3 flat panel roof windows.  Windows and doors are proposed to the south and east elevations.

The proposal also involves a first floor extension to the rear of the existing gable.  The proposed first floor extension measures 2.4m in depth and 4m in width and is set down from the roof of the existing gable by approximately 2m.  The room to which the extension is extending is currently a bathroom which has a rear and side facing window.  This is the same at the neighbouring property at 16 Morgan Road although the existing bathroom and the proposed first floor extension will become a bedroom.

The materials will consist of brick and tiles to match the existing property.

Plans Considered:

The agent submitted the following drawings, received on 15th June 2015:

Site Location Plan

Block Plan

Drawing No: AZ-1777-0001 – Existing Ground Floor Plan

Drawing No: AZ-1777-0002 – Existing First Floor Plan

Drawing No: AZ-1777-0003 – Existing Second Floor Plan

Drawing No: AZ-1777-0004 – Existing Elevations

Drawing No: AZ-1777-0005 – Existing Typical Section

Drawing No: AZ-1777-0006 – Proposed Ground Floor Plan

Drawing No: AZ-1777-0007 – Proposed First Floor Plan

Drawing No: AZ-1777-0008 – Proposed Second Floor Plan

Drawing No: AZ-1777-0009 – Proposed Elevations

Drawing No: AZ-1777-0010 – Proposed Typical Section

Drawing No: AZ-1777-0011 – Proposed Typical Section

Relevant Planning History:

None.

Consultation Responses:

Transport raised no objection.

Ecology advised although the property is likely to have features potentially suitable for use by roosting bats there is only very limited incursion into the roof space.  As such the risk of bats being affected by the proposal is minimal and there are no objections on ecology grounds.

Properties at 10, 12 and 16 Morgan Road and 32 and 34 Allcroft Road were consulted.  Two letters of representation have been received.  One letter raised no objection to the proposal and the second letter raised concerns with regards to the first floor extension being visually dominant, loss of light, impact on outlook, effect on the symmetry of the rear elevation and potential to affect value of property.

The effect of a proposal on the value of a neighbouring property is not a material planning consideration however, the other points raised will be addressed in the report below.

Summary of discussions with applicant/agent:

The applicant was advised of the concerns raised by the neighbour and asked whether a reduction in the depth of the first floor extension would be possible.  The applicant advised that they wished to proceed with the application as submitted.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

CS36 (Biodiversity and Geology)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, the proposal is of a large size at ground floor and the proposal does exceed 4m in depth which is contrary to the Council’s SPG however, the property is on a relatively large plot and it is considered that the property and the garden are capable of accommodating an extension of a longer depth.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO, the proposed single storey extension extends approximately 4.3m from the rear of the neighbouring property at 16 Morgan Road which is 1m deeper than the existing conservatory.  The neighbouring property at 12 Morgan Road has a single storey element that extends along the boundary and the proposed ground floor extension that extends along the boundary does not extend past the rear of this property.  The proposed first floor extension does encroach on a 45 degree line taken from the first floor window at the neighbouring property however, this is a window to a non habitable room and this room is also served by a second window to the side.  The proposals are therefore not considered to significantly reduce the amount of light to either 12 or 16 Morgan Road.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO, although a condition will be imposed to restrict any use of flat roofs as balconies.

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO, the neighbouring property at 12 Morgan Road is different in style and character and although the proposed extensions will alter the pair of semi-detached properties at 14 and 16 Morgan Road the majority of the works are single storey and the proposed first floor extension is considered in keeping in terms of design and appearance to the existing property.

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO, the proposal is a sufficient distance from the neighbouring property at 12 Morgan Road and although the proposed first floor extension does encroach on a 45 degree line taken from the neighbouring window at 16 Morgan Road, this window is to a non habitable room.  The depth of 2.4m and the orientation of the properties also reduces the impact and the proposal is not considered to significantly affect light to the window of this neighbouring property.

- Visual dominance and overbearing effects of a development; NO, although the neighbouring properties will notice the additional height and depth of the proposal, the proposed first floor extension does not extend past the rear of the existing single storey element at 16 Morgan Road and although there will be a change in outlook from the first floor window at this neighbouring property the proposal is not considered visually dominant or overbearing.  The proposal is also away from the boundary with 12 Morgan Road by 1.8m which is considered acceptable.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time limit

- Materials

- Approved plans

- No balconies

Informatives relating to:

- Terms and conditions

2.	Need for building regulations

3.	Demolition and construction

4.	Encroachment and Party Wall

5. 	Positive and proactive