DELEGATED APPLICATION REPORT

Target Date: 27/10/2015				Extended Target Date: 24/11/2015

Application No: 151531/HOU

Address: 40 Blandford Road, Reading

Proposal: Single storey rear extension, front porch and swimming pool

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a two storey mid-terrace property within a row of four properties.  The property is located on the edge of a roundabout.  The surrounding area is predominately residential.

Proposal:

The application is for a single storey rear extension, a front porch and a swimming pool in the rear garden.

The proposed single storey rear extension measures 3m in depth, 4.9m in width and 2.8m in height with a flat roof.  The property has a side walkway between the application site and the adjoining property at 38 Blandford Road and the proposal is 1.4m from the boundary with this neighbouring property.  Due to the splayed boundary with the neighbouring property at 42 Blandford Road the proposed extension extends away from the boundary.  Bi-fold doors are proposed to the rear elevation.

The proposed front porch measures 1.3m in depth, 2.4m in width, 2m in height to eaves level and 3.2m in height at its highest point.  The proposed porch would have a pitched roof with a door and window to the front elevation.

The proposed swimming pool measures 6m x 3m and would be located 0.8m from the side boundary with the neighbouring property at 42 Blandford Road.  The proposal would have a depth varying from 1.2m to 1.8m and would be only marginally raised above ground level with a tile edging.  A pump is proposed which is to be installed in a timber clad cabinet.

The materials to the extensions will consist of red brick and tiles to match the existing property.

Plans Considered:

The applicant submitted the following drawings, received on 27th August 2015:

Drawing No. BRADY-0815-01 – Proposed Ground Floor Plan

Drawing No. BRADY-0815-02 – Proposed First Floor Plan

Drawing No. BRADY-0815-03 – Proposed Loft Plan

Drawing No. BRADY-0815-04 – Proposed Elevations

Drawing No. BRADY-0815-05 – Existing Plans & Elevations

Drawing No. BRADY-0815-06 – Block & Location Plans

The applicant submitted the following drawing, received on 17th September 2015:

Drawing No. BRADY-0815-07 – Swimming Pool

The applicant submitted the following amended drawings, received on 22nd October 2015:

Drawing No. BRADY-0815-01 Rev A – Proposed Ground Floor Plan

Drawing No. BRADY-0815-04 Rev C – Proposed Elevations

Drawing No. BRADY-0815-05 – Existing Plans & Elevations

Drawing No. BRADY-0815-06 Rev B – Block & Location Plans

Drawing No. BRADY-0815-07 Rev B – Swimming Pool

Relevant Planning History:

None.

Consultation Responses:

Transport raised no objection subject to conditions as the front of the property has been converted into a driveway and a footway crossing should be installed.

Ecology raised no objection.

Environmental Health advised that further to additional information being submitted with regards to the proposed pump for the swimming pool on the 18th November 2015 a noise assessment is not required.  However, as the applicant is not sure which type of pump will be installed a condition is recommended for details of the pump to be submitted to, and approved in writing by, the local planning authority.  An informative has also been recommended with regards to any external lighting.

Properties at 36, 38 and 42 Blandford Road and 2 Meadowcroft Road were consulted and no comments have been received.

Summary of discussions with applicant/agent:

The applicant originally proposed a flat roof rear dormer as part of the proposals however this was considered contrary to policy due to its size and flat roof.  The applicant was advised to submit a lawful development certificate for this element of the proposal and the rear dormer was removed from the application.  The applicant was also asked to provide additional information relating to the pump equipment for the swimming pool.  Amended plans were submitted on the 22nd October 2015.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

CS34 (Pollution and Water Resources)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

DM12: Access, Traffic and Highway-Related Matters

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO.

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO

- Access to sunlight and daylight; NO

- Visual dominance and overbearing effects of a development; NO

- Noise and disturbance; NO, details provided by the applicant on the 18th November 2015 stated that only one pump is required and it will run for 8 hours a day.  Environmental Health are satisfied that a noise assessment will not be required however, a condition will be imposed which requires details of the pump to be submitted to, and approved in writing by, the local planning authority which will mitigate any potential harm to neighbouring properties.

- Artificial lighting; NO, the agent advised an outdoor light is proposed to the rear extension only and no lights to the pool however, an informative will be added advising the applicant that external lighting can be considered a statutory nuisance.

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time Limit

- Materials

- Approved plans

- Access constructed before development

- Construction Method Statement

- Details of pump equipment

Informatives relating to:

- Terms and conditions

- Building Regulations

- Construction and demolition

- Encroachment and Party Wall

- Access construction

- External lighting

- Positive and proactive