DELEGATED APPLICATION REPORT 			Target Date: 07/10/2015

Application No: 151431/CLP

Address: 176 Hemdean Road, Caversham, Reading, RG4 7RA

Proposal:  Certificate of Lawfulness for a Proposed loft conversion with rear

flat roof dormer

Case officer: John Gregory

Site Description and Surrounding Area:

No.176 Hemdean Road is a Victorian brick built narrow end of terrace dwellinghouse.  The dwellinghouse has a part width outrigger on the inner part of the rear elevation with a single storey extension attached at the rear.  The dwellinghouse has also been extended by way of a rear roof extension.

The properties rear garden is orientated to the west and measures 21 metres deep.

There is a concurrent planning application ref. 150564 for a single storey rear extension to adjoining, No.176 Hemdean Road.

Proposal:

The application seeks a certificate of lawfull development to construct a rear facing, second floor roofslope dormer to create accommodation in the roof. This application follows from application 150564 from June 2015 to construct a rear extension.

Plans Considered:

Location Plan 03

Drawing No: 01 – Location Plan, Existing Ground Floor Plan and Existing Elevations

Drawing No: 02 – Proposed Block Plan, Proposed Ground Floor Plan and Proposed Elevations

Received on 10th August 2015

Relevant Planning History:

150565 Erection of single storey rear extension (submitted simultaneously

with a duplicate application for the erection of a single storey rear extension at No.175 Hemdean Road) (Granted)

Consultation Responses:

Relevant Planning Policy & Guidance:

Annex 8 "Lawfulness and the Lawful Development Certificate" of Planning Circular 10/97 'Enforcing Planning Control: Legislative Provisions and Procedural Requirements" sets out the procedure for assessing applications for certificates of lawfulness:

Para 8.26 states, "if the LPA are supplied with information satisfying them that the operations described in the application would be lawful, if instituted or begun at the time of the application, they shall issue a certificate to that effect; and, in any other case, they shall refuse the application." "The burden of proof in a LDC application is firmly on the applicant."

## Assessment:

A Certificate of Lawfulness for a Proposed Development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

The proposal must be assessed to ensure that it complies with the conditions Class B, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015, which relates to ‘the enlargement of a dwellinghouse consisting of an addition or alteration to its roof’.

Development is not permitted by Class B if:

- B.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); COMPLIES.

- B.1 (b) any part of the dwellinghouse would, as a result of the works, exceed the height of the highest part of the existing roof; COMPLIES.

- B.1 (c) any part of the dwellinghouse would, as a result of the works, extend beyond the plane of any existing roof slope which forms the principal elevation of the dwellinghouse and fronts a highway; COMPLIES.

- B.1 (d) the cubic content of the resulting roof space would exceed the cubic content of the original roof space by more than 40 cubic metres in the case of a terrace house; COMPLIES, the calculated volume is 39 cubic metres.

- B.1 (e) it would consist of or include (i) the construction or provision of a veranda, balcony or raised platform, or (ii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe; NOT APPLICABLE IN THIS INSTANCE or

- B.1 (f) the dwellinghouse is on article 2(3) land.  COMPLIES.

Development is permitted by Class B subject to the following conditions:

- B.2 (a) the materials used in any exterior work shall be of similar appearance to those used in the construction of the exterior of the existing dwellinghouse; COMPLIES.

- B.2 (b) the enlargement must be constructed so that –

- other than in the case of a hip-to-gable enlargement or an enlargement which joins the original roof to the roof of a rear or side extension – (aa) the eaves of the original roof are maintained or reinstated; and (bb) the edge of the enlargement closest to the eaves of the original roof is, so far as practicable, not less than 0.2 metres from the eaves, measured along the roof slope from the outside edge of the eaves; COMPLIES, and

- other than in the case of an enlargement which joins the original roof to the roof of a rear or side extension, no part of the enlargement extends beyond the outside face of any external wall of the original dwellinghouse; COMPLIES.

- B.2 (c) any window inserted on a wall or roof slope forming a side elevation of the dwellinghouse must be (i) obscure-glazed, and (ii) non-opening unless the parts of the windows which can be opened are more than 1.7 metres above the floor of the room in which the window is installed. NOT APPLICABLE IN THIS INSTANCE.

RECOMMENDATION:

The proposal has been assessed against Class B, Part 1, Schedule 2, of The Town and Country Planning (General Permitted Development) Order 1995 (as amended by the 2015 Order) and it has been confirmed that the proposal would be lawful, on the basis of the following submitted drawings received on 10th August 2015:

Location Plan 03

Drawing No: 01 – Location Plan, Existing Ground Floor Plan and Existing Elevations

Drawing No: 02 – Proposed Block Plan, Proposed Ground Floor Plan and Proposed Elevations

Therefore the Local Planning Authority will, on this occasion, issue the applicant with a Certificate of Lawfulness for the proposed development.

Recommendation:

GRANT Certificate