DELEGATED APPLICATION REPORT 				Target Date: 07/10/2015

Application No: 151442/HOU

Address: 22 Lower Armour Road

Proposal: Part two storey and part single storey and rear extension.

Case officer: Claire Ringwood

Site Description and Surrounding Area:

The application site is a two storey semi-detached property with an existing single storey rear extension and a large outbuilding in the rear garden. The property has a shallow two storey front and rear projection which matches the attached property at 24 Lower Armour Road.  The surrounding area is predominately residential.

Proposal:

The proposal involves the demolition of the existing single storey 3.2m rear extension and the construction of a part two storey and part single storey rear extension.

The single storey element of the proposal measures a maximum of 6m in depth (5.8m in depth from the rear of the existing projection) and 6m in width.  The single storey would have a flat roof and measures 2.7m in height with a roof lantern.

The two storey element of the proposal measures 4m in depth and 3.5m in width.  The proposal has a hipped roof and is set down from the existing roof by 0.8m although the eaves height will match the eaves height of the existing property.

The proposal introduces a first floor side window to the existing bathroom which will be conditioned to be fixed up to 1.7m in height above internal floor level and be obscure glazed.  A ground floor window to the existing side elevation will be replaced with a door along with a ground floor side window to the proposed extension and bi-fold doors to the rear.

The proposed single storey would be approximately 0.2m from the boundary with the adjoining property at 24 Lower Armour Road and 2.6m from the boundary at two storey.  The neighbouring property at 20 Lower Armour Road has a staggered side elevation and the proposal would be a distance of 5.6m and 6.8m between the side of the proposal and the side of this neighbouring property.

The materials will consist of white painted brickwork and plain clay tiles to match existing property.

Plans Considered:

The agent submitted the following drawings, received on 12th August 2015:

Site Location Plan

Drawing No: 02843:1 – Plans and Elevations

Relevant Planning History:

None.

Consultation Responses:

Transport raised no objection.

Ecology raised no objection.

Properties at 20 and 24 Lower Armour Road were consulted. One letter of representation has been received with regards to:

- Loss of light

- Loss of views.

Officer note: in terms of planning matters private views are not protected and this is not a material planning consideration.

- Loss of privacy.

- The proposal is out of character with the area.

- Negative impact on property value

Officer note: this is not a material planning consideration.

Summary of discussions with applicant/agent:

None.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

The following local and national planning policy and guidance is relevant to this application:

Reading Borough Local Development Framework Core Strategy (2008)

CS7 (Design and the Public Realm)

CS36 (Biodiversity and Geology)

Sites and Detailed Policies Document (Oct 2012)

DM4: Safeguarding Amenity

DM9: House Extensions and Ancillary Accommodation

Supplementary Planning Guidance - A Design Guide to House Extensions (2003)

Appraisal Checklist

DM9 states: An extension to a house will be acceptable where it:

- Respects the character of the house in terms of scale, location, materials and design;

To assess this, the following has been considered – would the proposal:

- detract from the appearance of the original building through the use of ill-matching materials and by not fitting in with the design, scale and proportions of the original building; NO, the proposal is to the rear of the property and is set down from the main roof and set in from the side boundary with the adjoining property.  Although the single storey element of the proposal is 6m in depth which exceeds the Council’s guidance of 4m, the house and garden are considered capable of taking an extension of this depth.  The proposal meets general policy and guidance.

- reduce the amount of usable amenity space for the property to an unacceptable degree; NO, the property has a large rear garden.

- require the loss of trees in the garden or put significant trees at risk; NO

- Respects the character and pattern of neighbouring properties and the street as a whole in terms of scale, location, materials and design, and any important existing building line; Respects neighbour’s amenities and does not present a large blank façade to public areas.

To assess this, the following has been considered – would the proposal:

- significantly reduce the amount of daylight and sunlight enjoyed by neighbouring properties; NO, the neighbouring property at 24 Lower Armour Road has an existing rear conservatory approximately 4.4m in depth with a predominantly brick side elevation along the boundary with the application site.  The proposed single storey rear extension is not considered to significantly reduce the amount of light to this property.  The proposed two storey element of the extension is 2.6m from the boundary with this neighbouring property and it does not infringe on an area measured at an angle of 45 degrees from the midpoint of the closest window to a habitable room at this neighbouring property as highlighted in the Council’s SPD.  This distance protects light reaching this property and the two storey proposal is not considered to reduce the amount of light to No. 24.  The neighbouring property at 20 Lower Armour Road has side windows facing the application site however there is a distance of between 5.6m and 6.8m from the side of the proposal and this neighbouring property.  The orientation of No. 20 and No. 22 and the distance between the properties means the proposal is not considered to cause a reduction in the amount of light to this neighbouring property.

- invade the privacy of neighbouring properties through overlooking from windows or balconies; NO, a condition will be imposed to restrict side windows in the two storey extension and another to require the proposed first floor side window in the existing property to be fixed up to 1.7m in height above internal floor level and obscure glazed which will protect the privacy of neighbouring properties.  One of the concerns raised related to a reduction in privacy to the rear garden of the neighbouring property however, the application property has two existing first floor rear facing windows and a new window in a similar position is not considered significantly different from the current arrangement.

- detract from the character of neighbouring properties through siting, excessive bulk, ill-matching materials or inconsistent design; NO, the proposal is of a standard design which follows general policy and guidance.

- detract from the appearance of the street or area by using ill-matched materials, glazing and detailing; NO

DM4: Safeguarding Amenity states an extension to a house will be acceptable where it will not cause a significant detrimental impact to the living environment of existing or new residential properties.

To assess this, the following has been considered – would the proposal cause a significant detrimental impact to the living environment of existing or new residential properties in terms of:

- Privacy and overlooking; NO, see comments above.

- Access to sunlight and daylight; NO, see comments above.

- Visual dominance and overbearing effects of a development; NO, the residents of the neighbouring properties at 20 and 24 Lower Armour Road will notice the additional height and depth of the proposed extension and although there will be a change in outlook for the occupiers of these neighbouring properties the proposal at two storey is a sufficient distance from the boundaries with No. 20 and No. 24 and is not considered visually dominant or overbearing.  The proposed single storey element of the proposal will extend approximately 1.6m further than the existing conservatory at 24 Lower Armour Road which is considered acceptable and not visually dominant or overbearing.

- Noise and disturbance; NO

- Artificial lighting; NO

- Vibration; NO

- Dust and fumes; NO

- Smell; NO

- Crime and safety; NO

RECOMMENDATION:

GRANT

Subject to the following conditions:

- Time Limit

- Materials

- Approved plans

- No first floor side windows

- Fixed and obscure first floor side window

Informatives relating to:

- Terms and conditions

- Building Regulations

- Contaminated land

- Construction and demolition

- Encroachment and Party Wall

- Positive and proactive