DELEGATED HOUSEHOLDER APPLICATION REPORT

Ward: Church

App Ref: 240833/CLP

Address: 18 Elm Road, Reading, RG6 5TS

Proposals: Proposed hip to gable roof extension, loft conversion with rear dormer extension and insertion of roof lights in the roof slope

Case Officer: Chukwudi Onwudinanti

Applicant: Mrs Anitharani Maddi

Date received (valid from): 4 July 2024

Target Decision date: 29 August 2024

1.	INTRODUCTION

The proposal site is a semi-detached two storey dwelling on the southern side of Elm Road, one of several semi-detached two storey dwellings of the same style and scale on the street.

The application site is not a listed building or located in a conservation area. It is not in an area subject to any Article 4 restrictions.

PROPOSAL AND SUPPORTING INFORMATION

This proposal is for a Certificate of Lawfulness for various works to the dwellinghouse. The proposals include:

- a rear dormer roof extension,

- three rooflights on the front roofslope

- hip to gable roof conversion

- Install two windows on the new rear dormer

- Amendments to ground floor windows and doors

Plans and Documents Considered dated 20/06/2024 received 01/07/2024

Drawing No ER-01: Location and Block Plans

Drawing No ER-02: Existing Plans

Drawing No ER-03: Existing Elevations

Drawing No ER-04: Proposed Plans

Drawing No ER-05: Proposed Elevations

RELEVANT PLANNING HISTORY

None

CONSULTATIONS

No consultations are required for this application.  However, 20 Elm Road submitted comments.

Equality Act 2010:

In determining this application the Council is required to have regard to its obligations under the Equality Act 2010. However, there is no indication or evidence (including from consultation on the application) that the protected groups identified by the Act have or will have different needs, experiences, issues and priorities in relation to this particular planning application.  Therefore in terms of the key equalities protected characteristics it is considered there would be no significant adverse impacts as a result of the development.

Relevant Planning Policy and Guidance

National Planning Practice Guidance

The Town and Country Planning (General Permitted Development) (England) Order 2015 (GPDO) (as amended)

Permitted development rights for householders – Technical Guidance September 2019

A Certificate of Lawfulness for a Proposed Development has been submitted as the applicant considers the proposal falls within the permitted development rights of the property.

## ASSESSMENT UNDER CLASS A

The following elements of the proposals are considered under Class A:

- Install two windows on the new rear dormer

- Amendments to ground floor windows and doors

The proposal must be assessed to ensure that it complies with the conditions Class A, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended), which relates to ‘The enlargement, improvement or other alteration of a dwellinghouse’.

A.1 Development is not permitted by Class A if:

A.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use) Complies

A.1 (b) as a result of the works, the total area of ground covered by buildings within the curtilage of the dwellinghouse (other than the original dwellinghouse) would exceed 50% of the total area of the curtilage (excluding the ground area of the original dwellinghouse) NA

A.1 (c) the height of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the highest part of the roof of the existing dwellinghouse NA

A.1 (d) the height of the eaves of the part of the dwellinghouse enlarged, improved or altered would exceed the height of the eaves of the existing dwellinghouse NA

A.1 (e) the enlarged part of the dwellinghouse would extend beyond a wall which (i) forms the principal elevation of the original dwellinghouse; or (ii) fronts a highway and forms a side elevation of the original dwellinghouse NA

A.1 (f) subject to paragraph (g), the enlarged part of the dwellinghouse would have a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 4 metres in the case of a detached dwellinghouse, or 3 metres in the case of any other dwellinghouse, (ii) exceed 4 metres in height NA

A.1 (g) for a dwellinghouse not on article 2(3) land nor on a site of special scientific interest, the enlarged part of the dwellinghouse would have a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 8 metres in the case of a detached dwellinghouse, or 6 metres in the case of any other dwellinghouse, or (ii) exceed 4 metres in height NA

A.1 (h) the enlarged part of the dwellinghouse would have more than a single storey and – (i) extend beyond the rear wall of the original dwellinghouse by more than 3 metres, or (ii) be within 7 metres of any boundary of the curtilage of the dwellinghouse opposite the rear wall of the dwellinghouse N/A

A.1 (i) the enlarged part of the dwellinghouse would be within 2 metres of the boundary of the curtilage of the dwellinghouse, and the height of the eaves of the enlarged part would exceed 3 metres NA

A.1 (j) the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse, and would – (i) exceed 4 metres in height, (ii) have more than a single storey, or (iii) have a width greater than half the width of the original dwellinghouse N/A

A.1 (k) it would consist of or include – (i) the construction or provision of a verandah, balcony or raised platform (ii) the installation, alteration or replacement of a microwave antenna, (iii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe, or (iv) an alteration to any part of the roof of the dwellinghouse N/A

A.2 In the case of a dwellinghouse on article 2(3) land, development is not permitted by Class A if:

A.2 (a) it would consist of or include the cladding of any part of the exterior of the dwellinghouse with stone, artificial stone, pebble dash, render, timber, plastic or tiles N/A as not on article 2 (3) land

A.2 (b) the enlarged part of the dwellinghouse would extend beyond a wall forming a side elevation of the original dwellinghouse N/A as not on article 2 (3) land

A.2 (c) the enlarged part of the dwellinghouse would have more than a single storey and extend beyond the rear wall of the original dwellinghouse  N/A as not on article 2 (3) land

CONDITIONS UNDER CLASS A

Development is permitted by Class A subject to the following conditions:

A.3 (a) the materials used in any exterior work (other than materials used in the construction of a conservatory) must be of a similar appearance to those used in the construction of the exterior of the dwellinghouse Complies (as shown on Drawing No ER-05: Proposed Elevations)

A.3 (b) any upper-floor window located in a wall or roof slope forming a side elevation of the dwellinghouse must be (i) obscure-glazed, and (ii) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed Complies (as shown on Drawing No ER-05: Proposed Elevations)

A.3 (c) where the enlarged part of the dwellinghouse has more than a single storey, or forms an upper storey on an existing enlargement of the original dwellinghouse, the roof pitch of the enlarged part must, so far as practicable, be the same as the roof pitch of the original dwellinghouse NA

ASSESSMENT UNDER CLASSES B AND C

The following element of the proposals are considered under Class B:

- a rear dormer roof extension,

- hip to gable roof conversion

The following elements of the proposals are considered under Class C:

- three rooflights on the front roofslope

The proposed loft conversion with rear dormer, front rooflights and hip to gable roof conversion must be assessed to ensure that they comply with the relevant conditions of Class B and Class C, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended), which relates to ‘additions etc to the roof of a dwellinghouse’ and ‘other alterations to the roof of a dwellinghouse’ respectively.

CLASS B

The relevant conditions of the GPDO for a loft conversion with rear dormer which is not listed or in a conservation area, are as follows:

B.1 Development is not permitted by Class B if:

B.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); complies

B.1 (b) any part of the dwellinghouse would, as a result of the works, exceed the height of the highest part of the existing roof; complies

B.1 (c) any part of the dwellinghouse would, as a result of the works, extend beyond the plane of any existing roof slope which forms the principal elevation of the dwellinghouse and fronts a highway; complies

B.1 (d) the cubic content of the resulting roof space would exceed the cubic content of the original roof space by more than

(i) 40 cubic metres in the case of a terrace house:

(ii) 50 cubic metres in any other case

Complies, as the cubic content of the resulting roof space would exceed the cubic content of the original roof space by 34.57m3

B.1 (e) it would consist of or include (i) the construction or provision of a veranda, balcony or raised platform, N/A / complies or

(ii) the installation, alteration or replacement of a chimney, flue or soil and vent pipe; N/A / Complies

B.1 (f) the dwellinghouse is on article 2(3) land. complies

CONDITIONS UNDER CLASS B

B.2 Development is permitted by Class B subject to the following conditions:

B.2 (a) the materials used in any exterior work must be of a similar appearance to those used in the construction of the exterior of the existing dwellinghouse; complies

B.2 (b) the enlargement must be constructed so that (i) other than in the case of a hip-to-gable enlargement or an enlargement which joins the original roof to the roof of a rear or side extension (aa) the eaves of the original roof are maintained or reinstated; complies and (bb) the edge of the enlargement closest to the eaves of the original roof is, so far as practicable, not less than 0.2 metres from the eaves, measured along the roof slope from the outside edge of the eaves; complies and (ii) other than in the case of an enlargement which joins the original roof to the roof of a rear or side extension, no part of the enlargement extends beyond the outside face of any external wall of the original dwellinghouse; complies and

B.2 (c) any window inserted on a wall or roof slope forming a side elevation of the dwellinghouse must be (i) obscure-glazed, N/A / complies and (ii) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed, N/A / complies

CLASS C

C.1 Development is not permitted by Class C if:

C.1 (a) permission to use the dwellinghouse as a dwellinghouse has been granted only by virtue of Class M, N, P or Q of Part 3 of this Schedule (changes of use); complies

C.1 (b) the alteration would protrude more than 0.15 metres beyond the plane of the slope of the original roof when measured from the perpendicular with the external surface of the original roof; complies

C.1 (c) it would result in the highest part of the alteration being higher than the highest part of the original roof; complies or

C.1 (d) it would consist of or include (i) the installation, alteration, or replacement of a chimney, flue or soil and vent pipe, complies or (ii) the installation, alteration or replacement of solar photovoltaics or solar thermal equipment, N/A / complies

CONDITIONS UNDER CLASS C

C.2 Development is permitted by Class C subject to the condition that any window located on a roof slope forming a side elevation of the dwellinghouse must be:

C.2 (a) obscure-glazed; complies and

C.2 (b) non-opening unless the parts of the window which can be opened are more than 1.7 metres above the floor of the room in which the window is installed, complies

OVERALL RECOMMENDATION

Sufficient evidence has been provided to demonstrate that a Certificate of Lawfulness should be granted for the proposed rear dormer roof extension, rooflights on front slope, hip to gable conversion and installation of windows on the rear side of the new dormer.

The proposal has been assessed against the Town and Country Planning Act 1990 (as amended) and Class A, Class B, and Class C, Part 1, Schedule 2 of the Town and Country Planning (General Permitted Development) (England) Order 2015 (as amended). It can be confirmed that the proposed operations would be lawful for planning purposes on the basis of the following:

Drawing No ER-01: Location and Block Plans

Drawing No ER-02: Existing Plans

Drawing No ER-03: Existing Elevations

Drawing No ER-04: Proposed Plans

Drawing No ER-05: Proposed Elevations

Therefore the Local Planning Authority can issue the applicant with a Certificate of Lawfulness for the proposed development.

Informatives

Building Regulations

Construction and Demolition

Encroachment and Party Wall Act

Block Plan (Not to Scale)