EXTRACAIR INSTALLATIONS LTD

REFRIGERATION-AIR CONDITIONING-VENTILATION

Mechanical Plant Statement

Siren Craft Brewery

Client : Siren

Project: New Siren Craft Brewery

Station Hill, Friar Street, Reading.

INTRODUCTION

The site is located on a busy main road with residential dwellings above.

The building has a purpose constructed screened flat plant area provided for the main building and commercial tenant’s mechanical equipment.

(Located on level 7)

Proposal

The kitchen extract fan to be a purpose high temperature unit with the motor mounted outside of the airstream. The System Air MUB kitchen fan has an easily removable access door for cleaning and has good air movement to sound level properties. The fan will be inverter driven to allow precise speed control and operate on a timed system to automatically shut the fan down at or before 11 pm. The fan has insulated side panels and attenuation is proposed to both intake and exhaust sides of the fan.

The fan/ termination point is over 30 meters away from the nearest residential window.

Air conditioning plant.

4 off Toshiba high efficiency units are proposed which have soft start systems to ensure close to silent running. Nearest unit is over 25 meters away from the nearest residential window.

Drawing LX10937/1/rev4. Details the roof plant. Information is provided in a chart estimating the sound level at the nearest noise sensitive window to be bellow 40 dBA so we do not perceive any issues.