| Tree Group, Hedge or Shrub No. | SPECIES | AGE | HEIGHT S-VL | PHYSIO- LOGICAL STATUS Good Fair Poor Dead | STRUCT-URAL STATUS Good Fair Poor Dead | OBSERVATIONS | RECOMMENDATIONS | S.O.P + L.O.F + T.A.O 1-3 | Action Relative To Score 3-9 | Advised Time Scale In months from the date of survey |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| T5 | Horse Chestnut Aesculus hippo-castanum | M | L | Fair | Fair | Prominent tree over hanging the car park and drive. One dead branch approx. 1.5m in length present. One small area of exudation, not a concern at this time. No evidence of bleeding canker or necrotic bark strips at this time. Access required for high sided vehicles. | - Remove dead branch and any other deadwood >40mm in diameter or over 1m in length. - Lift the crown to a height above ground level of 4m. | 2 1 2 | 5 | 12 |
| T10 | Tree of Heaven Ailanthus altissima | M | L | Fair | Fair | Prominent tree over hanging the car park and drive. Tree within 1m of the buildings on the northern and southern quadrants. Medium deadwood present, throughout consistent with a tree of this age and species. | - Lift the crown to a height above ground level of 4m. - Selectively prune to give 2m clearance to the buildings. This will reduce their radial spread from 6m to 4m. - Remove deadwood >40mm in diameter or over 1m in length. | 2 2 2 | 6 | 6 |

| G1 | Sycamore Acer pseudo-platanus | M | L | Fair | Fair | Group of trees located on the rear northern boundary. Low canopy over the garden. Deadwood present consistent with trees of this age and species. These trees are shading out the apartments on the northern side. The mid-canopy of these trees are approximately 2m from the apartments. | - Lift the crowns on the garden side to a height above ground level of 4.5m. - Remove deadwood >40mm in diameter or over 1m in length. - Selectively reduce the indicated branches to give 4m clearance to the building. This will reduce the radial spread from 7m To 5m. | 2 2 2 | 6 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G1.1 | Sycamore Acer pseudo-platanus | EM | M | Fair | Fair/ Poor | Tree with tag 0724. A minor component of tree group G1. Multiple wounds and large cavity present. Heavily suppressed form with completely off-centre canopy. | - Remove as close to ground level as practicable. - Treat with a suitable herbicide to prevent regeneration. | 2 1 2 | 5 | 12 |
| G1.2 | Sycamore Acer pseudo-platanus | EM | M | Fair | Fair/ Poor | Tree with tag 0721. A minor component of tree group G1. Large cavity present at the stem base. Heavily suppressed form with completely off-centre canopy. Evidence of historic branch failure. | - Remove as close to ground level as practicable. - Treat with a suitable herbicide to prevent regeneration. | 2 1 2 | 5 | 12 |

| G1.3 | Common Ash Fraxinus excelsior | M | L | Fair | Fair | Major component of G1. Low canopy over the garden. Deadwood present consistent with a tree of this age and species. | - Lift the crowns on the garden side to a height above ground level of 4.5m. - Remove deadwood >40mm in diameter or over 1m in length. | 2 2 2 | 6 | 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |