Environmental Risk Assessment

Land to the Rear of 29-30 Tern Close, Tilehurst, Reading

February 2022

This reported has been prepared by Laura Farrell (BSc Environmental Science ((Hons)).

Contents

- Site address and details

- Current setting

- Assessment basis

- Summary including ground contamination summary and flood risk summary

- Assessment including ground contamination assessment and other issues assessment

- Site inspection

- History of the site and surrounding area

- Regulatory authority information including review of Landmark Envirocheck database, Planning Department information and underground pipelines

- Environmental setting

- Radon, ground gas and vapour

- Ground stability

- Preliminary Conceptual Site Model

Appendix 1 – Historical Maps

Appendix 2 – Environmental Database

| Address and Site Details |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Address: |  | Land to the Rear of 29-30 Tern Close, Tilehurst, Reading |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NGR: |  | 468360, 173030 | Area: |  |  | 0.06 hectares (ha) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Site use: |  | Derelict car parking garages |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Date: |  | c.1971 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Current Setting The site is located approximately 3.2km southwest of Reading town centre and is in the parish of Tilehurst. Residential properties with gardens border the site directly to the north, east and west of the site. Tern Close and associated roundabout borders the site directly to the south and further residential properties with gardens are beyond this. Tern Close was observed to slope downhill from west to east. The residential properties to the north of the site are topographically upgradient to the site and were noted to slope downhill from north-west to the southeast. The garden of 28 Tern Close adjacent to the west/ south-west of the site was observed to be predominantly covered in concrete hardstanding which visually appeared to be in good condition. 30 Tern Close adjacent to the west/ south of the site is approximately 0.5m above the subject site. Beyond the residential properties adjacent to the east of the site lies Taff Way with Lousehill Copse (Local Nature Reserve) 30m south-east of the site. Taff Way was observed to slope gently downhill from north to south. At the roundabout joining Taff Way and Tern Close the road furthest away from the subject from the site was observed to be topographically upgradient from the site by approximately 0.5m. Car parking garages adjacent to the west of the site. Car parking garages associated with Tern Close are located 10m west of the site and this was noted to be topographically upgradient of the subject site by approximately 0.5m. The topography of this car park was noted to gently slope downhill from west to east. The car parking garages (single storey structure) were noted have an area surfaced in asphalt hardstanding directly to the south of them and was observed to be in good condition. Adjacent to the east of the single storey car parking building is a concrete plinth (based on a review of historical mapping these would also have been former car parking garages and have subsequently been demolished) which was observed to have evidence of fly tipping of compost and barbeques. No evidence of vegetation die back was noted adjacent to this area of flytipping. Directly beyond the western boundary if the site is an area of grassland (also part of the car park) and no evidence of vegetation die back was observed in this area. The access road to the adjacent car park via Tern Close was observed to slope downhill from north to south and was observed to be surfaced in asphalt hardstanding which was noted to be generally in good condition with surface cracks in places. An electricity substation the responsibility of SSE is located 20m west of the site within the adjacent car park to the west. Directly in front of the electricity substation is an area of grassland and no evidence of vegetation die back was noted in this area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Assessment Basis |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☒ | Site Inspection |  |  | ☐ | Environmental Health Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☒ | Map History |  |  | ☐ | Environment Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☒ | Environmental Database Review |  |  | ☐ | Petroleum Licensing Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☒ | Planning History |  |  | ☒ | Geological and Hydrogeological Review |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ☐ | Review of Previous Reports |  |  | ☐ | Mining Records Search |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4a Ground Contamination Summary No potential sources of ground contamination have been identified associated with the current or historic uses of the site (derelict car parking garages formerly used by adjacent residential properties). |  |  |  |  |  |  | Low Risk |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4b Flood Risk Summary According to the Environment Agency (EA) website, the site lies in Flood Zone 1 (low probability). This zone comprises land assessed as having a less than 1 in 1000 annual probability of river or sea flooding in any year (<0.1%). |  |  |  |  |  |  | Flood Zone 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| - Assessment |  |  |  |  |  |  |  |  |  |  |
| 5a Ground Contamination Assessment The site comprises of two single storey disused car parking garages which were formerly used by residents on Tern Close. The car parking garages were built in the 1971 at the same time as residential properties on Taff Close adjacent to the north and east of the site. The remainder of the site is surfaced in concrete and asphalt hardstanding which was observed visually to be in good condition. The car parking garages are understood to have been disused for 25 years. Overall based on the information provided and the site walkover current activities are not considered to represent a significant source of contamination. Potentially contaminative current and historical land uses have been identified in the surrounding area, including various works, tile and pottery works, clay pits, railway lines, gravel pits, a chalk pit, a hospital, smithy, petrol filling stations, miniature rifle ranges, electricity substations and a fire station. These activities in the surrounding area therefore reduce the sites identifiability as a unique source of contamination (if present). The site is considered to be located in a low-moderate sensitivity setting with regard to groundwater resources. The bedrock underlying the site is classified as Unproductive stratum (London Clay Formation) and is overlain by superficial Winter Hill Gravel Formation (sand and gravel) classified as a Secondary A Aquifer. The site is not situated within a groundwater Source Protection Zone (SPZ) and there are no sensitive groundwater abstractions located within 2km of the site reducing its sensitivity The site is considered to be located in a low sensitivity setting with regard to surface water resources. The nearest surface water feature to the site is an unnamed drainage ditch in Lousehill Copse Local Nature Reserve 199m north of the site. The Environment Agency does not currently classify this drainage ditch under the Water Framework Directive classification scheme. In the wider surrounds the Holy Brook 1.3km south of the site and is classified by the EA as being of ‘moderate’ ecological status. In the wider surrounds the River Thames is situated 1.58km north of the site and is classified by the EA as being of ‘moderate’ ecological status. There are no surface water abstractions within a 2km radius of the site. In conclusion, no ongoing sources of contamination have been identified at site and historical land uses are unlikely to have caused significant land contamination. Furthermore, the proposed redevelopment includes building cover and hardstanding across the majority of the site which will offer a significant degree of protection to site end users from direct contact with any residual contamination (if present). This will also reduce the further rainfall infiltration and the subsequent leaching of any contaminants, if present beneath the site. Given the permeable nature of the underlying geology there is the potential for the lateral migration of contaminants from off-site sources onto the site, however it is noted that the site is topographically at a higher elevation from any current/ historical sources of contamination therefore the potential impact is considered to be negligible. In addition, given that significant contamination has not been identified onsite it is considered unlikely that any impacts to nearby receptors would be solely attributable to the subject site. Based on the information available the site is considered to be suitable for its proposed end uses and is considered to be of low risk with respect to contaminative liabilities or costs as defined under Part IIA of the Environmental Protection Act 1990 and associated legislation. In the event that unforeseen contamination is discovered during re-development of the site, all works will cease and the Planning Department of Reading Borough Council will be contacted. A remediation scheme shall then be prepared and submitted in writing to the Planning Department of Reading Borough Council. The remediation scheme will include details such as the proposed objectives, remediation criteria, an appraisal of remedial options, proposal of the preferred option(s) and a timetable of works and site management procedures. All of the aforementioned be agreed with the Planning Department of Reading Borough Council. Subsequently a validation report which demonstrates the effectiveness of the remediation carried out will be submitted to Reading Borough Council Planning Department prior to the construction of the development. |  |  |  |  |  |  |  |  |  |  |
| 5b Other Issues Assessment According to the EA website, the site lies in Flood Zone 1 (low probability). This zone comprises land assessed as having a less than 1 in 1000 annual probability of river or sea flooding in any year (<0.1%). |  |  |  |  |  |  |  |  |  |  |  |  |
| - Site Inspection The following information was derived from a site inspection undertaken by Laura Farrell on the 21st February 2022. The Site The site comprises of two single storey disused car parking garages which were formerly used by residents on Tern Close. The car parking garages were built in 1971 at the same time as residential properties on Taff Way adjacent to the north and east of the site. The site is an irregular shaped parcel of land, with an approximate area of 0.06 hectares and is accessed to the south via Tern Close. The garages are understood to have been disused for 25 years. The garages are situated in the north of the site and in the east of the site which are both of brick construction with corrugated metal roofs. Both of the garages were observed to be surfaced in concrete hardstanding which was visually observed to be in good condition. No evidence of surface water drainage was observed within both buildings. The garage in the north of the site was noted to have concrete hardstanding directly to the south which was observed to be in good condition. An area of asphalt hardstanding in between the two garages was observed to visually be in good condition and was noted to gently slope downhill from the northwest- to the southeast. The remainder of the site was observed to be surfaced in concrete hardstanding which was observed to be visually in good condition. The area of the site directly to the north of 29/30 Tern Close (with the car parking garage in the north) was observed to be approximately 0.5m above the remainder of the site. Generally, the remainder of the site (with the car parking garage in the east) was observed to slope gently downhill from north to south. Building Construction and Asbestos Containing Materials (ACMs) The roof of the garages were noted to comprise of corrugated metal which is likely to contain asbestos given the age of the buildings (c.1971). The roofs were observed to be in good condition. It is recommended that the roofs are tested for ACMs and if relevant are removed by a suitably licensed contractor and are disposed of at a suitably licensed facility. All relevant records should be retained on file. Storage of Chemicals and Hazardous Substances Underground Storage No indicators of USTs were observed during the site inspection (such as unexplained manhole covers or vent pipes). Above Ground Storage No ASTs were observed during the inspection. No further oil/ chemical storage was observed onsite. No spillage or staining was noted on the good quality hardstanding. Emissions to Air N/A Water, Wastewater and Drainage No site drainage plans were available nor was any surface water drainage identified onsite. No oil interceptors were observed as being present at the site. Waste Disposal Internally of the garages there is evidence of domestic furniture. All of the garages are surfaced in concrete hardstanding which was noted visually to be in good condition. Note that all waste will be removed by a suitably licensed contractor and records will be retained on file prior to re-development. Polychlorinated Biphenyls (PCB) No equipment was observed during the inspection that could potentially contain PCBs (i.e. electrical sub-stations). Ozone Depleting Substances N/A |  |  |  |  |  |  |  |  |  |  |  |  |
| History of the Site and Surrounding Area A review of historical maps obtained from Landmark Information Group has been undertaken. The Site Based on the first historical map dated 1881 the site comprises undeveloped land presumed to be in agricultural use. No further changes were identified to the site until 1971 when the site was developed with three sets of adjoining rectangular features presumed to be car parking garages associated with the development of Tern Close in the surrounding area. The three car parking garages border the eastern, northern and southern boundary of the site. The car parking garage in the south of the site appears to have been demolished by 1999. No further changes on subsequent mapping were identified on the most recent mapping dated 2022. The Surrounds Source; Surroundings. 1881, County Series, 1:2,500; The surrounding land is predominantly undeveloped presumed to be in agricultural use.. 1883, County Series, 1:1:10,560; Brick kilns 500m north-east (later identified as Grovelands Brick and Tile and Pottery works by 1900 which extended to approximately 600m northeast of the site. The clay pits extended to the northeast by 1914. No further changes were identified until the site was identified as a works by 1961 and the clay pits were no longer identified. The site was predominantly re-developed with residential dwellings by 1976 and completely redeveloped with residential dwellings by 1983.) Norcot kiln (Bricks) 850m north (by 1900 was noted to have two clay pits 750m north and 850m north of the site). The kiln was redeveloped with residential dwellings with gardens by 1932. The two clay pits were no longer present by 1976 and formed a part of Comparts Plantation.. 1899, County Series, 1:2,500 1900, County Series, 1:1,10,560; Brick works and clay pit 130m south (later renamed as Prospect Park Brick works by 1911, the clay pit also extended to the west and south at this stage and additional ancillary buildings were present including a kiln. A railway line was also developed linking the clay pit to the ancillary buildings. By 1932, tanks were present. The site was later identified as a works with additional ancillary buildings by 1961. The railway line was no longer present at this stage. On the former clay pit the Meadway Precint was developed in 1971. A garage was also built 185m south-east of the site in 1971 on the former works. It is understood that the site garage site has been in use as a car wash for the past ten years. Residential flats were redeveloped on the former works in 1999 and further residential houses with gardens were also developed between 1999 and 2022). Westwood Kiln Brick and Tile works 160m north-east (by 1911 a Clay pit was present, further buildings associated with the kiln were built and a railway is present connecting the buildings to the clay pit. In 1932 the site expands its clay pit significantly and is 95m north-east of the site with further buildings built. The railway line built in 1911 is no longer present and a new railway line is built in 1932. By 1961 Westwood Kiln Brick and Tile works and all associated buildings, railway line and clay pit are no longer identified. This whole area was re-developed with a series of depots, works, an electricity substation and residential properties with gardens by 1976. No further changes were identified on subsequent map editions). Old Clay pit 360m east (no longer by 1914) Old Clay pit 480m east (no longer identified by 1914) Tanks 500m north-east (no longer identified by 1961) Old gravel pits 600m north (no longer identified by 1976) Old gravel pit 660m north (no longer identified by 1976) Another brick works called Prospect Park Brick Works with associated clay pit 665m east Smithy 790m south (no longer identified by 1932) Old gravel pit 800m north (no longer identified by 1976) Old gravel pit 830m east (re-developed with residential dwellings by 1914) Cycle works 885m north-east (no longer identified by 1914) Old chalk pit 925m east (later identified as allotment gardens by 1932 and later redeveloped as residential properties by 1976) Old gravel pit 965m north (redeveloped with residential properties with gardens in 1961) Old gravel pit 970m south-east (no longer identified by 1932). 1911, County Series, 1:2,500 1914, County Series 1:10,560; Brick works with an associated clay pit and two kilns 130m east (no longer identified as a brick works by 1932 and subsequently re-developed with residential properties with gardens in the 1960s). Park Hospital 310m south-east (the hospital was extended further to the south and southeast by 1932 and also in 1938. The hospital was renamed as Prospect Park Hospital by 1961. The hospital was comprehensively re-developed by 2022. A clay pit 980m north-west (no longer identified by 1976 and later redeveloped with a Public House by 1983).. 1932, County Series, 1:2,500 1932, County Series, 1:10,560; Clay pits 230m north associated with Grovelands Brick, Tile and Pottery works (a miniature rifle range is identified in the clay pit in 1961 and clay pit was no longer identified at this stage. The area surrounding the rifle range was identified as part of Lousehill Copse by 1961 The miniature rifle range was later identified as part of Lousehill Copse by 1971) Tank 230m north associated with Grovelands Brick, Tile and Pottery works (no longer identified by 1961) Miniature rifle range 250m east (subsequently redeveloped as a depot in 1976). Pulsometer works 990m north-east (later identified as a works in 1961, additional works are built adjacent in 1976, and later identified as superstores in 1999).. 1938, County Series, 1:10,560; No significant changes identified in the surrounding area.. 1961, County Series, 1:2,500 1962, County Series, 1:2,500; Although this does not feature on historical mapping the electricity substation 20m west of the site is likely to have been built in the 1960s at the same time as the residential properties on Tern Close. A series of adjoining rectangular features are present adjacent to the west of the site likely to be the car parking garages associated with the development of the surrounding Tern Close. Residential properties are present adjacent to the south of the site, to the southwest and beyond Tern Close to the south. In the wider surrounds (adjacent- 750m) to the west, south-west and north-west residential developments were built.. 1971, County Series, 1:2,500 1976, County Series, 1:10,000; An electricity substation is present 135m north-east of the site. A fire station is present 360m north-east of the site. A works is present 770m south of the site.. 1983, County Series, 1:10,000; No significant changes to the surrounding area were identified on subsequent map editions.. 1999, County Series, 1:10,000; Although not identified on historical mapping, the Envirocheck dataset identified a petrol filling station (currently Tesco express) 810m south of the site with a Part B Environmental Permit authorised on the 8th September 1999.. 2022, County Series, 1:10,000; No further significant changes to the surrounding area were identified on the most recently available map edition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Regulatory Authority Information Environmental Database Information from an independent, third-party environmental database is reported below. Data Type; Distance; Relevant records within 250m of the site. On Site; < 250m; 250-500m; 500m-1km. Contaminated Land Register enquiries; 0; 0; 0; 0; None. Pollution incidents; 0; 0; 0; 2; A pollution incident occurred 710m north of the site involving unknown oils on the 9th June 1994. This was classified by the EA as a Category 3 Minor Incident. A spillage of unknown sewage occurred 918m north-west of the site on 7th December 1992 and was classed by the EA as a Category 3 Minor Incident.. Prosecutions or enforcement actions; 0; 0; 0; 0; None. Recorded landfill sites; 0; 0; 0; 0; None. Licensed waste management facilities; 0; 0; 0; 0; None. Environmental Permits; Part A(1); 0; 0; 0; 0; None. Part A(2); 0; 0; 0; 0; None. Part B; 0; 1; 0; 1; The nearest authorised Part B Environmental Permit relating to an Esso Petrol Filling Station 185m southeast of the site (permit reference number Epa/Par.B/42/98). Note on historical mapping this first appears in 1971 and although historical mapping does not indicate when this site became a car wash based on local knowledge it has been in use as a car wash for the past 10 years. In the wider surrounds there is one authorised Part B Environmental Permits relating to Calcot Service Station (a Tesco express service station) 810m south of the site (permit reference number Vr2).. Control of Major Accident Hazards Sites (COMAH); 0; 0; 1; 0; None. Fuel stations; 0; 0; 0; 1; Calcot Service Station/ Southcote express (Tesco express) service station is situated 810m south of the site.. Contemporary trade directory entries; 0; 3; 22; 41; There are three active contemporary trade directory entries within a 250m radius of the site and these include Brewers Decorators Centre (painting and decorating supplies) 130m northeast, N & B Foods (meat product manufacturers and wholesalers) 205m east and Tilehurst Launderama (dry cleaners) 220m south. Note that according to the Envirocheck database that the Tilehurst Launderama’s Part B Environmental Permit was revoked.. Registered radioactive substances; 0; 0; 0; 0; Information on certain radioactive substance authorisations is not publicly accessible. EA discharge consents; 0; 0; 0; 1; A discharge consent is registered to M I Whiteway 950m southwest of the site for the discharge of final/treated effluent into a tributary of the Holy Brook. The consent was issued in November 2004 and its current status is not known.. Radon affected area (Y/N); N; -; -; -; N/A. Designated ecological sites; 0; 1; 0; 2; Lousehill Copse Local Nature Reserve 30m southeast of the site designated by Natural England 22nd November 1992. Blundells Copse Local Nature Reserve 906m northwest of the site designated by Natural England 22nd November 1992. Round Copse Local Nature Reserve 962m northwest of the site designated by Natural England 10th July 2002.. Ancient woodland; 0; 0; 0; 0; N/A. Archaeological sites and Ancient Monuments There are no archaeological or ancient/ scheduled Monuments recorded onsite. Nor are there any archaeological or ancient/scheduled monuments recorded within 2km of the site. The site is not situated within a Conservation Area. The nearest Conservation Areas are situated 875m west of the site (Routh Lane) and Horncastle 890m southwest of the site. Planning Department A review of the Reading Borough Council online planning portal has revealed the following applications of interest (date accessed 19/02/2022): 150400: Demolition of derelict garages & construction of 4 no1 bedroom houses with associated parking. Planning application refused 30 April 2015. - The following pre-commencement conditions associated with planning permission reference 211000 are detailed below: - No development shall take place (including demolition and any preparatory works) until an assessment of the nature and the extent of contamination has been submitted to and approved in writing by the Local Planning Authority. This assessment must be undertaken by a competent person, and shall assess any contamination on the site, whether or not it originates on the site. Moreover it must include: - a survey of the extent, scale and nature of contamination; - an assessment of the potential risks to: - human health, - property (existing or proposed) including buildings, crops, livestock, pets, woodland and service lines and pipes. - adjoining land. - groundwaters and surface waters - ecological systems - archaeological sites and ancient monuments Underground Pipelines According to the LinesearchbeforeUdig database, there are no records of underground pipelines on the site or within 250m. The LinesearchbeforeUdig database lists pipelines distributing crude oil and refined hydrocarbon products owned and/or operated by a number of UK pipeline operators, including BPA, BP, ConocoPhillips, Esso, Government Pipelines and Storage System, Sabic, Shell and Total. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Environmental Setting Geology/Hydrogeology The solid geology BGS map (Sheet 268, Reading) and BGS online geology viewer indicates that the underlying Superficial deposits (Winter Hill Gravel Formation) is classified as a Secondary A Aquifer which may be important in supporting local abstractions or in providing baseflow to rivers and streams. This is further underlain by Unproductive Strata (London Clay Formation) which contains insignificant quantities of vertically or laterally extensive groundwater. A summary of the anticipated geology of the site is provided below: Stratum; Area Covered; Estimated Thickness; Typical Description. Made Ground; Entire site; 45cm~; Concrete/ asphalt. Winter Hill Gravel Formation (River Terrace Deposits); Entire site; 2-4m; Sand and gravel occasionally clayey. London Clay Formation; Entire site; 1m- 8.5m; Clay, variably silty with beds of sand and silt and flint pebble seams. Lambeth Group (Reading beds and Upnor Formation); Entire site; 39m ~; Mottled clay with sand beds with flint nodules at base. Upper Chalk Formation; Entire site; 11m – 130m; White chalk with soft flint seams. The Site is not in an area that could be affected by coal mining activity. The Site is not situated in an area that could be affected by non-coal mining activity. It has not been possible to find the classification status of the groundwater quality of the site (typically found under the EA’s local River Basin Management Plan) in terms of its chemical and quantitative status under the Water Framework Directive classification scheme. Any impacts to the groundwater body beneath the site are unlikely to be attributable to the site given its current and historical land uses. There are currently seven licensed groundwater abstractions within 2km of the site as detailed in the table below. Licence Holder; Distance from Site; Abstraction Source; Purpose of Abstraction. Calcot Park Golf Club Ltd; 1.21km SW; Not specified; General agriculture: Spray Irrigation-direct. Reading Town Football Club; 1.49km N; Not specified; Sports Grounds/ Facilities: Spray irrigation-Direct. Prudential Ibis Club; 1.56km N; Not specified; Sports Grounds/ Facilities: Drinking; Cooking; Sanitary; Washing (Small Garden). Mr D A Drayton; 1.62km NE; Not specified; General Farming and Domestic. Reading Borough Council; 1.73km NE; Not specified; General Farming and Domestic. Network Rail Infrastructure Ltd; 1.8km SE; Not specified; Transport: General use (Medium Loss). Mr D A Drayton; 1.91km NE; Not specified; General Farming and Domestic. The site is not situated within a groundwater Source Protection Zone (SPZ). There are no surface water features onsite. The nearest surface water feature to the site is an unnamed drainage ditch in Lousehill Copse Local Nature Reserve 199m north of the site. The Environment Agency does not currently classify this drainage ditch under the Water Framework Directive classification scheme. The nearest surface water body is the Holy Brook 1.3km south of the site and is classified by the EA as being of ‘moderate’ ecological status. In the wider surrounds the River Thames is situated 1.58km north of the site and is classified by the EA as being of ‘moderate’ ecological status. There are no licensed surface water abstractions within 2km of the site. According to the EA, the site is located in Flood Zone 1 (low probability). This zone comprises land assessed as having a less than 1 in 1000 annual probability of river or sea flooding (<0.1% in any year). The EA’s Risk of Flooding for Surface Water maps indicates that the site is at ‘very low’ risk of surface water flooding. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Radon, Ground Gas and Vapour Radon According to the UK Radon map/Landmark Envirocheck database the site is not in an area of high radon levels as less than 1% of properties are above the Action Level. Correspondingly no Protective Measures will be required in the proposed development. Ground Gas and Vapour The former tile and pottery works and their associated clay pits within a 500m radius of the site have all subsequently been re-developed into residential properties or works/ depots in the 1970s onwards. Given the length of time that these properties were re-developed it is considered that any gases from any potential infilling associated with the clay pits are likely to have off-gased by this stage. There are currently no authorised or historical landfills onsite or within a 1km radius of the site based on a review of the Landmark Envirocheck database. There are no licensed waste management facilities within 1km of the site based on a review of the Landmark Envirocheck database. A petrol filling station 185m south of the site was redeveloped as a car wash ten years ago and as such its potential to act as a potential source of organic vapours derived from any potential leakages or spillages from underground storage tanks (USTs) is considered to be negligible. Additionally, this site is topographically downgradient from the subject site so any potential vapours from contaminated groundwater are unlikely to have impacted the subject site. Furthermore, residential properties/flats with gardens have been subsequently developed adjacent to the east of the former petrol filling station and as such are likely to have been subjected to thorough contaminated land assessment as part of the relevant planning application. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Ground stability The Site is not in an area that could be affected by coal mining activity. The Site is not situated in an area that could be affected by non-coal mining activity. The BGS map does not reveal any structural, geomorphological or geochemical features on or near the site. Ground stability risks onsite are reported in the table below: Ground Stability Risk; Risk Classification. Collapsible ground; Very low. Compressible ground; Negligible. Ground dissolution; Negligible. Landslide; Very low. Running sand; Very low. Shrinking or swelling clay; Low. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Preliminary Conceptual Site Model Preliminary Conceptual Site Model is present below. Receptor; Potential sources; Pathways; Risk; Justification/ Mitigation; Residual Risk. Human Health. Future site users; Potential contamination in shallow soils and groundwater Contaminants from offsite historical sources such as petrol filling station, brick and tile works, pottery works, clay pits, gravel pits, railway lines, hospital, depots and miniature rifle ranges.; Direct contact, dermal contact, ingestion and inhalation; Low; The site comprises of two unused former garages and concrete/ asphalt hardstanding. No potential sources of ground contamination have been identified associated with the current or historic uses of the site (derelict car parking garages formerly used by adjacent residential properties). The offsite historical sources of contamination have subsequently been re-developed predominantly into residential end use or depots/ works from the 1970s onwards and any contamination is likely to have been dealt with at the time of their re-development. Additionally, the site is topographically at a higher elevation from the subject site and are unlikely to have impacted the subject site. Furthermore, the proposed end development is due to include extensive areas of hardstanding and building cover acting as a barrier to any potential contamination. The above recommendation should not unduly affect a reasonable level of selected landscaping appropriate to the site.; Lo Low. Construction workers; Potential contamination in the shallow soils and groundwater. Contaminants from offsite historical sources such as petrol filling station, brick and tile works, pottery works, clay pits, gravel pits, railway lines, hospital, depots and miniature rifle ranges.; Inhalation, migration through granular deposits and accumulation in excavation spaces and voids.; Low; Construction workers could come into contact with potential contamination of natural soils and groundwater during demolition/ construction works. Should any unforeseen contamination be identified during redevelopment all works will cease and the Planning Department of Reading Borough Council will be contacted, and a remediation scheme shall be prepared and submitted to the Council and agreed with the LPA. Subsequently a validation report which demonstrates the effectiveness of the remediation carried out will be submitted to Reading Borough Council Planning Department prior to the construction. Construction workers should utilise appropriate personal Protective Equipment (PPE) and Respiratory Protective Equipment (RPE) when handling soils and groundwater. If required dampening down of materials should take place during excavation works. Construction workers will adhere to good practice hygiene and safety measures and the Control of Asbestos Regulations 2012.; Low. Property. On-site structures and services; Potential contamination in shallow soils and groundwater. Contaminants from offsite historical sources such as petrol filling station, brick and tile works, pottery works, clay pits, gravel pits, railway lines, hospital, depots and miniature rifle ranges.; Direct contact. Chemical attack to buried foundations.; Low; The site comprises of two unused former garages and concrete/ asphalt hardstanding. No potential sources of ground contamination have been identified associated with the current or historic uses of the site (derelict car parking garages formerly used by adjacent residential properties). All new services will be installed to nominated suppliers’ requirements and specification.; Low. Adjacent residential properties; Potential contamination in the shallow soils and groundwater.; Leaching and lateral migration offsite through shallow granular deposits and groundwater.; Low; The site comprises of two unused former garages and concrete/ asphalt hardstanding. No potential sources of ground contamination have been identified associated with the current or historic uses of the site (derelict car parking garages formerly used by adjacent residential properties). However, in consideration of the current and historical land uses in the surrounding area, impacts to existing or proposed properties offsite are unlikely to be solely attributable to the site.; Low. Off-site structures; Potential contamination in the shallow soils and groundwater. Contaminants from offsite historical sources such as petrol filling station, brick and tile works, pottery works, clay pits, gravel pits, railway lines, hospital, depots and miniature rifle ranges.; Lateral migration through shallow granular deposits. Ingress into offsite structures.; Low; In consideration of the current and historical land uses in the surrounding area, impacts to existing or proposed properties offsite are unlikely to be solely attributable to the site.; Low. Crops; Potential Contamination in shallow soils and groundwater.; Leaching and lateral migration offsite through shallow granular deposits and groundwater.; Low; There are no arable farms within 1km of the subject site.; Low. Livestock; Potential Contamination in shallow soils and groundwater.; Leaching and lateral migration offsite through shallow granular deposits and groundwater.; Low; There are no pastoral farms within 1km of the subject site.; Low. Pets; Potential Contamination in, shallow soils and groundwater.; Leaching and lateral migration offsite through shallow granular deposits and groundwater.; Low; In consideration of the current and historical land uses in the surrounding area, impacts to existing or proposed properties offsite are unlikely to be solely attributable to the site. Given that the proposed re-development consists of a residential dwelling and is similar to the current surrounding land use any impacts to pets are unlikely to be attributable to the site.; Low. Ecological systems. On-site vegetation; Potential Contamination in the shallow soils and groundwater. Contaminants from offsite historical sources such as petrol filling station, brick and tile works, pottery works, clay pits, gravel pits, railway lines, hospital, depots and miniature rifle ranges.; Direct contact; Low; The proposed end development is due to include extensive areas of hardstanding and building cover acting as a barrier to any potential contamination. The above recommendation should not unduly affect a reasonable level of selected landscaping appropriate to the site. The offsite historical sources of contamination have subsequently been re-developed predominantly into residential end use with gardens from the 1970s onwards. Additionally, the site is topographically at a higher elevation from the subject site and are unlikely to have impacted the subject site.; Low. Proposed soft landscaping; Potential contamination in the shallow soils and groundwater.; Plant uptake in Made Ground and shallow soils; Low; The proposed end development is due to include extensive areas of hardstanding and building cover acting as a barrier to any potential contamination. The above recommendation should not unduly affect a reasonable level of selected landscaping appropriate to the site.; Low. Lousehill Copse Local Nature Reserve (LNR) ((30m southeast- also extends to 500m north/north-west of the site); Potential Contamination in the shallow soils and groundwater.; Leaching and lateral migration offsite through shallow granular deposits and groundwater.; Low; Permeable areas of the Winter Hill Gravel Formation (Secondary A Aquifer) have the potential to allow lateral migration of contamination offsite. However, considering that this site was formerly occupied by clay pits and associated BGS recorded mineral site (Grovelands Brick Tile and Pottery works) which mined sand and shale via opencast measures, a tank and a miniature rifle range, any impact to the Lousehill Copse is unlikely to be attributable to the subject site. Furthermore, no potential sources of ground contamination have been identified associated with the current or historic uses of the site (derelict car parking garages).; Low. Blundells Copse Local Nature Reserve (906m northwest); Potential Contamination in shallow soils and groundwater.; Leaching and lateral migration offsite through shallow granular deposits and groundwater.; Low; Permeable areas of the Winter Hill Gravel Formation (Secondary A Aquifer) have the potential to allow lateral migration of contamination offsite. However, given the distance from the site, impacts to Blundells Copse are considered unlikely.; Low. Round Copse Local Nature Reserve (962m northwest); Potential Contamination in the Made Ground, shallow soils and groundwater.; Leaching and lateral migration offsite through shallow granular deposits and groundwater.; Low; Permeable areas of the Winter Hill Gravel Formation (Secondary A Aquifer) have the potential to allow lateral migration of contamination offsite. However, considering that this site was formerly occupied by clay pits any impact to Round Copse is unlikely to be attributable to the subject site. In addition, given the distance from the site, impacts to Round Copse are considered unlikely.; Low. Controlled waters. Shallow groundwater: Winter Hill Gravel (Secondary A Aquifer); Potential Contamination in shallow soils and groundwater.; Direct contact with contaminated soils. Leaching with contaminated soils; Low; The site is not situated in a groundwater Source Protection Zone (SPZ). The site comprises of two unused former garages and concrete/ asphalt hardstanding. No potential sources of ground contamination have been identified associated with the current or historic uses of the site (derelict car parking garages formerly used by adjacent residential properties). Furthermore, the proposed end development is due to include extensive areas of hardstanding and building cover acting as a barrier to any potential contamination. It has not been possible to establish the classification status of the groundwater quality of the site (typically found under the EA’s local River Basin Management Plan) in terms of its chemical and quantitative status under the Water Framework Directive classification scheme however, any impacts to the groundwater body beneath the site are unlikely to be attributable to the site given its current and historical land uses.; Low. Deep groundwater: Secondary A Aquifer (Lambeth Group) and Principal Aquifer (Upper Chalk); Potential contamination in the shallow soils and groundwater.; Leaching through granular deposits; Low; The site comprises of two unused former garages and concrete/ asphalt hardstanding. No potential sources of ground contamination have been identified associated with the current or historic uses of the site (derelict car parking garages formerly used by adjacent residential properties). A significant thickness of low permeability London Clay is likely to be present underlying the Site and in the surrounding area and will act as an aquiclude, limiting the vertical migration of contaminants to the underlying Secondary A and Principal Aquifers.; Low. Unnamed drainage ditch in Lousehill Copse Local Nature Reserve 199m north of the site.; Potential contaminants in the shallow soils and groundwater. Contaminants from offsite historical sources such as brick and tile works, pottery works, clay pits, and miniature rifle ranges.; Leaching and migration offsite through granular shallow deposits and groundwater.; Low; The site comprises of two unused former garages and concrete/ asphalt hardstanding. No potential sources of ground contamination have been identified associated with the current or historic uses of the site (derelict car parking garages formerly used by adjacent residential properties). Permeable areas of the Winter Hill Gravel Formation (Secondary A Aquifer) have the potential to allow lateral migration of contamination offsite. However, considering that this site was formerly occupied by clay pits and associated BGS recorded mineral site (Grovelands Brick Tile and Pottery works) which mined sand and shale via opencast measures, a tank and a miniature rifle range, any impact to the Lousehill Copse is unlikely to be attributable to the subject site. Furthermore, the Environment Agency does not currently classify this drainage ditch under the Water Framework Directive classification scheme and this is therefore not considered to be a sensitive receptor.; Low. Holy Brook (1.3km south); Potential contaminants in the shallow soils and groundwater.; Leaching and migration offsite through shallow granular deposits and groundwater. Migration into drains flowing into Holy Brook; Low; The site comprises of two unused former garages and concrete/ asphalt hardstanding. No potential sources of ground contamination have been identified associated with the current or historic uses of the site (derelict car parking garages formerly used by adjacent residential properties). However, in consideration of current and historical land uses in the surrounding area, and its distance from the Site, the risk of contaminants migrating to the River Thames from the Site are considered low.; Low. River Thames (1.58km north); Potential contaminants in the shallow soils and groundwater.; Leaching and migration offsite through shallow granular deposits and groundwater. Migration into drains flowing into the River Thames.; Low; The site comprises of two unused former garages and concrete/ asphalt hardstanding. No potential sources of ground contamination have been identified associated with the current or historic uses of the site (derelict car parking garages formerly used by adjacent residential properties). However, in consideration of current and historical land uses in the surrounding area, and its distance from the Site, the risk of contaminants migrating to the River Thames from the Site are considered low.; Low. Archaeological sites and ancient monuments. There are no archaeological or ancient/ scheduled Monuments recorded onsite. Nor are there any archaeological or ancient/scheduled monuments recorded within 2km of the site. The site is not situated within a Conservation Area. The nearest Conservation Areas are situated 875m west of the site (Routh Lane) and Horncastle 890m southwest of the site.; N/A; N/A; N/A; N/A; N/A. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |