SAP Summary Overview Report

Project Ref: 2no. new dwellings at 1a Eaton Place

SAP Type: New Build Part L 2021

This document must be submitted with your SAP calculations for building control approval

| Building Fabric |  |  |
| --- | --- | --- |
| Element | Construction build up including insulation & thickness | U value (W/m2K) |
| Ground floor | 75mm screed, 150mm Kingspan (0.019), 100mm concrete | 0.11 |
| Exposed floor | 50mm screed, 200mm concrete, 130mm Rockwool Soffit slab | 0.25 |
| External wall | 100mm brick, 10mm cavity, 90mm Kingspan (0.019), 100mm lightweight 7n block (0.018), plasterboard on dabs | 0.17 |
| Ceiling | 300mm mineral wool above joists, 100mm mineral wool between joists, plasterboard | 0.11 |
| Windows | Double Glazed (unit U-value including frame and glazing) G-value 0.66 | 1.40 |
| Solid Front Door | (unit U-value including frame and glazing) | 1.20 |
| Air test | Design target 4.00 | 4.00 m3/hm2 |
| Thermal bridging | Recognised Construction Details. Copy of relevant checklists can be found here | Y-Value |
|  | PLEASE NOTE: For calculated / bespoke psi value details to be used in the SAP Calculations (L1A ONLY) the thermal Junction checklist guidance MUST be implemented on site. Failure to adhere to these standardized junction details will require the use of default PSI values for the junction heat loss which will adversely affect the performance of the dwelling(s) against Part L. If you cannot meet the requirements of the ACDs or the construction changes, please inform us at the earliest stage so compliance can be achieved before the project progresses. |  |
| HVAC |  |  |
| Type | Specification |  |
| Primary heating | Electric panel heaters | % Efficiency |
| Secondary heating | N/A |  |
| Emitter | Panel heaters |  |
| Primary heating Controls | Programmer & appliance thermostats |  |

| DHW | Dimplex EDL170 hot water heat pump https://www.dimplex.co.uk/products/heat-pumps/hot-water-heat-pumps?gad_source=1&gclid=EAIaIQobChMIku7LjPm4jAMVUZhQBh1u2whwEAAYASAAEgJ1nPD_BwE |  | kWh/day |
| --- | --- | --- | --- |
| Ventilation | Intermittent extract fans to all wet rooms |  | 2 |
| Lighting | 100% low energy light fittings throughout including external lights. Minimum 75 lumens per circuit watt |  | 100% |
| Solar Hot Water | N/A |  |  |
| Photo Voltaic panel | N/A |  |  |
| DISCLAIMER: Please note that the design inputs outlined above are key to the project's compliance with Part L1A/L1B of building regulations. Should any of the details above not accurately represent the proposals of the project or if any of the measures above cannot be incorporated for any reason, please let us know, as the project may fail the regulations. If changes are made to the above specification without contacting us, and the project fails there will be a consultation fee charged to help ensure the building passes the relevant regulations |  |  |  |
| Client/contractor Signature: |  | Date: |  |

| Plot | EPC Rating |
| --- | --- |
| GF | 80 C |
| FF | 73 C |