29/09/2025

Planning Application Ref: 			PL/25/1204
Officer: 					Anthony Scholes
Application Type: 				Prior approval Part 3 Class MA: Commercial,

business and service uses to dwellinghouses

Address: 					20 TUDOR ROAD, READING, RG1 1NH
Proposal:					Change of use from Class E (offices) to C3

(dwelling houses) to comprise 58 flats (28x 1b1p 26x 1b2p 4x 2b3p) flats with associated car and cycle parking. Prior Notification under Class MA, Part 3 of Schedule 2 of the Town and Country Planning (General Permitted Development) Order 2015.

Previous application for this site - PL/25/0929 - REFUSED

The site at the junction of Tudor Road and Caversham Road and is within Zone 2, the primary core area but on the periphery of the central core area which lies at the heart of Reading Borough, consisting primarily of retail and commercial office developments with good transport hubs.  Extensive parking restrictions are in place around the site. This application is for change of use from Class E to 58 residential units comprising of 58 x 2 bedroom Units.

In accordance with the adopted Parking SPD, the development would require the following parking provision.  Given the site is on the border of Zone 1 and close to the town centre Reading main station and access to all main bus routes, Zone 1 parking standards can be applied in this instance.

| Size of Units | No of units | Required Provision Per Unit Zone 1 | Zone 1 Total |
| --- | --- | --- | --- |
| 1 & 2 bed units | 58 | 0.5 | 29 |
| Total required |  |  | 29 |

It should be noted that the submitted Planning Statement relates to the previous application which was for 70 residential units and not the 58 proposed under this application.

Submitted proposed ground floor plan drawing 32508 /P012   illustrate a total 37 parking spaces located at ground level within an existing car park area This is inn excess of Zone 1 requirements.

For developments up to 200 spaces, 3 disabled spaces or 5% of total capacity, whichever is greater should be provided.  This application seeks to provide 2 disabled spaces which is deemed acceptable.   However, it is noted that a door open into electric charging bay No 22 as shown below.  A vehicle parked in his bat would obstruct access to the bay and vice versa

No changes to the existing access arrangements are proposed.

The Councils Residential Permit Parking Scheme operates in this area, therefore under the Borough’s current parking standards, this proposal would generate additional pressure for parking in the area, if this application is approved there should be an assumption that any future occupants of the proposed dwellings will not be issued with resident or visitor parking permits, this will ensure that the development does not harm the existing amenities of the neighbouring residential properties by adding to the already high level of on street car parking in the area.

In line with the Council’s adopted Parking Standards and Design SPD, each of the 1 and 2 bedroom unit should be provided with 0.5 secure cycle parking, however under GPD we would not comment on this.

Bin storage should comply with Manual for Streets and British Standard 5906: 2005 for Waste Management in Buildings and should be located no further than 10m from the access point of the site to avoid the stationing of service vehicles on the carriageway for excessive periods.  It is recommended that the Councils Waste Management team be contacted to ensure the correct capacity of bins is provided.

There are no Transport objections to this permitted development application.

IF3	Highways

I13	Parking Permits

N. Hussain

Transport Development Control Advisor