I have reviewed the contaminated land assessment and have the following comment:

•	The risk from any land contamination has been dismissed, however, the report should fully acknowledge that there is a potentially contaminative use on site, the former fever hospital, and it does not appear to have taken this into account.

I have reviewed the noise assessment and have the following comment:

•	The noise from the proposed commercial use which will co exist with the residents on site needs to be further considered. What plant will be installed, in what locations? If the plant is not yet known due to the commercial uses, then some consideration needs to be made of what likely plant will be installed to ensure that these will be designed appropriately to minimise impact on residents, e.g. is a plant room required? Will windows require an upgrade if a kitchen extraction is likely to be installed on the facade, or is internal ventilation provided to the commercial units such that an external flue won't be required? A noise limit has been proposed in the assessment, of 5 dB below background but this does not meet policy EN17 which states it should be 10 dB below.

Environmental Protection